3GPP TSG-RAN WG2 Meeting #129	R2-2501291

Athens, Greece, Feb 17th – 21st, 2025		Revision of R2-2410806

Agenda item:	8.8.2

Source: 	Sequans Communications

Title: 	Downlink coverage enhancement access control

Document for:	Discussion and Decision

# Introduction

During the previous meetings, the following was agreed:

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Agreements:

1.	[…]

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

In this contribution, we discuss further this topic.

# Discussion

## Barring

Possible barring was discussed in the context of SSB periodicity extension. Currently the SSB periodicity is included in SIB1 (ssb-PeriodicityServingCell in ServingCellConfigCommonSIB) and may be configured up to 160ms. A UE only assumes 20ms periodicity for initial cell selection (cell search).

For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

Hence, up to 160ms, it seems not required to bar a cell in SIB1 for that reason, since it is also in SIB1 that the UE knows the real SSB periodicity – and can then operate perfectly.

Observation 1: Up to 160 ms periodicity, barring doesn’t seem required

If the SSB periodicity is extended to higher value than 160ms, it seems more likely that barring is needed. However, it was decided in RP#106 to limit the maximum periodicity to 160ms. We believe additional input is needed to understand whether barring support is required.

Proposal 1: RAN2 waits for RAN1 before deciding whether barring is needed

Moreover, above RAN2 agreements do not define what “DL-CE” means, which makes progress difficult.

## Down-prioritizing / preventing re-selection

This was put in the agenda item based on observations from ‎[1]:

“Let’s consider a serving cell without DL CE, where both UE capable/incapable of DC CE could exit in the serving cell, and 6 neighboring cells, where cell 1/5 are cells with DL CE while the rest of cells are not operating with DL CE. In this case, there is no need for a UE incapable of DL CE (including legacy UEs) to perform neighboring cell measurement or re-select to cell 1/5 since the UE may not be able to camp or normally operate in such cells. However, cell 1/5 can still be considered for UEs capable of DL CE during cell reselection.”

### Preventing re-selection with barring

If a UE is not able to camp or normally operate in such DL CE cells, the cell would be barred. A UE already cannot reselect on a barred cell, as it will not be considered as a candidate for cell reselection.

Observation 2: Preventing re-selection may already be realized by legacy way (cell barring)

However, it is correct that this might come with additional power consumption from UE side, which we believe should be addressed.

### Preventing legacy UEs to search/reselect DL CE cells

The considered deployment scenario consists in a mix of normal cells/DL CE cells (with extended periodicity) on same or different frequency layers. It is desirable to have the flexibility to prevent legacy UEs to search/reselect DL CE cells.

Normal cells/DL CE cells on same frequency layer

With legacy SMTC signalling (smtc1/smtc4), the NW can configure up to 4 SMTCs with different offsets, however they will share the same periodicity. To avoid any impact on legacy UEs, the deployment should allow e.g. 20ms periodicity for normal cell SMTC, and 160ms for DL CE cell SMTC.

Observation 3: Legacy SMTC signaling (smtc/smtc4) only allows a single periodicity

As detailed in our companion contribution, we believe DL CE (due to beam hopping) will require signalling more than one SMTC configuration. E.g., we could be in following scenario where blue cells does not support DL CE, and share the same SMTC; while other neighbour cells support DL CE, and have different SMTC configuration (on a given frequency layer, as SMTCs are per frequency layer).

In general, the issue will be that the UE spends time and power searching DL CE cells on those additional SMTC configurations. If ones want to avoid legacy UEs to search/reselect DL CE cells, it is crucial that the NW can avoid signalling DL CE SMTC configurations within legacy SMTC signalling.

Observation 4: Efficient support of normal/DL cells on same frequency require separate SMTC signaling

Another issue is related to handling of RRMs. E.g., currently the cell reselection requirements are conditional to the number of configured SMTCs. Configuring more SMTCs than supported by legacy UEs would result in making the existing RRM requirements no longer applicable (see Annex). This is more a RAN4 topic, but we believe it would also make sense to not mix up those DL CE cells with legacy cells.

Hence, an alternative could be to consider signalling the DL CE cells SMTC configurations in a new list (e.g. SMTC5). That way, the NW (if it wishes so) can isolate those DL CE cells so that only UEs “supporting DL CE” would try to search for/reselect those cells.

Proposal 2: Introduce separate optional signaling for DL CE cells SMTCs (e.g. SMTC5 list)

The NW could either ensure complete non-overlap of SSBs from legacy/DL CE cells, or only partial, as it wishes. In case of partial overlap, if a legacy UE happens to decode a DL CE SSB (by chance), this is not an issue, as it can be handled the legacy way (i.e. barring the DL CE cell, if required).

For this purpose, smtc5list could be added in intraFreqCellReselectionInfo (SIB2), and InterFreqCarrierFreqInfo (SIB4).

Proposal 3: smtc5list is added to intraFreqCellReselectionInfo (SIB2), and InterFreqCarrierFreqInfo (SIB4)

In our understanding, the NW should still have the flexibility to accommodate legacy UEs on DL CE cells, if it wishes so. Especially, RAN plenary decision to limit periodicity to 160ms was driven by backward compatibility concerns, so the signaling should still allow to accommodate legacy UEs.

Proposal 4: Legacy SMTC signaling can still be used for DL CE cells (up to NW configuration)

Normal cells/DL CE cells on different frequency layers

Similarly, the NW should be able to isolate DL CE only frequencies, so that only UEs “supporting DL CE” would try to search for/reselect those frequencies. Even though legacy SMTC field is optional, above SMTC signalling framework cannot be used since if legacy smtc is absent, 5ms default periodicity is assumed.

Hence, the NW should be able to indicate separately the DL CE only inter-frequencies.

Proposal 5: Introduce separate signaling for DL CE only inter-frequencies

### Down-prioritization of DL CE cells

Normal cells/DL CE cells on different frequency layers

In case of separate frequency layer, the existing prioritization scheme could be reused.

Normal cells/DL CE cells on same frequency layer

If needed, this could be implemented based e.g. on prioritizing SMTC with smaller periodicity, at least to the extent where it is currently up to UE implementation to select SMTCs.

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Up to 160 ms periodicity, barring doesn’t seem required

Proposal 1: RAN2 waits for RAN1 before deciding whether barring is needed

Observation 2: Preventing re-selection may already be realized by legacy way (cell barring)

Observation 3: Legacy SMTC signaling (smtc/smtc4) only allows a single periodicity

Observation 4: Efficient support of normal/DL cells on same frequency require separate SMTC signaling

Proposal 2: Introduce separate optional signaling for DL CE cells SMTCs (e.g. SMTC5 list)

Proposal 3: smtc5list is added to intraFreqCellReselectionInfo (SIB2), and InterFreqCarrierFreqInfo (SIB4)

Proposal 4: Legacy SMTC signaling can still be used for DL CE cells (up to NW configuration)

Proposal 5: Introduce separate signaling for DL CE only inter-frequencies

# References

- R2-2408655, “Consideration on downlink coverage enhancements”,  ZTE Corporation, Sanechips

# Annex – 38.133

#### 4.2C.2.3 Measurements of intra-frequency NR cells […] The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

#### 4.2C.2.4 Measurements of inter-frequency NR cells [….] The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the [UE capability], otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in Table 4.2C.2.4-1 and Table 4.2C.2.4-2 is expected.