3GPP TSG-RAN WG2 Meeting #129	R2-2501292

Athens, Greece, Feb 17th – 21st, 2025		Revision of R2-2410878

Agenda item:	7.4.2

Source: 	Sequans Communications

Title: 	Remaining open issues of satellite switch with resync

Document for:	Discussion and Decision

# Introduction

During the last meeting, the following was discussed and agreed:

R2-2410971	Report of [AT128][301][NR NTN] SMTC issues		Huawei	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 to further discuss whether to specify the following for soft satellite switching:

Option 2b: After t-Service, UE applies ssb-TimeOffet to the SMTC configured by source satellite.

Option 3: Temporarily shifted SSB pattern (determined by ssb-TimeOffset) is used during [t-Start, t-Service] for early synchronization with target satellite, FFS whether RAN4 input is needed.

-	vivo is fine to consider option 3 and send a LS to RAN4/RAN1 asking for feedback

-	QC thinks it’s late to introduce a change and prefers not to send the LS

-	Google/Sequans are fine to send the LS. Sequans thinks option 2b is not complete

Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

In this contribution, we discuss further this topic.

# Discussion

## Soft satellite switch with resync – SMTC issues

In previous meeting, RAN2 discussed again SMTC issues in case of soft satellite switch with resync (initially brought up in ‎[1] and ‎[2]). The issue is related to the fact that current implementation of “Soft satellite switch with resync” eventually ends up in changing the SSB timing before/after the switch. That can be realized in different ways by the NW (changing the cell timing, changing the SSB location only, or a combination of both). But in the end, all those ways will have an impact on SMTCs, e.g. requiring reconfigurations at the time of the switch. This is expected as SMTC configuration is aligned on SSB occurrences.

RAN2 sent out an LS to RAN1/RAN4 on the feasibility of option 3, and agreed to work on solution 2b or solution 3 depending on the LS response.

In the following, we detail those options, comparing them to hard switch:

Figure 1 – Hard-switch (SSB half radio frames at gNB)

### Solution 3

The “Solution 3” has following characteristics:

- Cell timing before/after t-Service is not changed (same as for hard switch)

- SSB pattern (for SSB sent via target satellite) is shifted by SSB-timeOffset, but only during [t-Start, t-Service]

Figure 2 – Solution 3

Following analysis in ‎[4], RAN2 previously captured “RAN2 understands that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)”. This might enable to skip MIB reading. It would be useful to capture this understanding too if solution 3 is agreed.

Proposal 1: If solution 3 is adopted, capture that cell timing is unchanged (at gNB)

For soft switch, the only requirement for the UE is to start acquiring DL synchronization at t-serviceStart. Hence, it is expected that only SSBs are sent by target satellite during [t-Start, t-Service]. In such case, it could be beneficial to specify that those SSB do not indicate CORESET0/other SSB (similar to NSA only cell).

Proposal 2: If solution 3 is adopted, discuss whether SSBs during [t-Start, t-Service] should be “NSA-only” SSB

### Solution 2b

The “Solution 2b” has following characteristics:

- Cell timing before/after t-Service is not changed (same as for hard switch)

- SSB pattern (for SSB sent via target satellite) is shifted by SSB-timeOffset

- After t-Service, UE applies ssb-TimeOffet to the SMTC configured by source satellite

Figure 3 - Solution 2b

Applying ssb-TimeOffet to the SMTC configured by source satellite is an attempt to avoid the need for SMTC reconfiguration, and keep the benefits of satellite switching feature in that case.

In the following we check if/to which extent this would avoid SMTC reconfiguration. In ‎[1], we did consider this option, highlighting that the UE would need to apply ssb-TimeOffet to the SMTCs corresponding to serving cell/neighbor cells that would perform the same change in a synchronized way.

Mobility to other unchanged PCI cells (from same constellation)

This would be the basic scenario to support.

In case of frequency reuse 1 deployment, the UE may be configured just with 1 intra-freq SMTC. In case of frequency reuse > 1 deployment, the UE may also be configured an inter-freq SMTC.

Earlier (simplified) analysis in ‎[1] assumed that neighbors from same “satellite orbit” would all perform the switch at the same time, however as discussed in ‎[3] we think that would not be the case. This is because cell footprints would be typically smaller than the whole satellite footprint, and as the satellite footprint moves, the cells would gradually switch from source satellite (in green below) to target satellite (in blue below):

Figure 4 – Gradual switching of neighbor cells

As an example, if we consider the LEO 600 / Elevation >= 30° scenario, the whole satellite footprint is around 1600km in diameter. Assuming 200km cell diameter, each gradual step above would last around 25s, for a total serving time of 211s. Hence, with this example, the SMTC would still be off for around 1/3 of neighbor cells during 1/4 of the time.

Observation 1: Neighbor cells from same/previous/next satellite would switch around serving cell switching time, but not at the same time

Due to this, it is challenging to support correct configuration of SMTCs/gaps with current soft switch specification. It would require up to 2 successive RRC reconfigurations to set correctly the SMTCs/gaps during a cell switch (one for the serving, one for the target), and it would also require several SMTCs activated simultaneously where only one was needed with hard switch.

From this aspect, the solution 2b is a compromise using no RRC reconfiguration, but with only limited accuracy of configured SMTCs/gaps.

Regarding neighbor cells from the same constellation but from a different satellite orbit, it seems difficult to ensure that they switch around the same time (and with the same SSB-offset).

Observation 2: Neighbor cells from other satellite may have unrelated switching time

Mobility to TN cells (hand-in) / GSO cells / NGSO cells from different constellations

The corresponding SMTCs/gaps should not be updated, as those target cells would not switch.

- TN cells: they can be identified from the frequency band

- GSO: they can be identified from the ephemeris

- NGSO cells from different constellation: it seems difficult to identify, however this scenario may be ok to not support

In summary, there is no perfect solution to avoid reconfiguration of SMTCs/gaps. Applying SSB-offset in certain case can still provide benefits.

Proposal 3: If solution 2b is adopted,  SMTCs/gaps related to TN/GSO should not be updated, while other configured SMTCs/gaps should apply SSB-offset

### Way forward

Solution 2b only very partially address the issues related to SMTC in soft satellite switching.

In this table we consider an unchanged PCI cell S, with neighbour cells N.

- The cell S has intra-frequency and inter-frequency SMTCs (SIBs or configured to UE in connected) used to perform SSB-based measurements for out-bound mobility towards cells N

- The cells N have intra-frequency and inter-frequency SMTCs (SIBs or configured to UE in connected) used to perform SSB-based measurements for in-bound mobility towards cells S

The table captures what are the impacts at the time of cell S switching

| SMTC (same apply for corresponding gap in connected) | Soft switch (solution 2b) | Hard switch / Soft switch (solution 3) |
| --- | --- | --- |
| Cell S intra-frequency SMTC (SIB2) (for reselection S->N) | Need to update SI change notification | No impact |
| Cell S inter-frequency SMTC (SIB4) (for reselection S->N) | If N switches simultaneously, need to update SI change notification | No impact |
| Cell N SMTC used to measure Cell S (in SIB2, SIB4, or LTE SIB) (for reselection N->S) | Need to update SI change notification | No impact |
| Cell S intra-frequency SMTC (intra-freq MO) (for handover S->N) | Updated (Autonomous) (but SMTC might still be off) | No impact |
| Cell S inter-frequency SMTC (inter-freq MO) (for handover S->N) N=NGSO case | Updated (Autonomous) (but SMTC might still be off) | No impact |
| Cell S inter-frequency SMTC (inter-freq MO) (for handover S->N) N=GSO/TN case | No impact | No impact |
| Cell N configured SMTC used to measure Cell S (for handover N->S) N=NGSO case | Updated (Autonomous) (but SMTC might still be off) | No impact |
| Cell N configured SMTC used to measure Cell S (for handover N->S) N=GSO/TN case | Need to update RRC reconfiguration | No impact |

Table 1 – SMTC impacts at the time cell S switches

The main drawbacks of solution 2b are as follows:

- Satellite switch with resync generally avoids SI change notification, which is crucial for power consumption of IDLE UEs. This is no longer avoided with solution 2b. When a cell S switches, SIBs must be updated in S, and also N cells (S neighbors providing SMTCs towards S)

- On top of that, due to “gradual switching” of neighbor cells, the updated SMTCs in cell S/N would not be always correct (that would require setting up several SMTCs)

- Reconfiguration of SMTCs/gaps would also be required for UEs measuring cell S as a target for handover

- The autonomous application of SSB-offset does not work well considering “gradual switching” of neighbor cells

Observation 3: Compared to hard switch/solution 3, solution 2b requires SMTC reconfiguration in SIBs, which partially works, MO (for inbound HO), moreover SSB-offset applied to SMTC only partially works

Given the drawbacks of solution 2b compared to hard switch, it seems unlikely to be deployed. Hence, in case of non-conclusive answer from RAN1/RAN4 (no consensus), we propose to adopt solution 3, which keeps all the benefits of hard switch.

Proposal 4: In case RAN1/RAN4 doesn’t have a conclusive answer, adopt solution 3

# Conclusion

In this contribution, we make the following observations and proposals:

Proposal 1: If solution 3 is adopted, capture that cell timing is unchanged (at gNB)

Proposal 2: If solution 3 is adopted, discuss whether SSBs during [t-Start, t-Service] should be “NSA-only” SSB

Observation 1: Neighbor cells from same/previous/next satellite would switch around serving cell switching time, but not at the same time

Observation 2: Neighbor cells from other satellite may have unrelated switching time

Proposal 3: If solution 2b is adopted,  SMTCs/gaps related to TN/GSO should not be updated, while other configured SMTCs/gaps should apply SSB-offset

Observation 3: Compared to hard switch/solution 3, solution 2b requires SMTC reconfiguration in SIBs, which partially works, MO (for inbound HO), moreover SSB-offset applied to SMTC only partially works

Proposal 4: In case RAN1/RAN4 doesn’t have a conclusive answer, adopt solution 3

# References

- R2-2405680, “Soft satellite switch SSB time offset and SMTC impact”,	Sequans Communications

- R2-2405757, “Summary of [AT116][301][NR NTN Enh] SMTC impacts for soft satellite switch”, Sequans Communications

- R2-2410971, “Report of [AT128][301][NR NTN] SMTC issues”, Huawei

- R2-2405672, “[H010][H115] and skipping MIB acquisition”, Sequans Communications