3GPP TSG-RAN2 Meeting #129	R2-2501293

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			ZTE Corporation

Title: 	Discussion on measurement event prediction

Agenda item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

In the RAN2 127bis meeting, the following agreements about measurement event prediction and system level simulation were achieved [1]:

- Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. - Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. - Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. - Measurement event prediction results are expected in RAN2#129 - The System level performance (e.g. HO performance) evaluation is optional (i.e. companies can bring results if they chose). - System level performance for measurement event prediction can be prioritized by companies if they chose to do it.

Based on the previous agreements, the simulation results for measurement event prediction are expected in this meeting. So, in this contribution, we provide the simulation results for measurement event prediction and system level simulation (SLS), and give the corresponding analysis.

- Discussion

## 2.1 Evaluation results for measurement event prediction

In this contribution, we study the indirect measurement event prediction simulation with intra-freq temporal domain case A in FR2 scenario, which is high priority based on the previous agreements. Specifically, future measurement results of serving cell and neighbor cells are predicted by temporal domain prediction case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model, as shown in Figure 1:

Figure 1: Indirect measurement event prediction model

The table 1 below show some important simulation assumptions in our simulation. The details please see our simulation results table in the FTP.

Table 1: Simulation assumptions for measurement event prediction and SLS

| Parameters | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 90 |
| OW length (ms) | 800 |
| PW length (ms) | 400 |
| Max ETD (ms) | 80 |

In our simulation, TimeToTrigger (TTT) is set to 320ms. Depending on whether and how to consider actual measurement results in the TTT, the following three cases are considered:

Case 1: using all predicted L3 RSRP in the TTT for A3 event prediction

Case 2: using 1 actual L3 RSRP + predicted L3 RSRP in the TTT for A3 event prediction

Case 3: using 2 actual L3 RSRP + predicted L3 RSRP in the TTT for A3 event prediction

With Case 1, only predicted results are used to derive measurement event. For example, at time instance t0, the predicted results at future time instance t1 do not fulfill the entering condition for the event, then at time instance t1, the actual measurement result at time instance t1 can be used to predicted future results but are not used to derive measurement event. With Case 2, if one actual measurement result fulfills the entering condition for the event, the measurement event prediction is performed (i.e. to derive whether future predicted results fulfill the leaving condition or not). With Case 3, if two actual measurement results fulfill the entry condition for the event, the measurement event prediction is performed.

The corresponding simulation results are shown in the Figure 2:

Figure 2: the simulation results for measurement event prediction

Based on the simulation results, we find that for precision, recall and F1 score, the Case 3 is better than Case 2, and the Case 2 is better than Case 1. Specifically, compared with case 1, the F1 score of Case 2 and Case 3 increase 16% and 41% respectively; the recall of Case 2 and Case 3 increase 20% and 40% respectively, the precision of Case 2 and Case 3 increase 19% and 41%. In my understanding, this is because with the involvement of actual measurement results, the prediction performance can be improved.

Observation 1: In the measurement event prediction, using actual measurement results in the TTT can improve the F1 score, recall and precision.

Observation 2: In the measurement event prediction, the number of actual measurement results in the TTT increases, the F1 score, recall and precision improve.

Proposal 1: Regarding measurement event prediction, capture observation 1 and 2 into the TR.

In the previous discussions, we focus on the absolute value of time difference between predicted-event-triggered time and actual event-triggered time, do not consider which one comes first. For example, it is tolerated if time difference between predicted-event-triggered time and actual event-triggered time is +80ms or -80ms. However, from the perspective of network, the predicted-event-triggered time is earlier than the actual-event-triggered time would bring more severe problem if the handover command is not only relying on the legacy measurement event. This is because if the predicted-event-triggered time is earlier than the actual-event-triggered time, the network may send the handover command too early, and cause handover failure. But if the predicted-event-triggered time is later than the actual-event-triggered time, the handover command can rely on legacy measurement event, the handover performance would not be impacted.

We study the distribution of time difference between predicted-event-triggered time and actual event-triggered time, the simulation results are as follows:

Figure 3: the distribution of time difference between predicted-event-triggered time and actual-event-triggered time

Based on the simulation results in the Figure 3, we find that for Case 2 and Case 3, with the involvement of actual measurement results, the percentage for ‘-80ms~+80ms’ are increased, the percentage for ‘after +80ms’ and ‘predicted A3 is missed’ are decreased. And the percentage for ‘before -80ms’ is nearly the same for Case 1 and Case 2, and are decreased for Case 3.

Observation 3: In the measurement event prediction, using actual measurement results in the TTT can increase the percentage that the time difference between predicted-event-triggered time and actual-event-triggered time is within [-80ms, +80ms].

Proposal 2: Regarding the measurement event prediction, RAN2 to study the time difference distribution between the predicted-event-triggered time and the actual-event-triggered time.

## 2.2 Evaluation results for system level simulation

The simulation assumptions for SLS reuses that for measurement event prediction. And in the last meeting, two handover model are recommended, in our simulation, option 2 (i.e. the handover command is relying on legacy measurement event) is adopted. Besides, in order to study the gain that AI provides, we also study the handover performance with non-AI (i.e. sample and hold). The handover performance is evaluated via handover failure rate, the correspond results is shown as follows:

Figure 4: the handover failure rate for SLS with AI and non-AI

It is observed that the handover failure rate for SLS with non-AI is 7.56%, while the handover failure rate for SLS with AI is 6.84%, which decreases 9.5%. So, the handover performance for SLS with AI outperforms than that with non-AI. This is because with handover model option 2, the handover preparation time are saved if the prediction is accurate, which may avoid too-late-handover. If the prediction is inaccurate, the handover command is based on the legacy measurement event, in this way, the handover performance is same as the handover performance with non-AI. On the other hand, the benefit that the AI provides is limited, this is because only handover preparation time (i.e. 40ms) can be saved.

Observation 4: With handover model option 2, the handover performance for SLS with AI outperforms than that with non-AI.

Proposal 3: Regarding system level simulation, capture Observation 5 into the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Observation 1: In the measurement event prediction, using actual measurement results in the TTT can improve the F1 score, recall and precision.

Observation 2: In the measurement event prediction, the number of actual measurement results in the TTT increases, the F1 score, recall and precision improve.

Observation 3: In the measurement event prediction, using actual measurement results in the TTT can increase the percentage that the time difference between predicted-event-triggered time and actual-event-triggered time is within [-80ms, +80ms].

Observation 4: With handover model option 2, the handover performance for SLS with AI outperforms than that with non-AI.

Proposal 1: Regarding measurement event prediction, capture Observation 1 and 2 into the TR.

Proposal 2: Regarding the measurement event prediction, RAN2 to study the time difference distribution between the predicted-event-triggered time and the actual-event-triggered time.

Proposal 3: Regarding system level simulation, capture Observation 4 into the TR.

- Reference

- Report of RAN2 127bis meeting.