3GPP TSG-RAN WG2 Meeting #129	R2-2501296

Athens, Greece, Feb 17th – 21st, 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

In the past meetings, RAN2 discussed the detail of inter-CU LTM procedure, and the following was agreed.

Agreements on inter-CU LTM in RAN2-128[1]

Reply to SA3 that RAN2 think avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

Include the time plan information to indicate until when their feedback is required or until when RAN2 needs to down-select the options. We assume in RAN2#130, to complete WI.

Agreements on inter-CU LTM in RAN2-127bis[2]

1.	The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

2.	For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

3.	The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

Agreements on inter-CU LTM in RAN2-127[3]

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Agreements on LTM cell switch execution phase: in RAN2-125bis[4]

1.	Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

In this contribution, we provide our understanding for inter-CU LTM procedure, such as the one for the security update.

- Discussion

In RAN2-128[1] meeting, RAN2 concluded that avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection. Additionally, RAN2 sent the LS to ask SA3 to take the above RAN2 response into account and requests SA3 to provide the feedback before RAN2#130. However, RAN2 has the many issues that will be discussed after completing security key update procedure. Therefore, it is worthwhile to discuss the security key update procedure before receiving feedback from SA3.

In Rel-19 LTM, RAN2 agreed that the UE performs security key update upon inter-CU LTM execution, and Rel-19 LTM introduces a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update. Regarding the detail of security key update procedure, RAN2 provided a few options for indicating the NCC value to derive security key used on the UE. We think that RAN2 can proceed the discussion that how to deliver the security information by MAC CE or existing RRC signaling in consideration of the security threat.

Using the MAC CE to deliver the security information itself may have a security threat. Furthermore, in conditional LTM discussed in RAN2, the source CU does not send the cell switch command MAC CE to indicate the cell switch. Therefore, considering the conditional LTM, delivering the security information without using the MAC CE allows for flexibility.

On the other hand, sending the NCC value via an RRC message during the LTM preparation phase can indicate the NCC value to the UE in protected RRC signaling. Since the UE updates the security key using the NCC value included in conditional reconfiguration in CHO, Rel-19 LTM can perform the security key update procedure by applying the same procedure as CHO. Therefore, we prefer that the NCC value is included in the RRC signalling during LTM Preparation.

Proposal 1: The NCC value is included in existing RRC signalling during LTM Preparation.

In LTM Preparation Phase, the MN includes the NCC value for subsequent inter-CU LTM in LTM configuration included in RRCReconfiguration. Then the UE can update the security key by using the preconfigured NCC value upon inter-CU LTM. Currently, the option 1b, 2 and 3 are being considered in RAN2. For example, in option1b, the MN sends the list of NCC values to the UE over protected RRC signalling, and the MN sends the index of the NCC value used for inter-CU LTM via cell switch command MAC CE. In option 2 and 3b, the UE selects the first unused NCC value from preconfigured list(s), similar to Rel-18 SCPAC. In option 3a, the UE increases the NCC value by 1 upon every inter-CU LTM. In our understanding, the CN increases the NCC value by 1 upon every inter-CU mobility in Xn mobility. Therefore, option 3a is also useful. In these options, we prefer the option 2 and 3a, 3b, which operate the security key update without indication via the cell switch command MAC CE.

Proposal 2: Upon inter-CU LTM, the UE selects the NCC value by:

- selecting the first unused NCC values of NCC list(s) (option2, 3b).

- increasing the NCC value by 1 (option3a).

However, if the UE selects the new NCC value without the MN indication such as option2, 3a, and 3b, the UE may select the different NCC value from the one used at MN, and this may lead to LTM failure. Therefore, a mechanism for sharing the security key between the UE and the MN may be necessary for subsequent inter-CU LTM.

One such mechanism is that the UE indicates the NCC value to the MN by including in RRCReconfigurationComplete message, similar to Rel-18 SCPAC. However, in inter-CU LTM, if the NCC value (i.e. security key) is different between the UE and the MN, the UE may not be able to send the RRCReconfigurationComplete message to the MN. Here, if the UE can select the NCC value without the MN indication, it can also estimate the NCC value to be used in the next inter-CU LTM. Therefore, the UE can indicate the NCC value to be used at next the inter-CU LTM to the MN by including in RRCReconfigurationComplete message. Then, the MN can confirm whether or not the NCC value to be used at the UE in next inter-CU LTM is same as the NCC value indicated by the CN, without additional RRC signalling and no impact to the CN.

Proposal 3: The UE indicates the NCC value to be used at next the inter-CU LTM to the MN by including it in RRCReconfigurationComplete message.

- Summary and proposal

This paper discussed the procedure for mixture of inter-CU LTM and intra-CU LTM. In summary, the followings were proposed:

Proposal 1: The NCC value is included in existing RRC signalling during LTM Preparation.

Proposal 2: Upon inter-CU LTM, the UE selects the NCC value by:

- selecting the first unused NCC values of NCC list(s) (option2, 3b).

- increasing the NCC value by 1 (option3a).

Proposal 3: The UE indicates the NCC value to be used at next the inter-CU LTM to the MN by including it in RRCReconfigurationComplete message.

- References

[1] R2-2500002, “RAN2#128 Meeting Report,” ETSI MCC.

[2] R2-2409502, “RAN2#127bis Meeting Report,” ETSI MCC.

[3] R2-2407902, “RAN2#127 Meeting Report,” ETSI MCC.

[4] R2-2404102, “RAN2#125bis Meeting Report,” ETSI MCC.