3GPP TSG-RAN WG2 Meeting #129	R2-2501307

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.9.2

Source:	Ericsson

Title:	Support for store and forward in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusions from SA2 study report [1], the following objectives were defined in RAN for store and forward in the related work item [2]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

In this contribution, we aim to address the most pressing points.

# 2	Discussion

## 2.1	Satellite IDs in the NAS provided list

During RAN2#128, there was extensive discussion on the interpretation of the satellite ID list provided to the UE via NAS and the related UE behaviour. Some companies found the SA2 study inconclusive regarding whether the satellites in the list either support or operate in S&F mode when serving the UE. Consequently, it was agreed to send an LS to SA2 [3] to confirm the following understanding:

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

During SA2#166-e adhoc meeting, the RAN2 LS was addressed, but without reaching enough consensus to reply to the LS [4]. Most companies preferred that the S&F monitoring list serves as assistance information, and a CR to TR 23.401 was agreed, adding the following editor’s note [5]:

“It is for further consideration whether specific UE behaviour or clarification needs to be added to prevent the UE from accessing satellites where the NW is not expected to keep a UE context”.

The issue will continue to be discussed in the next SA2 meeting. Therefore, we recommend that RAN2 awaits further progress and input from SA2 before deciding on how the UE should use the NAS satellite list information within RAN protocols.

RAN2 should wait for further input from SA2 regarding the use of the satellite ID list and related UE behaviour.

## 2.2	New RRC Release cause

Another item for further study is the introduction of a new release cause due to the transition to S&F operation. While we agree that it may be necessary to inform the UE of the RRC disconnection cause due to S&F operation, and potentially prevent from further connections, it may also be managed by upper layers (NAS). SA2 is currently discussing transition scenarios between non-S&F and S&F modes. The main debate centres on whether a UE would need to Attach again or could trigger a TAU. We believe that understanding the higher-layer procedure is essential to determine whether a UE may maintain its RRC connection or needs to be released and initiate a new one. Therefore, we recommend waiting for further progress in SA2 before introducing this UE behaviour.

RAN2 should wait for further progress in SA2 before introducing a new release cause due to the unavailability of feeder link connectivity.

## 2.3	S&F transition scenario

In a few cases, operational changes (S&F/non-S&F) may coincide with the cell's stop serving time (t-Service), particularly when the satellite gains or loses feeder link connectivity near the minimum elevation angle (from the UE's perspective). However, in scenarios like Earth-moving cells, this is not feasible, and it is crucial for the UE to know when an operational change may occur. Following recent RAN2 agreements, the UE is aware of when the current satellite will regain feeder link connectivity and switch from S&F mode to non-S&F mode. Nonetheless, from a deployment perspective, both transitions should be considered: a cell in S&F mode may switch to non-S&F mode and vice versa. Therefore, RAN2 should enable the network to broadcast time information indicating when the satellite switches between non-S&F and S&F modes.

RAN2 should consider the transition scenario when a satellite operating in normal (non-S&F) mode changes to S&F during a cell serving time.

The UE should be informed of the remaining time in normal (non-S&F) mode via System Information broadcast.

During RAN2#128, several companies argued that the intended UE behaviour should be discussed alongside signalling. We foresee impacts on UEs in both RRC_CONNECTED and RRC_IDLE/INACTIVE modes due to the availability or lack of feeder link connectivity. Since a cell in S&F mode offers limited services, a UE may prefer to camp or connect to a different cell supporting end-to-end connectivity. If the network informs the UE of the remaining time in the satellite’s current S&F mode, we consider beneficial that the UE would use this information to initiate neighbour cell measurements regardless other existing measurement relaxation conditions (e.g., t-Service).

The remaining time in S&F or normal mode should be used to trigger neighbour cell measurements.

## 2.2	Neighbour and upcoming satellites

In their study [1], SA2 has concluded that the MME can provide, during the Attach or Tracking Area Update (TAU) procedure, a list of Satellite IDs over which the UE may later exchange signalling and data. A UE can determine a satellite’s ID from information provided by the eNB in SIB31/SIB32/SIB33. This mechanism allows the UE to identify which satellite(s) may have the necessary context to fulfill its connection requirements such as Attach or TAU. This information is beneficial for the UE implementation as (together with the information in SIB32) it helps to plan optimal network access times, thereby saving energy by avoiding unsuccessful connection attempts.

- In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

- In NAS, the satellite ID list is based on the values present in system information and broadcast in RAN.

Similarly, it would be beneficial for the UE in AS level to have access to the operation mode of not only its serving satellite but also neighbouring and upcoming satellites. For neighbouring satellite, this information may be included in SIB33, along with their respective satellite ID. For upcoming satellites, this information could draw from synergies with the discontinuous coverage feature and be included in SIB32. This additional information can enhance the efficiency of various AS procedures, such as cell re-selection, paging, or PSM, and facilitate power-saving mechanisms up to UE implementation. For example, the UE that is moving and is located close to the cell border may prioritize the reselection to a cell which does not support S&F architecture.

- A UE may benefit from knowing the S&F operation status of neighbor satellites to steer mobility procedures.

During RAN2#128, it was discussed whether RAN2 should introduce a S&F-specific intraFreqAllowedCellList in SIB4 and interFreqAllowedCellList in SIB5. The purpose would be for the network to include S&F NTN neighbour cells so that Rel-19 UEs supporting S&F may perform measurement and cell reselection in a certain manner. However, the presence of both lists (S&F cells and non-S&F cells) would increase the size of the SIBs and the complexity of the RRC_IDLE mobility procedures specially when transition scenarios are considered. In contrast, we think that a solution more aligned with legacy behaviour would be to add new conditions to the cell reselection criteria to consider the satellite operation mode. In this way, the UE may permanently or temporally change the cell ranking criteria taking into consideration the remaining time in S&F operation mode of the serving and/or neighbour cells.

UE is informed of the (S&F/non-S&F) operation mode of neighbour and upcoming satellites.

The remaining time in S&F or normal (non-S&F) operation mode is used to as a condition in the cell reselection ranking criteria.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In NAS, the UE is provided with a list of satellites over which it may exchange signaling and data.

Observation 2	In NAS, the satellite ID list is based on the values present in system information and broadcast in RAN.

Observation 3	A UE may benefit from knowing the S&F operation status of neighbor satellites to steer mobility procedures.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should wait for further input from SA2 regarding the use of the satellite ID list and related UE behaviour.

Proposal 2	RAN2 should wait for further progress in SA2 before introducing a new release cause due to the unavailability of feeder link connectivity.

Proposal 3	RAN2 should consider the transition scenario when a satellite operating in normal (non-S&F) mode changes to S&F during a cell serving time.

Proposal 4	The UE should be informed of the remaining time in normal (non-S&F) mode via System Information broadcast.

Proposal 5	The remaining time in S&F or normal mode should be used to trigger neighbour cell measurements.

Proposal 6	UE is informed of the (S&F/non-S&F) operation mode of neighbour and upcoming satellites.

Proposal 7	The remaining time in S&F or normal (non-S&F) operation mode is used to as a condition in the cell reselection ranking criteria.

# 4	References

TR 3GPP TR 23.700-29 v19.0.0

RP-241624 - WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

R2-2411197, LS on Satellite IDs for store-and-forward operation, CATT, RAN2#128, Orlando, USA, November 2024.

S2-2500790r06, [DRAFT] Reply LS on Satellite IDs for store-and-forward operation, SA2, SA2#166-e, January 2025.

S2-2501339, Support of S&F Monitoring List in S&F Mode, Huawei, HiSilicon, Samsung , SA2#166-e, January 2025.