3GPP TSG-RAN WG2 Meeting #129	R2-2501308

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.9.4

Source:	Ericsson

Title:	Enhancements to support PWS in NB-IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

The Rel-19 WID on IoT-NTN [1] was updated in RAN#105 with the following additional objective:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

RAN2#128 has made several agreements. In this contribution we focus on some of the open issues.

# 2	PWS support for NB-IoT NTN

## 2.1	Time budget for the delivery of emergency notification

The prompt and reliable delivery of emergency notifications is critical for their effectiveness. Within 3GPP, both SA1 and RAN2 have recognized the challenges associated with meeting the stringent delivery requirements of PWS. A notable example is the ETWS primary notification, which is mandated to be delivered within 4 seconds. However, this requirement presents significant challenges in the context of NB-IoT devices, where the extended DRX periods could be as long as 3 hours. This discrepancy highlights the need for a thorough evaluation of current protocols and potential enhancements to ensure timely emergency notifications, even in constrained scenarios. Following a previous analysis in [2], we provide a detailed time budget for the delivery of the ETWS primary notification, comprising the following steps:

- Initial actions based on UE’s RRC mode:

  - RRC_CONNECTED:

    - Time to wait during C-DRX sleep mode depending upon its configuration.

    - Acquiring the Direct Indication message with up to 256 repetitions on NPDCCH.

  - RRC_IDLE:

    - Wait for at least a paging cycle in DRX. For simplicity, eDRX is not considered in this analysis.

- Acquisition of SIB1-NB to obtain the scheduling information of ETWS related SIBs (e.g., SIB10-NB).

- Acquisition of SIB10-NB which contains the ETWS primary notification.

For the first step, if the UE is in RRC_CONNECTED, the minimum and maximum values of the long DRX cycle are 256 and 10240 subframes, respectively. The longer values pose a challenge to the 4-second requirement. The acquisition of the Direct Indication may involve up to 32 NPDCCH repetitions in CE mode A and 2048 NPDCCH repetitions in CE mode B.

If the UE is in RRC_IDLE, the minimum and maximum values of the paging cycle are 128 and 1024 radio frames, respectively. The exact number of paging occasions within a paging frame depend on the specific network configuration. In any case, the longer values pose a challenge to the 4-second requirement. The scheduling of the paging message may involve up to 32 NPDCCH repetitions in CE mode A and 2048 NPDCCH repetitions in CE mode B. The acquisition of the paging message may involve up to 32 NPDSCH repetitions in CE mode A and 2048 NPDSCH repetitions in CE mode B.

In the second step, the acquisition of SIB1-NB with 16 repetitions may take less than 30.72 seconds in CE Mode A, and longer than 40.96 seconds in CE Mode B, while in normal coverage up to around 25 seconds [3]. We assume that the UE may start immediately the acquisition of SIB1-NB.

For the final step, the acquisition of SIB10-NB depends on the scheduling periodicity (64, 128, 256, or 512 frames) and the number of repetitions (every 2nd, 4th, 8th, 16th radio frames). Considering PWS SIBs are only scheduled during an emergency, we can assume that network configures a short periodicity in order to deliver them quickly. The contents of SIB10 would be less than 500 bits. Based on the simulations for SIB2 in [2], it can be estimated that this step may take less than 1.28s in CE Mode A, and less than 2.56s in CE Mode B.

Table 1. Time budget estimation for ETWS primary notification delivery time for RRC_CONNECTED mode.

|  | Long DRX | NPDCCH | SIB1-NB | SIB10-NB |
| --- | --- | --- | --- | --- |
| CE Mode A | 2.56 s | 0.032s | 30.72 s | 1.28s |
| CE Mode B | 2.56 s | 2.048s | 40.96 s | 2.56s |

The time budget estimation for the best-case scenario (RRC_CONNECTED) is summarized in Table 1. The results reveal two significant bottlenecks: DRX and SIB1-NB acquisition. Without enhancements, it may be impossible to meet the existing requirements for PWS notification delivery. In the following sections, we will discuss alternatives to address these bottlenecks within the context of both NTN and TN.

## 2.2	Enhancements to eDRX

Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases).

During RAN2#127bis, it was agreed to include a disclaimer on PWS performance in eDRX as follows:

While such a disclaimer is reasonable for specific scenarios, it is important to emphasize that PWS is a safety-critical mechanism and should not be treated as a best-effort service. This could lead to dangerous situations where users expect to receive warning messages, but the UE is not actively monitoring these notifications or changes in broadcast signaling during emergencies. Therefore, it is essential to explore potential optimizations to enhance PWS performance.

Investigate methods to improve the reception of PWS messages for NB- NTN UEs in challenging scenarios (e.g., eDRX).

In the initial commercial deployments of NTN, there is a clear preference for using NB-IoT to provide messaging and emergency services via satellite [4]. Although regular 5G NR UEs will continue to operate in terrestrial cells, they may also be equipped with NB-IoT NTN modems to cover areas lacking terrestrial network connections, such as remote regions. In this context, one of the foundational paradigms of NB-IoT—battery-constrained operation—may not be as critical as in previous releases. These UEs might prioritize better service delivery over energy-saving configurations.

- First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

An example of improved service delivery is the timely reception of PWS emergency notifications. As discussed earlier, this is challenging or even impossible when an NB-IoT UE employs certain power-saving mechanisms like eDRX. Therefore, it may be advantageous for such NB-IoT UEs to inform the network that they are not battery-constrained and are interested in specific services, such as PWS. This could involve extending existing NAS mechanisms to allows the UE to express its desired power-saving parameters (e.g., eDRX or PSM). Based on the UE's preferences, the network can adjust the assigned power-saving configurations to align with the UE's specific interests, such as receiving timely PWS notifications.

Allow a UE to indicate its interest in PWS services to the network.

## 2.3	SIB1-NB acquisition

During RAN2#127bis, the acquisition of SIB1-NB upon receiving an emergency message was briefly discussed and identified as an area requiring further study. In emergency situations, acquiring SIB1-NB is essential to understand the new System Information scheduling configuration, enabling the UE to determine how and when to acquire the emergency SIBs that contain the emergency notification details. Given the lengthy time required for this process (see Table 1 above), RAN2 should investigate ways to address this issue, particularly for time-sensitive cases like the ETWS primary notification. This is the ETWS primary notification. The requirements for delivering the ETWS primary notification are stringent, necessitating its arrival at the UE within 4 seconds. Therefore, efforts should focus on optimizing this system. Other notifications, such as the ETWS secondary notification, CMAS, or EU-alert, do not have such stringent requirements and can continue to use existing procedures for SIB acquisition.

RAN2 studies ways for allowing an NB-IoT UE to skip SIB1-NB acquisition to speed up the delivery of the ETWS primary notification.

## 2.4	Support of geofencing in ETWS

## 5.6	Additional PWS Requirements Specific to ETWS

Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN.

Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

In this meeting, RAN2 has received the LS response from SA1 [5] indicating the creation of geofencing requirements for PWS in NTN. The following changes have been introduced in SA1’s specifications [6].

Given the positive indication from SA1 and the inclusion of new requirements for both NR and IoT NTN, it is evident that RAN2 should implement geofencing for ETWS notifications in NTN deployments.

Introduce geographic information in ETWS notifications for geo-fencing in NTN cells.

The current geofencing mechanism for CMAS uses message segmentation to inform the UE of the target emergency are. The IE warningAreaCoordinatesSegment provided in SIB12 is encoded in octet strings, with its contents defined in [7]. This format is straightforward and can be easily adapted for ETWS minimizing the impact on RAN.

Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

In a simple scenario, an emergency area can be described using a single circle defined by the latitude and longitude of its center and the radius length. According to TS 23.041, this representation requires 11 bytes due to an additional byte for encoding the format's length. Given that the current size of an ETWS primary notification is 50 bits (as per SIB12's ASN.1 structure in TS 36.331), the total size, including geofencing, would be 138 bits, which fits into a single segment. Therefore, following the existing CMAS mechanism, we propose that RAN2 introduces the Warning Area Coordinates format to describe an emergency area in system information using existing ETWS SIBs, namely SIB10/11 (LTE) and SIB6/7 (NR).

- In some scenarios, an emergency area may be described with a single Warning Area Coordinates segment.

Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10/11 (LTE) and SIB6/7 (NR).

In more complex scenarios, a detailed description of the emergency area may require the use of polygons or multiple circles, necessitating several Warning Area Coordinates segments. This will inevitably increase the delay in acquiring the emergency notification. In such cases, a more efficient approach would be to introduce a new SIB with pre-configured geofencing information that the UE can acquire in advance. For instance, an operator could decide to divide a continent in several areas/regions. When an emergency happens, a UE may have already acquired the SIB with the emergency area description, and the primary notification would only need to include a pointer to the specific affected area. This approach would minimize the impact of geofencing evaluation on the delivery of primary or secondary notifications.

- When the emergency area description requires more complex shapes, multiple Waring Area Coordinates segments may be needed, potentially increasing the delay in delivering the emergency notification.

For complex emergency area descriptions, RAN2 should consider introducing a new SIB with pre-configured emergency areas.

## 2.5	PWS reception during initial access

During last meeting, RAN2 discussed on the possibility of supporting PWS in RRC_CONNECTED mode and subsequently sent an LS to RAN1 to assess its feasibility within the Release 19 timeframe [8]. Another aspect that remains unclear is whether a UE can receive PWS while performing initial access procedures. The transition from RRC_IDLE/INACTIVE to RRC_CONNECTED can be time-consuming, depending on radio conditions and the number of required repetitions. For instance, NPRACH allows for a maximum of 128 repetitions, and a single preamble repetition unit using Format 0 (for a cell radius of 10km) takes 5.6ms. Consequently, transmitting the preamble could take nearly a second. It would be beneficial for the UE to quickly receive emergency notifications during its network access attempts, without having to wait for the completion of the RRC mode transition.

RAN2 to discuss the feasibility of enabling the reception of PWS during initial access.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

Observation 2	In some scenarios, an emergency area may be described with a single Warning Area Coordinates segment.

Observation 3	When the emergency area description requires more complex shapes, multiple Waring Area Coordinates segments may be needed, potentially increasing the delay in delivering the emergency notification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Investigate methods to improve the reception of PWS messages for NB- NTN UEs in challenging scenarios (e.g., eDRX).

Proposal 2	Allow a UE to indicate its interest in PWS services to the network.

Proposal 3	RAN2 studies ways for allowing an NB-IoT UE to skip SIB1-NB acquisition to speed up the delivery of the ETWS primary notification.

Proposal 4	Introduce geographic information in ETWS notifications for geo-fencing in NTN cells.

Proposal 5	Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

Proposal 6	Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10/11 (LTE) and SIB6/7 (NR).

Proposal 7	For complex emergency area descriptions, RAN2 should consider introducing a new SIB with pre-configured emergency areas.

Proposal 8	RAN2 to discuss the feasibility of enabling the reception of PWS during initial access.

# 4	References

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.

R2-1910428, Ericsson, ETWS/CMAS notification delivery in connected mode for non-BL UEs in CE, RAN2#107, Czech Republic, May 2019.

R4-1609680, Ericsson, Simulation result of MIB-NB/SIB1-NB/SIB2-NB acquisition time, RAN4#81, USA, November, 2016.

https://samsungmagazine.eu/en/2024/04/19/samsung-oficialne-predstavil-svuj-prvni-5g-modem-s-obousmernym-satelitnim-pripojenim/

LS S1-244057/R2-2409243 on PWS support for NB-IoT NTN, Novamint, SA1#108, Orlando, USA, November 2024.

S1-244756, PWS for IoT NTN, Novamint, Qualcomm, SyncTechno Inc., InterDigital, SA1#108, Orlando, USA, November 2024.

ATIS-0700041, Wireless Emergency Alerts (WEA) 3.0: Device-Based Geo-Fencing.

R2-2411194, LS on PWS reception in RRC_CONNECTED, RAN2#128, Nov. 2024