3GPP TSG-RAN WG2 Meeting #129	R2-2501309

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	7.3.1

Source:	Ericsson

Title:	Location information reporting for NB-IoT

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has received an LS from SA2 about the need of UE location information for NB-IoT NTN.

1. Overall Description:

SA2 thanks RAN2 and RAN3 for the reply LS on UE Location Information for NB-IoT NTN. SA2 approved the attached CR which allows the MME, if requested from eNB, to send the coarse location information received from the UE to NB-IoT eNB.

SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

2. Actions:

To RAN2, RAN3:

ACTION: 	SA2 asks RAN2 and RAN3 to take the above information into account and indicate whether Coarse Location Information is needed by the eNB in the procedures defined in clause 5.3.4B.4, 5.3.5A and 5.3.5B.

# 2	Discussion

Following extensive LS exchanges in both Releases 17 and 18, RAN3 and SA2 have formalized a procedure that allows NB-IoT UE to report its coarse location via NAS signaling to the MME. This information can be subsequently forwarded from the MME to the eNB [1].

RAN3 agrees for NB-IoT NTN UE, the coarse UE location reporting from MME to eNB is performed upon eNB request, per UE, and only once for UE during RRC connected state.

Based on eNB request, MME will provide Coarse UE Location Information to the eNB for a given UE.

One of the primary obstacles in previous releases to reporting UE location directly to the eNB using Control Plane CIoT (DoNAS) was the lack of AS security. This solution originated from a specific use case involving an IoT NTN satellite operator's need for radio resource management, such as load balancing. RAN2 concluded that having UE location information at the eNB could be beneficial in this particular case, but there was no clear consensus on how or why it should be implemented.

- A NB-IoT NTN UE can report its coarse location via NAS to the MME, which will then forward it to the eNB once.

- This solution is tailored to a specific use case related to RRM configuration.

SA2 has indicated several other procedures in their LS, namely “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation”, “Connection Resume procedure”, and “MT-EDT procedure for User Plane CIoT EPS Optimisation”, and seeks to understand whether coarse UE location information would be beneficial or necessary in these contexts. In our assessment, it is not evident that NB-IoT UE coarse location would be useful for these other cases, based on the following premises ([2][3]):

- In this particular use case, load balancing, including beam selection from the NTN payload, was performed only at UE attach.

- An NB-IoT UE typically attaches, exchanges data, detaches, and then optionally moves before reattaching.

- The session duration is expected to be very short.

Consequently, while an NB-IoT UE could theoretically provide its Coarse Location Information directly to the eNB via the RRC protocol—as already supported in eMTC NTN—this would constitute a new feature rather than a maintenance issue. Currently, there is insufficient evidence on the need to justify adding this feature so late in the specification process.

NB-IoT coarse UE location is not needed for the procedures “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation”, “Connection Resume procedure”, and “MT-EDT procedure for User Plane CIoT EPS Optimisation”.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A NB-IoT NTN UE can report its coarse location via NAS to the MME, which will then forward it to the eNB once.

Observation 2	This solution is tailored to a specific use case related to RRM configuration.

Based on the discussion in the previous sections we propose the following:

Proposal 1	NB-IoT coarse UE location is not needed for the procedures “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation”, “Connection Resume procedure”, and “MT-EDT procedure for User Plane CIoT EPS Optimisation”.

# 4	References

[1]	R3-244652	Summary of Offline Discussion – Coarse UE Location Information for IoT NTN, Ericsson (moderator).

[2]	RAN3 #125bis Chair’s Notes.

[3]	R3-245686	Summary of Offline Discussion on NBIoT UE location, Qualcomm.