3GPP TSG-RAN WG2 Meeting #129         											                     R2-2501420

Athens, Greece, Feb 17th – 25th, 2025

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Report of [AT129][306][R19 IoT NTN] TX and RX window for CB-msg3

Document for:	Discussion and Decision

# Introduction

This is to discuss the following offline discussion in RAN2#129:

[AT129][306][R19 IoT NTN] TX and RX window for CB-msg3 (Mediatek)

Scope: discuss details of transmission window (e.g. sliding or fixed), monitoring window and whether 1 or multiple RNTIs should be considered for CB-msg3/DSA

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Thursday 2025-02-20 20:00

Deadline for rapporteur's summary (in R2-2501420):  Friday 2025-02-21 08:00

In this discussion, the following terminology is used.

- Transmission window: time window for DSA CB-Msg3 resource selection

- Monitoring window: time window for response (CB-Msg4) reception

- RNTI: The RNTI used for schedule Msg4 transmission

Related agreements are copy below:

Transmission Window

- There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

- Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

RNTI

- The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

Monitor Window

- For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

- For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

Contention resolution

- The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

- UE early terminate

- Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

eNB early response

- RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

# Discussion

During the online discussion on CB-Msg3 in RAN2#129, it was found that the DSA transmission window, the Msg4 monitoring window, and the RNTI for Msg4 are closely related. The rapporteur suggested discussing these three aspects together. The intention of this offline discussion is to narrow down the combinations of these three aspects. Companies can further discuss the merits and defects of the options we have consensus on, and down select one of the options during the next round of discussions. The details of the option can also be discussed further after the option is selected.

Based on the contributions of the companies on CB-Msg3, we propose the following 2 options for further discussion. The options are combinations of DSA transmission window, the Msg4 monitoring window, and the RNTI for Msg4. The rapporteur believes these 2 options are the most representative.

- Option 1: Sliding DSA transmission window + multiple Msg4 monitoring windows + multiple RNTIs.

- Option 2: Fixed DSA transmission window + single Msg4 monitoring window + single RNTI.

Option 1

- Transmission Window

  - Starting Point: at the first replica that the UE randomly select from CB-MSG3 occasions.

  - Window length: is configured by NW (via SIB)

- CB-MSG3 Resource selection

  - Periodically CB-MSG3 occasions configured by NW (via SIB)

  - Assume that SA and DSA could use same resource pool

  - After selecting the first replica, the UE randomly select other replicas in the transmission window

- Monitor window

  - A monitor window is started for each Replica transmission respectively

  - After the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding monitor window, taking UE-eNB RTT into account

  - The length of the monitoring windows is configured by network in SIB. FFS the offset.

- RNTI

  - Calculated independently for each replica according to the resource selected (as in SA)

  - The UE must monitor multiple RNTIs at the same time (unless we could ensure non-overlapping window)

Option 2

- Transmission Window

  - Starting Point: configured by NW (e.g. H-SFN offset)

  - Window length and window periodicity: configured by NW

- CB-MSG3 Resource selection

  - Not sure that SA and DSA could use same resource pool

  - Periodically CB-MSG3 occasions configured by NW (via SIB)

  - The UE first selects a nearest DSA transmission window and then randomly select K replicas inside the window

  - (Q: could UE just select current DSA window and use less replicas in some case)

- Monitor window

  - A single monitor window is used

    - Alt-1 Started after first replica

    - Alt-2 Started after last replica

    - Alt-3 Started after first replica and re-restart after each replica

  - Window length is configured by NW (via SIB)

- RNTI

  - A single RNTI is used to monitor Msg4, which is calculated based on the location of the selected DSA transmission window.

  - Q: Does it imply that all UE select this DSA window should use the same RNTI

Comments to option 1

- E///: RTT should be carefully considered.

- ESA/QC: This is not DSA.

- E///:  Sliding window implies that every time eNB should wait enough offset time to reply replica.

- ZTE: periodicity between replica may not be correct.

Comments to option 2

- E///: DSA Window Periodicity is not always need. Could be continue resource.

- E///: Not require one RNTI.

- ESA: This has more NW control on when the UE could do the transmission

Other comments

- QC/ZTE: Separate transmission window and monitor window proposal

# Conclusion

Based on the offline discussion, the rapporteur has the following proposal

Proposal 1: For CB-Msg3 DSA transmission window design, RAN2 to discuss below two options:

- Option 1

  - Transmission window is started at the first replica that the UE randomly select from CB-MSG3 occasions. Window length is configured by network via SIB.

- Option 2

  - Transmission window is configured by network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity.

  - The UE first selects a nearest DSA transmission window and then randomly select K replicas inside the window.

Proposal 2: For CB-Msg3 DSA monitor window and RNTI design, RAN2 to discuss below two options:

- Option 1

  - RNTI is calculated for each Replica respectively according to the selected resource (same as SA).

  - After the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding monitor window, taking UE-eNB RTT into account.

  - The UE has to monitor multiple RNTIs in multiple monitor windows.

- Option 2

  - A single RNTI is used for all replicas and it is calculated based on the location of the selected DSA transmission window.

  - A single monitor window is used for all replicas.