3GPP TSG RAN WG2 #129                                                                         	  R2-2501446

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Summary of offline-205 for potential UE capability for FDD-FDD inter-band CA (CATT)

Agenda Item:	8.19

Document for:	Discussion and Decision

- Introduction

This is the summary of the following offline discussion:

[AT129][205][NR_Others] Questions to R4 on potential UE capability for FDD-FDD inter-band CA (CATT)

Scope: Discuss and form questions to R4 on UE capability for FDD-FDD inter-band CA

Intended outcome: Draft LS in R2-2501445 for CB, including our agreements and questions. Discussion summary in R2-2501446.

Deadline: Thursday 7PM

- Discussion

The discussion minutes are as below, and RAN2 decided to ask questions to RAN4 to further clarify the details of this UE capability.

Discuss on need of UE capability - Xiaomi do not support having the ue cap, since no UE will support this combination anyway. - HW think the intention of the R4 LS to let us introduce cap signalling, and even if no UE support this today we should be future proof. HW prefer to introduce signalling. CATT has same view. - QC not ready to introduce capability. QC not sure what the capability means, e.g., it could be full duplex in uplink and downlink. ZTE, vivo share this view. - Nokia think there could be the case some UE support simultaneous tx/rx in some part of those band. Nokia also ok for us to check further before doing the singaling. - vivo think even if we introduce the UE cap for now, network may still need to update to support scheduling of simultaneous tx/rx in those BCs. - China Telecom ok to introduce the UE cap, since it helps network to know and make the scheduling decision. - CATT think having UE cap help us to support more cases, e.g., more BCs, higher order of CA. CATT think detailed signalling design can FFS and if needed we can further ask R4. Ericsson, HW agree. - QC OK to check with R4. - OPPO want to check with R4 before agree anyway.

From Rapp perspective, we suggest to ask RAN4 the following questions based on the online discussion:

- What does the UE capability mean? E.g., it indicates whether simultaneous transmission of band X and reception of band Y is supported when there is an overlap between band X UL and band Y DL.

- Whether there is a case where a UE can support simultaneous tx/rx in some part(s) of those overlapped FDD bands.

- If a UE capability signalling is defined, what are the details of it, e.g., granularity.

Companies are invited to provide views on the questions RAN2 needs to check with RAN4.

| Company | Whether the questions above need to be checked with RAN4? | What other questions RAN2 needs to check with RAN4? | Comments |
| --- | --- | --- | --- |
| OPPO | We are open to ask those questions, yet the question can be refined (see comment) | For 1, it seems too obvious to ask. So 2 and 3 are more important, i.e., whether there is a case/scheme already now, and if so, what is the case/scheme. This is particularly related to 3: the related details seem not the granularity, but what the detailed scheme for simul-operation is. As commented by others online, e.g., whether it is based on functions like full-duplex, or requires some frequency domain gap/guard-subband (if so, how large it is and etc.). Those details causes real difference to the network behavior in terms of configuration/scheduling. And it would be good to limit to n5-n8 in order to be specific. |  |
| ZTE | The second question only | For the second question, we think we can confirm the second question for the R19, e.g. Whether there is a case where a UE can support simultaneous tx/rx in some part(s) of those overlapped FDD bands in R19. Whether there would be a new capability would depends on the Q2, thus the Q1/3 is not necessary at this stage. |  |

Based on the comments received in the table above and added in the draft LS directly, rapporteur proposes to ask the following questions to RAN4:

Question 1.	Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

Question 2.	If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

- Conclusion

Based on the comments received in the table above and added in the draft LS directly, we propose:

Proposal 1: Regarding UE capability for FDD-FDD inter-band CA, RAN2 asks the following questions to RAN4:

Question 1.	Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

Question 2.	If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?