3GPP TSG-RAN WG2 Meeting #129	R2-2501496

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.0.2.16

Source:	Nokia (Rapporteur)

Title:	Offline #013 on PDCP SN Gap Reporting

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT129][013][XR] PDCP SN gap reporting (Nokia)

Intended outcome: Agree on correct behaviour and then CR to capture the behaviour

Deadline:  Thursday

# 2	Contact Points

Participants in the email discussion are requested to complete the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Benoist Sébire | benoist.sebire@nokia.com |
| Ericsson | Nithin Srinivasan | nithin.srinivasan@ericsson.com |
| Qualcomm | Linhai He | linhaihe@qti.qualcomm.com |
| Lenovo | Joachim Löhr | jlohr@lenovo.com |
| Apple | Ping-Heng Wallace Kuo | pingheng_kuo@apple.com |
| Xiaomi | Yujian Zhang | zhangyujian@xiaomi.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| CATT | Hao Xu | xuhao@catt.cn |
| ZTE | Eswar Vutukuri | eswar.vutukuri@zte.com.cn |
| MediaTek Inc. | Ming-Yuan Cheng | ming-yuan.cheng@mediatek.com |
| Fujitsu | Sue Yi | yisu@fujitsu.com |
| Canon | Yacine EL Kolli | yacine.elkolli@crf.canon.fr |
| Huawei, HiSilicon | Dawid Koziol | dawid.koziol@huawei.com |
| Samsung | Vinay Shrivastava | shrivastava@samsung.com |
| vivo | Chenli | Chenli5g@vivo.com |
| OPPO | Zhe Fu | fuzhe@OPPO.com |

# 3	Discussion

The setting of the bits of the bitmap of the PDCP SN Gap Report is described in section 5.16.1 of 38.323:

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

Several contributions raised a possible issue with the setting of the bits [R2-2500181] [R2-2500928] [R2-2500492] [R2-2501011]. Essentially, the question is whether the condition specified in the trigger (that the discarded PDCP SDU(s) have not been submitted by the RLC to the lower layers) should also be considered when setting the bits, or if the apparent flexibility that the current specification offers is preferred. Let us first discuss whether the setting of the bits of the bitmap should be deterministic and if so, how should it be specified.

Question 1: Should the setting of the bits in the bitmap be strictly deterministic?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Ericsson | Yes | Indeterministic behaviour leads to race conditions as explained in the next response. Test cases can only be defined based on deterministic outcomes. Thus, triggering and reporting should go hand-in-hand. |
| Qualcomm | Yes | Transmitter and receiver need to have consistent understanding on which PDUs are discard. |
| Lenovo | Yes | Deterministic behaviour should be defined to have consistent understanding in Tx and Rx as outlined by QC. Also to derive meaningful test cases in RAN5 we think that a deterministic behaviour should be defined. This is also inline with other Status/control PDUs in RLC/PDCP |
| Nokia | Yes | The apparent freedom that the specification offers in not a conscious design choice but an oversight. This needs to be corrected for the bitmap to be always useful and for RAN5 to be able to design a test case. |
| Apple | Yes | We think it is better to have a clear guideline in the specification, to reduce the ambiguity in terms of implementation. |
| Xiaomi | Yes | Our understanding is that the bitmap is only useful if it is deterministic. If different implementation sets the bitmap in different manners, the receiver would be confused on how to utilize the SN gap report. |
| Futurewei | Yes | We also prefer NOT to leave the ambiguity to implementation. |
| LGE | Yes | Bitmap setting is already clear in the specification. - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; |
| CATT | Yes | Deterministic behaviour is required which can be further clarified to make the specificaiton more clear. |
| ZTE | Yes | Based on the Rel-18 specification, only the discarded PDCP SDU(s) that have not been submitted by RLC to lower layers can trigger PDCP SN Gap report. Thus, to set the FDC field, only the discarded PDCP SDU(s) that have not been submitted by RLC to lower layers should be considered. |
| Samsung | Yes | Current spec is clear and true status of the “discard” for the SDUs is indicated. |
| MediaTek | Yes | Agree with LG, current spec is already clear. |
| Fujitsu | Yes | Agree with QC. |
| Canon | Yes | We should avoid specification ambiguity |
| Huawei | Yes | We prefer having a deterministic behaviour. |
| vivo | Yes | Deterministic behaviour is required and some clarification is needed. |
| OPPO | Yes | Deterministic behaviour is necessary to ensure an aligned understanding of the discard status between the transmitter and the receiver; otherwise, the expected benefit of having PDCP SN gap reporting cannot be achieved. |

Summary 1: All companies prefer to have a deterministic behaviour.

Proposal 1: The specifications need to capture a setting of the bits of the bitmap of the PDCP SN Gap Report that is deterministic.

For those having answered YES to the first question, the two choices are:

A.	always exclude the discarded PDCP SDU(s) that have been submitted by the RLC to the lower layers when indicating discarded PDCP SDU(s) via the bitmap i.e. set the field to zero for those SDU(s).

B.	always include the discarded PDCP SDU(s) that have been submitted by the RLC to the lower layers when indicating discarded PDCP SDU(s) via the bitmap i.e. set the field to one for those SDU(s).

Question 2: Which approach to specify?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | A or B | Technical Arguments |
| Ericsson | A | In a race condition where the PDCP gap report is received first, the corresponding PDCP PDU even if successfully received is discarded. This in a non-PDU set case is an error. Even in the PDU set case, any data successfully received should be forwarded to the application. In addition, it is likely that the PDCP gap report is prioritized hence resulting in this likely outcome. |
| Qualcomm | A | Triggering and reporting should be consistent. |
| Lenovo | A | Same as QC |
| Nokia | A | This is consistent with the trigger and with the intention of the bitmap. |
| Apple | A | If the UE would include those in-flight SDUs in the SN gap report, then RX_DELIV will be updated at the PDCP receiver based on such report. This means many SDUs may get transmitted completely in vain, as they would be out of the receive window and will be discarded by the PDCP RX. |
| Xiaomi | A | This is consistent with the triggering of PDCP SN gap report. In Rel-18, if a PDCP SDU has been submitted by RLC to the lower layer, it is transmitted in lower layer even if there is PDCP discard indicator. |
| Futurewei | Maybe B | During on-line discussion, we suggested that we are OK with Nokia’s TP for CR (which is basically A), because it leaves no room for other interpretations. However, after hearing Samsung’s comment, we have the following question now: if there is a PDCP SDU, which PDU has been submitted to RLC and is now discarded in the same batch as other PDCP SDUs that have not been submitted to RLC, if the PDCP SN gap report does not cover it, there is no guarantee that the RLC layer will transmit it (e.g. the RLC layer receives the discard indication from PDCP and is able to discard the RLC SDU before any transmission of it can begin, then in the case, a SN gap will still be created at the receiving side, which will still cause the unnecessary re-ordering. Isn’t that right? In addition, in Rel-19, it looks like we will further enhance the RLC layer to avoid unnecessary transmissions of a pending RLC retransmissions or of any un-transmitted segment, for which a discard indication has been received from PDCP, the chance that such PDUs will not be transmitted at the RLC layer will be increased. So, B seems to be more forward compatible with Rel-19 RLC enhancements for XR. We understood the comment that transmission may be wasted if the PDCP SN gap report covers them. But these PDUs are indeed discarded at the TX’er side, aren’t they, which means that the TX’er has deemed that they are no longer needed. Then, what is the harm? Hopefully, the Rel-19 RLC enhancements will reduce the chance of such wasteful transmissions. |
| LGE | None | The UE behaviour does not need to be mandated. Referring to Figure 1 in CATT’s paper: There is no problem if the FDC is set to SDU2. The issue is whether the FDC can be set to SDU1. We think there is no problem to set the FDC to SDU1. As the SDU1 is already discarded in the transmitter due to PDB, even if the receiver already receives the SDU1, the receiver does not need to deliver it to upper layer. |
| CATT | A | As we shared, the third condition for SN Gap report triggering was introduced to avoid the race condition happen. We share the view that this condition should also be followed when we do the SN Gap compling progress. Follow the reply from LG above, if we allow that the SDU1 can be included in the SN Gap report, that is to say, we allow the discarded PDCP SDU included in the PDCP SN gap report can include the PDCP SDU(s) which have been delivered by RLC to lower layers. This will cause more ambiguity in the current specification and introduce another new issue: how to set the FDC filed? In the following Figure, besides the SDU1 of PDU Set1, there may be lots of discarded SDUs which had been delivered from RLC to low layer (In the box in the top left corner). The scope of determining the smallest COUNT value of the discarded PDCP SDU which had been delivered from RLC to low layer is not captured in the current specification, hence it is hard to decide how to set the FDC field. For example, SDU1 of PDU Set x had been discarded and delivered from RLC to low layer for a long time ago, it is doubt that set the COUNT value of SDU1 of PDU Set x into the FDC field. |
| ZTE | A | Triggering and reporting should be consistent. |
| Samsung | Refer to comment | Reporting should reflect the true status about the discard for the SDUs. If SDU is discarded it is set to 1 and if SDU is not discarded, it is set as 0. We do not need to report discard status differently for same SDU(s) based what which discarded SDUs triggered the gap reporting. Further, with discardTimerLowImportance for discard, there may be out of order discard is possible and gap report bitmap being contiguous, same SDU discard status may need to be reported again. |
| MediaTek | A | It is better to decouple MAC and RLC behavior, the discarded PDCP SDU(s) that have been submitted by the RLC to the lower layers, could still under HARQ retransmission. Even Rx side successfully receive those SDUs, upper layer (RLC, PDCP) still can handle that (discard if out of window or stored in the buffer), not a critical issue. |
| Fujitsu | A | We think by default the triggering and reporting criteria are the same. We have never discussed/agreed that they might be different. |
| Canon | B | Sometimes it is OK to have a difference between triggering and reporting content. For example, R19 DSR have triggering threshold different from reporting thresholds. If the transmitter detects that a PDU is outdated then it should inform the receiver even if the PDU may have been correctly received. Indeed, although correctly recevied the outdated PDU is not useful to the receiver. |
| Huawei | A | Since those PDCP SDUs that have been provided to lower layers still have a chance to be received successfully by the other end, they should not be indicated in the gap report. If they were, the receiver may unnecessarily proceed the receiving window leading to loosing these SDUs unnecessarily. |
| Vivo |  | We have the same understanding as LGE, i.e. there is no problem to set the FDC to SDU1. But if we really want to address this issue (even we think it is not needed), triggering and reporting should be consistent, i.e. A should be adopted. |
| OPPO | A | When one SDU has already been delivered from the RLC layer to the lower layer, the receiver has the opportunity to obtain it. This reception scenario is different from the case where an SDU has not yet been submitted by the RLC, thus the value setting should be different. |

Summary 2: out of the 17 companies which responded, a large majority (12) prefer to specify behaviour A. Two companies expressed a preference for specifying behaviour B, and two other companies believe the current specification is clear (one of which would prefer to have A captured if a clarification is deemed necessary).

Proposal 2: Specify behaviour A i.e. always exclude the discarded PDCP SDU(s) that have been submitted by the RLC to the lower layers when indicating discarded PDCP SDU(s) via the bitmap i.e. set the field to zero for those SDU(s).

# 4	Conclusion

The following two proposals were made:

Proposal 1: The specifications need to capture a setting of the bits of the bitmap of the PDCP SN Gap Report that is deterministic.

Proposal 2: Specify behaviour A i.e. always exclude the discarded PDCP SDU(s) that have been submitted by the RLC to the lower layers when indicating discarded PDCP SDU(s) via the bitmap i.e. set the field to zero for those SDU(s).