3GPP RAN2-Meeting #129	R2-2501588

# Athens, Greece, 17th – 21st February, 2025	was R3-250773

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0970 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 correction for network slice support during XN handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Ericsson, JIO Platforms (JPL), AT&T, Rakuten, FiberCop, InterDigital, Deutsche Telekom, NTT DoCoMo, Nokia, Huawei, Samsung, ZTE, CMCC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Stage 2 description, clause 16.3.4.5, it is mentioned that “Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.”. However, clause 8.2.1.3 in TS 38.423 specifies that “If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node.”. It is also understood that as part of Xn Setup procedure, the source and target nodes exchange their Slice Support List as part of the TAI Support List IE. With regard to a handover procedure where there is partial or complete Slice Support List overlap between a source and a target node, it may be the case that no PDU sessions on supported S-NSSAIs may be admitted at the target, and therefore a HANDOVER PREPARATION FAILURE may be triggered. However, when the Slice Support List between the source and the target nodes have no overlap, attempting an XN handover will always result in a HANDOVER PREPARATION FAILURE, and therefore attempting a handover when it is already known that the slices are not supported at the target is unnecessary. Therefore, it is proposed to correct the current Stage 2 description to reflect that Xn handovers are only allowed when at least one slice configured for the UE is supported at the target NG-RAN node. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sentence “Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.” is corrected for the Xn handover case to reflect that Xn handovers are allowed only when at least one slice (used by the UE) is supported at the target NG-RAN node. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has a limited impact on previous version of the specification because it applies corrections to the description of handover initiation that are already implied by the specification. This CR has an impact under functional point of view only for implementations that allow XN HO initiation even if none of the slices for PDU sessions are supported in the target node. This CR is BC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description in TS 38.300 and TS 38.423 will not be aligned and can be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-250773. |  |  |  |  |  |  |  |  |

CHANGES START

## 16.3.4	Signalling Aspects

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 16.3.4.5	Mobility

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

NG handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node. Xn handovers are allowed only when at least one of the slices with active PDU sessions for the UE at the source gNB is supported at the target gNB. The source gNB performs this check as part of the Handover Preparation. An example for the case of connected mode mobility across different Registration Areas is shown in Figure 16.3.4.5-1 for the case of NG based handover and in Figure 16.3.4.5-2 for the case of Xn based handover.

CHANGES END