3GPP TSG-RAN WG2 Meeting #129bis	R2-2501763

Wuhan, China, April 7-11, 2025

Agenda item:	8.7.4.2

Source:	Qualcomm Incorporated

Title:	Discussion on DSR enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss two remaining issues on DSR enhancements: DSR cancelation in DC and how to report control PDUs and retransmission data in DSR.

# Discussion

## DSR cancelation in DC

At the RAN2#129 meeting, RAN2 discussed Rel-18 DSR cancelation in DC configuration. The concern was that DSRs are triggered and canceled per MAC entity whereas PDCP SDUs are managed per UE. Companies had divergent views on whether any changes are needed. The conclusion after online and offline discussions was that no clarification is needed for Rel-18.

In this paper, we discuss whether any change is needed in Rel-19.

There are three cases for DSR cancelation (which are the same for Rel-18 and Rel-19):

- When all PDCP SDUs associated with a pending DSR have been discarded;

- When a MAC PDU is sent and this MAC PDU includes all the PDCP SDUs associated with a pending DSR.

- When a DSR MAC CE is sent and this DSR MAC CE includes delay status information of all the PDCP SDUs associated with a pending DSR;

In the following, we analyze UE behaviors per current spec/agreements in the above three cases.

Case 1.

Since PDCP SDU are managed per UE, both MAC entities should be aware of the new delay status when PDCP SDUs are discarded. Therefore, pending DSR in both MAC entities are canceled when PDCP SDUs are discarded. The current spec works fine in this case.

Case 2.

When a MAC entity receives a UL grant and performs a PUSCH transmission are local to a MAC entity. WLOG, suppose all the PDCP SDUs associated with a pending DSR are sent in MAC entity #1. Unless inter-CG coordination within the UE is available, MAC entity #2 is not visible to this change, until it receives an UL grant itself. When it is about to assemble a DSR MAC CE, it can discover that all the PDCP SDUs associated with its pending DSR no long exits. Then it should cancel its own DSR, similar to Case 1.

As we can see from the above analysis, the current spec works fine in this case too.

Observation 1. 	In DC, no enhancements are needed when a pending DSR is canceled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

Case 3.

When delay status is reported in a DSR MAC CE, there is no impact on the PDCP SDUs. Therefore, unless one MAC entity can inform the other of its report, the other MAC entity will still send its own DSR MAC CE. This may result in duplicated reporting to network.

It is to be discussed whether this duplicated reporting needs to be avoided or not. On one hand, DSR MAC CE sent to both networks can help UE get UL grants sooner. On the other hand, it may result in resource waste for network, if a DC configuration is not capable of inter-gNB coordination.

Another aspect to consider is whether a UE is capable of cancelation pending DSRs in one MAC entity due to transmission of DSR MAC CE in another MAC entity. For example, this coordination may not be a problem if MN and SN are in the same band and supported by the same modem module. On the other hand, it would be challenging if MN and SN are in different bands or frequency ranges, because they likely are supported by two different modem modules.

In either case, we think it is useful for network to be aware of UE’s capability in coordinating DSR cancelations between two MAC entities. On top of that, it can be left to network configuration whether transmission of a DSR MAC CE in one MAC entity should cancel the associated DSRs in both MAC entities.

Proposal 1.	For Rel-19, define an optional UE capability, which is per band combination, for the support of canceling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 2.	For UEs that support the capability defined in Proposal 1, network configures UE whether it shall cancel pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

The above proposals are for Rel-19. Similar analysis applies to Rel-18 too, because the DSR cancelation conditions are the same for both releases. However, since Rel-18 is already frozen, network configuration and UE capabilities can’t be defined any more. Instead, whether to have coordinated DSR cancelation across both MAC entities can be left to UE implementation.

Proposal 3.	For Rel-18, make the following changes:

- Clarify in the normative text that pending DSRs in both MAC entities are cancelled when the corresponding DSR cancelation conditions are met in any MAC entity.

- Add a note to allow UE implementation whether to support cancelling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 4.	Adopt the TP in the Appendix if Proposal 3 is agreed.

## Special cases in data volume calculation

At the RAN2 #129 meeting, an offline discussion [AT129][504] was held to determine how PDCP/RLC control PDUs and retransmission data in Rel-19 DSR MAC CEs. Based on the comments, the rapporteur has proposed the following options that we can further discuss [1]:

Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

Among these three options, we think the same principle can be applied to PDCP and RLC. No different UE behaviors are required because the same buffering rules are applied to control PDUs and retransmission data.

Concerns on a special case that only control PDUs and retransmission data are present were raised in the offline discussion. Since those types of PDUs do not have explicit remaining time themselves, it is a good idea for them to use a special value (e.g. “0”) as their “remaining time” when reported in a DSR MAC CE. This approach can help network easily identify the amount of control PDUs and retransmission data. For UE, it is simple to implement.

From UE implementation perspective, it is desirable to have a single UE behavior that works well in all scenarios, i.e. regardless of whether data PDUs are present or not.

- When there are no data PDUs in the buffer, network can tell from the special value that only control PDUs and/or retransmission data are in the buffer. It then schedules them accordingly (e.g. asap).

- When there are data PDUs in the buffer, the remaining time of the first included reporting threshold in received DSR MAC CE gives network an upper-bound on how soon it should schedule control PDUs and how much control PDUs and retransmission data it needs to schedule before the data PDUs.

The only downside is that whenever there are control PDUs and/or retransmission data, a DSR MAC CE needs to report two extra bytes for them. If this small overhead is a concern, then UE may report different remaining time depend on whether data PDUs are in the buffer:

- When there are data PDUs in the buffer, control PDUs and retransmission data are reported together with the data PDUs associated with the first included reporting threshold.

- Where there are no data PDUs in the buffer, UE reports a special value (e.g. “0”) as the remaining time of control PDUs and retransmission data.

This option has lower overhead but a bit more complexity.

Proposal 5.	Discuss and adopt one of the following options:

- Alt 1. UE always reports a special value (e.g. “0”) for the remaining time of control PDUs and retransmission data, regardless of whether there are data PDUs.

- Alt 2. UE reports a special value (e.g. “0”) for the remaining time of control PDUs and retransmission data when there are no data PDUs. Otherwise, UE reports control PDUs and retransmission data together with the data PDUs associated with the first included reporting threshold.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

DSR cancelation in DC

Observation 1. 	In DC, no enhancements are needed when a pending DSR is canceled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

Observation 2.	In DC, UE may cancel a pending DSR only within an individual MAC entity if the cancelation is due to transmission of a DSR MAC CE, unless the UE is capable of dynamically coordinating between its two MAC entities.

Proposal 1.	For Rel-19, define an optional UE capability, which is per band combination, for the support of canceling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 2.	For UEs that support the capability defined in Proposal 1, network configures UE whether it shall cancel pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 3.	For Rel-18, make the following changes:

- Clarify in the normative text that pending DSRs in both MAC entities are cancelled when the corresponding DSR cancelation conditions are met in any MAC entity.

- Add a note to allow UE implementation whether to support cancelling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 4.	Adopt the TP in the Appendix if Proposal 3 is agreed.

Special case in data volume reporting

Proposal 5.	Discuss and adopt one of the following options:

- Alt 1. UE always reports a special value (e.g. “0”) for the remaining time of control PDUs and retransmission data, regardless of whether there are data PDUs.

- Alt 2. UE reports a special value (e.g. “0”) for the remaining time of control PDUs and retransmission data when there are no data PDUs. Otherwise, UE reports control PDUs and retransmission data together with the data PDUs associated with the first included reporting threshold.

# Appendix – TP for Rel-18 DSR cancelation

### 5.4.9	Delay status reporting

### (text omitted)

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted in any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

NOTE x:	For a band combination in which the UE does not support dynamic coordination of DSR cancelation between two MAC entities, when UE cancels a pending DSR in one MAC entity due to transmission of a DSR MAC CE, it is not required to cancel the corresponding DSR in the other MAC entity.