3GPP TSG-RAN WG2 Meeting#129bis	R2-2501790

Wuhan, China, 7th – 11th Apr. 2025

Source:	vivo

Title:	Simulation results of model generalization on cell configuration

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

At RAN2#129 meeting, RAN2 made the following agreements about simulation assumption of model generalization on cell configuration [1]:

Agreements 1: two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. 2: FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). 3: agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 2: generalization parameters for FR1 Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 3: generalization parameters for FR2

In this contribution, we provide our simulation results and analyses of model generalization on cell configuration.

- Discussion

  - FR1 intra-frequency temporal domain

Based on our previous simulation results, for the FR1 intra-frequency temporal domain, sub-use case 1 achieves better performance than sub-use cases 2 and 3. Thus we carry out the generalization simulation toward sub-use case 1 for FR1. Some key simulation parameters are shown in the following table:

Table 2.1-1: Key parameters of generalization study on cell configuration in FR1

| Parameter | Values |
| --- | --- |
| Sub-use case | 1 |
| L1/L3 filtering | Non-sliding |
| OW length | 10*200=2000 ms |
| PW length | 200 ms |
| MRRT | 50% |
| Pattern |  |

The simulation results of FR1 for Case B are illustrated in Table 2.1-2.

Table 2.1-2: Simulation results of generalization study for Cell configuration in FR1

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | UMi | UMa |
| --- | --- | --- |
| UMi | 0.558 | 0.498 |
| UMa | 0.577 | 0.424 |
| Mixed data | 0.559 | 0.428 |

  - FR2 intra-frequency temporal domain

Based on the agreed observation, for the FR2 intra-frequency temporal domain, sub-use case 2 achieves better performance than sub-use cases 1 and 3. Thus we carry out the generalization simulation toward sub-use case 2 for FR2. Some key simulation parameters are shown in the following table:

Table 2.2-1: Key parameters of generalization study for UE speed in FR2

| Parameter | Values |
| --- | --- |
| Sub-use case | 2 |
| L1/L3 filtering | Sliding |
| OW length | 5*80=400 ms |
| PW length | 5*80=400 ms |
| Pattern |  |

The simulation results of FR2 for Case A are illustrated in Table 2.2-2.

Table 2.2-2: Simulation results of generalization study for Cell configuration in FR2

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | UMi | UMa |
| --- | --- | --- |
| UMi | 1.127 | 0.915 |
| UMa | 1.187 | 0.889 |
| Mixed data | 1.151 | 0.878 |

  - Analysis of Cell Configuration

Based on the results in FR1 and FR2 respectively in Table 2.1-2 and Table 2.2-2, it can be observed that the variation in generalization performance between GC and Baseline are similar. Since the PW length of FR2 for Case A is longer leading to decreased prediction accuracy, to better to illustrate the impact of configuration on generalization, FR2 is selected for analysis.

In this section, we illustrate the generalization cases of Dimension 1 and Dimension 2 towards different cell configurations for GC#1, where two dimensions are demonstrated in Table 2.3-1:

Table 2.3-1: Two dimensions of model generalization study on GC#1

|  | Model generalization study Dimension 1 | Model generalization study Dimension 2 |
| --- | --- | --- |
| Training dataset | Training datasets from multiple configurations | Training dataset from a single configuration |
| Testing dataset | Testing dataset from a single configuration | Testing datasets from multiple configurations |

Dimension 1 of GC#1: model trained in different configurations and tested in the same configuration

The generalization cases of Dimension 1 for cell configuration is described in Table 2.3-2.

Table 2.3-2: Generalization cases of Dimension 1 for cell configuration

|  | Training @Dataset: UMi | Training @Dataset: UMa | testing Dataset @UMi | Testing Dataset @UMa |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 |  | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |
| Baseline |  | Yes |  | Yes |
| GC#1 | Yes |  |  | Yes |
| GC#2 | Yes | Yes |  | Yes |

Table 2.3-3 illustrates the prediction accuracy of baseline, GC#1 in Dimension 1.

Table 2.3-3: Simulation results of GC#1 for Cell configuration in Dimension 1

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Testing @ UMi | 1.127 (Training @ UMi) | 1.187 (Training @ UMa) |
| Testing @ UMa | 0.889 (Training @ UMa) | 0.915 (Training @ UMi) |

Based on the simulation results in FR2, we have the following observations:

- For the model testing in UMi, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.06 dB.

- For the model testing in UMa, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.026 dB.

GC#2: model trained in mixed configurations and tested in the same configuration

Based on Table 2.3-3, it can be observed that:

- For the model testing in UMi, the model performance of GC#2 (when trained with a mixed dataset) is slightly worse than that of Baseline, at 0.024 dB.

- For the model testing in UMa, the model performance of GC#2 (when trained with a mixed dataset) is slightly better than that of Baseline, at 0.011 dB.

Based on the result of GC#1 Dimension 1 and GC#2, we have the following observations and proposals:

Observation 1: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain cell configuration (UMa or UMi), the model trained from the dataset at different cell configuration (UMi or Uma) (GC#1) or mixed cell configurations (GC#2) can achieve comparable performance with the Baseline.

Proposal 1: For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across different cell configurations.

Dimension 2 of GC#1: model trained in a single configuration and tested in different configurations

The generalization cases of Dimension 2 for cell configuration is described in Table 2.3-4.

Table 2.3-4: Generalization cases of Dimension 2 for cell configuration

|  | Training @Dataset: UMi | Training @Dataset: UMa | Testing @Dataset: UMi | Testing @Dataset: UMa |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 | Yes |  |  | Yes |
| Baseline |  | Yes |  | Yes |
| GC#1 |  | Yes | Yes |  |

Table 2.3-5 illustrates the prediction accuracy of baseline, GC#1 in Dimension 2.

Table 2.3-5: Simulation results of GC#1 for prediction direction in Dimension 2

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Training @ UMi | 1.127 (Testing @ UMi) | 0.915 (Testing @ UMa) |
| Training @ UMa | 0.889 (Testing @ UMa) | 1.187 (Testing @ UMi) |

Based on the result of GC#1 Dimension 2, we have the following observations and proposals:

- For the model trained in UMi, the prediction accuracy of GC#1 increases compared with that of Baseline, at 0.212 dB.

- For the model trained in UMa, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.298 dB.

From our understanding, this is due to UMi scenario is more complicated than UMa, resulting in larger prediction errors for the intra-frequency temporal domain.

Based on the result, we propose:

Observation 2: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario.

Observation 3: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMa scenario shows worse prediction accuracy when tested in UMi scenario.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Generalization Study for Cell configuration

Observation 1: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain cell configuration (UMa or UMi), the model trained from the dataset at different cell configuration (UMi or Uma) (GC#1) or mixed cell configurations (GC#2) can achieve comparable performance with the Baseline.

Observation 2: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario.

Observation 3: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMa scenario shows worse prediction accuracy when tested in UMi scenario.

Proposal 1: For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across different cell configurations.

- Reference

- R2-2500559	Email discussion summary for [Post128][018][AI Mob] generalization  (Apple)