3GPP TSG-RAN WG2 Meeting #129bis	R2-25001791

Wuhan, China, Apr 7th – 11th, 2025

Source:	vivo

Title:	Simulation results of measurement event prediction and SLS

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

At the RAN2#129 meeting[1], RAN2 discussed the initial simulation results of indirect measurement event prediction based on temporal domain Case A and Case B, and made the following agreements:

Agreements on Measurement event case A

- Most of the results show that the F1 score for prediction of measurement event is very good.

- For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate.

Agreements on Measurement event case B

- Measurement event case B can have very good F1 score depending on filtering approach/PW

- F1 score is good for the PW window for case B (i.e. low PW)

- F1 score decreases with an increasing MRRT value

- We will focus on finding cases that bring benefits to the system rather than trying to compare

- With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

However, due to different inference modelings, the simulation results based on measurement event case A from different companies were diverse. To align the inference modeling, the following simulation assumptions were updated and clarified:

Agreements - For evaluation purpose, it is assumed PW length = TTT length =320ms - For evaluation purpose, Option 2 is agreed i.e. each counter can be updated only one time for related predicted and/or ground truth measurement event. - To introduce “Total number of HOF per UE per second” as SLS metric

In addition, the following agreement was made regarding direct measurement event prediction:

Agreements on direct measurement event prediction

-	F1 score for direct measurement is very good based on the current simulation results.

In this contribution, we will provide simulation results of indirect and direct measurement event prediction for temporal domain case A.

- Discussion

  - Indirect prediction of Temporal domain Case A

Based on the agreed observation, for the FR2 intra-frequency temporal domain Case A, sub-use case 2 achieves better performance than sub-use cases 1 and 3. Thus we carry out the measurement event prediction simulation with sub-use case 2 for Case A. Some key simulation parameters are shown in the following table:

Table 2.1-1: Key parameters of temporal domain prediction Case A

| Parameter | Values |
| --- | --- |
| Sub-use case | 2 |
| L1/L3 filtering | Sliding |
| L3 filtering | Option 1 |
| OW length | 5*80=400 ms |
| PW length | 4*80=320 ms |
| Pattern |  |

The measurement event specific parameters are illustrated in Table 2.1-2. To analyze the impact of RRM prediction accuracy on the event prediction, all the candidate UE speeds are simulated.

Table 2.1-2: measurement event specific parameters for Case A

| Parameters | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 60/90/120 |
| Max ETD (ms) | 80 |

    - Methodology

In indirect measurement event prediction for intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW via an RRM measurement prediction model. Afterward, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Based on the pattern in Table 2.1-1, 4 prediction instances are included within the PW length of 320 ms. On the other hand, the TTT is started after the entering condition of the associated event is fulfilled, and the condition should be fulfilled during the TTT to trigger the event. Therefore, 5 instances are needed to determine whether an event is triggered with the TTT = 320ms. As a result, 4 potential cases as illustrated in Figure 2.2-1 may happen when UE performs RRM measurement prediction:

Figure 2.1-1: Cases to perform measurement event prediction in Case A

- Case 1: 1 historical measurement instances already fulfill entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 4 instances.

- Case 2: 2 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 3 instances.

- Case 3: 3 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 2 instances.

- Case 4: 4 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first instance.

Another aspect is how to identify the gain of AI on the intermediate KPI. From our understanding, non-AI mechanisms can be compared as a benchmark. One of the simplest non-AI mechanisms is based on sample-and-hold. For Case A, the sample-and-hold mechanism can be: once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

Proposal 1: To evaluate the gain of AI-based event prediction, introduce a non-AI mechanism based on sample-and-hold as the benchmark. For Case A, the sample-and-hold mechanism can be: once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

In addition, at the RAN2#129 meeting, some companies’ simulation results reveal that F1 score does not have impact on the HO performance, which may be due to too-late inference of true positive cases, e.g., case 4 in Figure 2.1-1. To address the issue, one new metric was raised in the non-official email discussion:

- Average advance duration (ms): Average time difference between inference time of predict positive and ground truth positive.

We think the intention is valid, therefore, we propose:

Proposal 2: Introduce another metric to evaluate the gain of AI-based event prediction from timing perspective:

- Average advance duration (ms): Average time difference between inference time of predict positive and ground truth positive.

    - Results of Measurement Event Prediction

The simulation results of measurement event prediction for Case A are illustrated in Table 2.1-3.

Table 2.1-3: Simulation results of Indirect measurement event prediction for Case A

|  |  | Precision | Recall | F1 score | Average prediction accuracy（dB） | Average advance duration (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 60 km/h | AI | 0.939 | 1.0 | 0.968 | 0.870 | 319.03 |
|  | Non AI | 0.900 | 1.0 | 0.947 | 1.773 | 320 |
| 90 km/h | AI | 0.935 | 1.0 | 0.966 | 1.172 | 318.37 |
|  | Non AI | 0.869 | 1.0 | 0.930 | 2.187 | 320 |
| 120 km/h | AI | 0.936 | 1.0 | 0.967 | 1.360 | 314.53 |
|  | Non AI | 0.886 | 1.0 | 0.940 | 2.382 | 320 |

Based on the above simulation result, we observe that:

Observation 1: The indirect measurement event prediction based on temporal domain Case A prediction can achieve a better F1 score than the non-AI approach based on sample-and-hold.

In addition, with the results for different UE speeds, it can be observed that higher RRM measurement prediction accuracy results in better performance, e.g. Precision, Recall and F1 score.

Observation 2: For AI-based indirect measurement event prediction, higher RRM measurement prediction accuracy results in better performance (Precision, Recall and F1 score).

At the RAN2#129 meeting, some companies’ simulation results reveal that F1 score does not have impact on the HO performance, which may be due to too-late inference of true positive cases, e.g., case 4 in Figure 2.1-1. To address the issue, one new metric was raised in the non-official email discussion:

- Average advance duration(ms): Average time difference between inference time of predict positive and ground truth positive.

Based on the above simulation result, we observe that:

Observation 3: For AI-based indirect measurement event prediction, higher RRM measurement prediction accuracy results in better advance prediction duration.

The confusion matrix is provided in Table 2.1-4 for information.

Table 2.1-4: Confusion Matrix of Indirect Measurement Event Prediction for Case A

| AI-based prediction (60 km/h) |  | Ground Truth |  |
| --- | --- | --- | --- |
|  |  | Positive (413) | Negative (46) |
| Prediction | Positive | 413 | 27 |
|  | Negative | 0 | 19 |
| Non-AI (60 km/h) |  | Ground Truth |  |
|  |  | Positive (413) | Negative (46) |
| Prediction | Positive | 413 | 46 |
|  | Negative | 0 | 0 |
| AI-based prediction (90 km/h) |  | Ground Truth |  |
|  |  | Positive (490) | Negative (74) |
| Prediction | Positive | 490 | 34 |
|  | Negative | 0 | 40 |
| Non-AI (90 km/h) |  | Ground Truth |  |
|  |  | Positive (490) | Negative (74) |
| Prediction | Positive | 490 | 74 |
|  | Negative | 0 | 0 |
| AI-based prediction (120 km/h) |  | Ground Truth |  |
|  |  | Positive (527) | Negative (68) |
| Prediction | Positive | 527 | 36 |
|  | Negative | 0 | 32 |
| Non-AI (120 km/h) |  | Ground Truth |  |
|  |  | Positive (527) | Negative (68) |
| Prediction | Positive | 527 | 68 |
|  | Negative | 0 | 0 |

    - Results of SLS

At the RAN2#128 meeting[2], two handover modeling options below were provided for system-level simulation.

2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

The simulation results of SLS based on measurement event prediction for Case A are provided in Table 2.1-5.

Table 2.1-5: Simulation results of SLS based on measurement event prediction for Case A

|  | HOF rate | HO_num/UE/sec | HOF_num/UE/sec |
| --- | --- | --- | --- |
| Legacy HO | 5.98% | 0.3505 | 0.021 |
| AI-based HO (Option 2) | 5.11% | 0.3649 | 0.0186 |
| AI-based HO (Option 3 with Event-triggered inference) | 3.77% | 0.3705 | 0.014 |

In terms of total number of HOF per UE per second, the AI-based HO (temporal domain Case A) with HO modeling Option 3 (HO command based on predicted measurement event) outperforms HO modeling Option 2 (HO command based on actual measurement event).

Based on the above SLS result, we observe that:

Observation 4: The AI-based HO (temporal domain Case A) with HO modeling Option 3 (HO command based on predicted measurement event) outperforms HO modeling Option 2 (HO command based on actual measurement event) in terms of total number of HOF per UE per second.

  - Direct prediction of Temporal domain Case A

    - Methodology

In the section, the simulation results are provided to compare the performance of indirect and direct predictions. Some key simulation parameters are shown in the following table:

Table 2.2-1: Key parameters of direct prediction

| Parameter | Values |
| --- | --- |
| OW length | 400ms |
| PW length | 320ms |
| Input | History RSRP of serving cell + history RSRP of target cell + serving cell id + target cell id |
| Probability threshold | 80% |
| Pattern |  |

As to the measurement event specific parameters, the values in Table 2.1-2 are reused.

    - Results of Measurement Event Prediction

The confusion matrix is provided in Table 2.2-2.

Table 2.2-2: Confusion Matrix of Direct Measurement Event Prediction for Case A

| AI-based prediction (60 km/h) |  | Ground Truth |  |
| --- | --- | --- | --- |
|  |  | Positive (413) | Negative (46) |
| Prediction | Positive | 412 | 31 |
|  | Negative | 1 | 15 |
| Non-AI (60 km/h) |  | Ground Truth |  |
|  |  | Positive (413) | Negative (46) |
| Prediction | Positive | 413 | 46 |
|  | Negative | 0 | 0 |
| AI-based prediction (90 km/h) |  | Ground Truth |  |
|  |  | Positive (490) | Negative (74) |
| Prediction | Positive | 481 | 50 |
|  | Negative | 9 | 24 |
| Non-AI (90 km/h) |  | Ground Truth |  |
|  |  | Positive (490) | Negative (74) |
| Prediction | Positive | 490 | 74 |
|  | Negative | 0 | 0 |
| AI-based prediction (120 km/h) |  | Ground Truth |  |
|  |  | Positive (527) | Negative (68) |
| Prediction | Positive | 524 | 43 |
|  | Negative | 3 | 25 |
| Non-AI (120 km/h) |  | Ground Truth |  |
|  |  | Positive (527) | Negative (68) |
| Prediction | Positive | 527 | 68 |
|  | Negative | 0 | 0 |

The simulation results of direct measurement event prediction for Case A are illustrated in Table 2.2-3.

Table 2.2-3: Simulation results of direct measurement event prediction

|  |  | Precision | Recall | F1 score | Average advance duration (ms) |
| --- | --- | --- | --- | --- | --- |
| 60km/h | AI | 0.930 | 0.998 | 0.963 | 310.10 |
|  | non AI | 0.900 | 1.0 | 0.947 | 320 |
| 90km/h | AI | 0.906 | 0.982 | 0.942 | 308.83 |
|  | non AI | 0.869 | 1.0 | 0.930 | 320 |
| 120km/h | AI | 0.924 | 0.994 | 0.958 | 302.72 |
|  | non AI | 0.886 | 1.0 | 0.940 | 320 |

Note: probability threshold = 80%.

Based on the above simulation result, we observe that:

Observation 5: The direct measurement event prediction can achieve better F1 score than the non-AI approach based on sample-and-hold.

In addition, with the results for different UE speeds, it can be observed that higher UE speed results in lower performance, e.g. Precision, Recall, F1 score and advance prediction duration.

Observation 6: For direct measurement event prediction, higher UE speed will result in decreased performance (Precision, Recall, F1 score and advance prediction duration).

To evaluate the impact of probability threshold on the performance, the simulation results of direct measurement event prediction with different probability thresholds (50% and 80%) are illustrated in Table 2.2-4.

Table 2.2-4: Simulation results of direct measurement event prediction

(probability threshold comparison: 50% vs 80%)

|  | Threshold | Precision | Recall | F1 score | Average advance duration (ms) |
| --- | --- | --- | --- | --- | --- |
| 60km/h | 50% | 0.910 | 1.0 | 0.953 | 319.03 |
|  | 80% | 0.930 | 0.998 | 0.963 | 310.10 |
| 90km/h | 50% | 0.877 | 1.0 | 0.934 | 318.69 |
|  | 80% | 0.906 | 0.982 | 0.942 | 308.83 |
| 120km/h | 50% | 0.893 | 1.0 | 0.943 | 318.33 |
|  | 80% | 0.924 | 0.994 | 0.958 | 302.72 |

Based on the above comparison results, we observe that:

Observation 7: For direct measurement event prediction, the probability threshold has some impact on the prediction performance:

-   Precision increase when threshold increases;

-   Recall decreases when threshold increases;

-   F1 score increases when threshold increases;

-   Average advance duration decreases when threshold increases.

At the RAN2#129 meeting, some companies observed that F1 score for direct measurement is very good. We compare the F1 score of indirect and direct prediction, the simulation results are illustrated in Table 2.2-5.

Table 2.2-5: Simulation results of measurement event prediction

(indirect vs direct)

|  |  | F1 score | Average advance duration (ms) |
| --- | --- | --- | --- |
| 60km/h | Indirect | 0.968 | 319.03 |
|  | Direct | 0.963 | 310.10 |
| 90km/h | Indirect | 0.966 | 318.37 |
|  | Direct | 0.942 | 308.83 |
| 120km/h | Indirect | 0.967 | 314.53 |
|  | Direct | 0.958 | 302.72 |

Note: for direct prediction, probability threshold = 80%

Based on the above comparison results, we observe that:

Observation 8: Indirect measurement event prediction shows better F1 score and advance prediction duration than direct prediction (probability threshold = 80%)

In addition, the model of direct measurement prediction may have generalization issue, e.g., only applicable at specific network configuraton. Besides, the model of direct measurement prediction can not reuse the model for RRM measurement prediction.

Based on the above considerations, we suggest focusing on indirect measurement event prediction in Rel-19 and deprioritizing the study of direct prediction.

Proposal 3: Only consider indirect measurement event prediction in specification impact analysis, i.e., exclude direct measurement event prediction.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Indirect prediction of Temporal domain Case A

Methodology

Proposal 1: To evaluate the gain of AI-based event prediction, introduce a non-AI mechanism based on sample-and-hold as the benchmark. For Case A, the sample-and-hold mechanism can be: once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

Proposal 2: Introduce another metric to evaluate the gain of AI-based event prediction from timing perspective:

- Average advance duration (ms): Average time difference between inference time of predict positive and ground truth positive.

Results of indirect measurement event prediction for Case A

Observation 1: The indirect measurement event prediction based on temporal domain Case A prediction can achieve a better F1 score than the non-AI approach based on sample-and-hold.

Observation 2: For AI-based indirect measurement event prediction, higher RRM measurement prediction accuracy results in better performance (Precision, Recall and F1 score).

Observation 3: For AI-based indirect measurement event prediction, higher RRM measurement prediction accuracy results in better advance prediction duration.

Results of SLS for Case A

Observation 4: The AI-based HO (temporal domain Case A) with HO modeling Option 3 (HO command based on predicted measurement event) outperforms HO modeling Option 2 (HO command based on actual measurement event) in terms of total number of HOF per UE per second.

Direct prediction of Temporal domain Case A

Results of direct measurement event prediction for Case A

Observation 5: The direct measurement event prediction can achieve better F1 score than the non-AI approach based on sample-and-hold.

Observation 6: For direct measurement event prediction, higher UE speed will result in decreased performance (Precision, Recall, F1 score and advance prediction duration).

Observation 7: For direct measurement event prediction, the probability threshold has some impact on the prediction performance:

-   Precision increase when threshold increases;

-   Recall decreases when threshold increases;

-   F1 score increases when threshold increases;

-   Average advance duration decreases when threshold increases.

Observation 8: Indirect measurement event prediction shows better F1 score and advance prediction duration than direct prediction (probability threshold = 80%)

Proposed way forward for direct measurement event prediction

Proposal 3: Only consider indirect measurement event prediction in specification impact analysis, i.e., exclude direct measurement event prediction.

- Reference

- R2_129_ChairNotes

- R2_128_ChairNotes