3GPP TSG-RAN WG2 Meeting #129bis	R2-2501861

Wuhan, China, 7 –11 April 2025

Agenda Item:	8.13.3

Source:	NEC

Title:	Control Plane aspects for Multi-hop Relay

Document for:	Discussion, Decision

# Introduction

In RAN2#129 meeting, a number of agreements were reached for the basic aspects of Control Plane for Multi-hop Relay (refer to Annex).

This paper discusses the open issues that were not resolved so far on CP aspects for Multi-hop Relay.

# Discussion

## Latency reduction for Approach-1

In the last meeting, during the discussion of Approach-2 based control plane procedure for multi-hop relay, large concerns was raised regarding the latency for the connection establishment for Remote UE.

In our opinion, the handling of the first UL RRC message forwarding is critical when weighing the latency for the connection establishment.

In this section, we intends to discuss the possible latency reduction mechanism for the first UL RRC message for Approach-1.

The current Rel-17 UL first RRC message may apply to the last Relay UE for Multiple Hop sidelink relaying but cannot apply to the first Relay UE and intermediate Relay UE, especially when the first Relay UE and/or intermediate Relay UE are not in RRC Connected state. More specifically, if first Relay UE and/or intermediate Relay UE are not in RRC Connected state, the first Relay UE and intermediate Relay UE needs to send their own first RRC messages and these RRC messages will be subject to forwarding. The question is in the case how the first Relay UE and intermediate Relay UE handle the forwarding for the Remote UE’s first UL RRC message and for its own first UL RRC message. In Rel-17, if the U2N Relay UE is not in RRC_CONNECTED, it will establish an RRC connection with the gNB before forwarding the first UL RRC message from Remote UE. Rel-17 mechanism basically requests the Relay UE to buffer the first UL RRC messages for Remote UE until it is RRC connected, which is suitable for single hop relay scenario. This Rel-17 mechanism may be reused for Rel-19 Multiple Hop sidelink relaying for simplicity. However, with the increase of hop(s) for sidelink relay, that mechanism will increase significantly the delay for the Remote UE to establish a RRC connection, which then downgrade the sidelink relay performance.

For Rel-19 Multiple Hop sidelink relaying case, one possibility can be the Relay UE always immediately forwards the first UL RRC message received from its downstream node (i.e., the SRAP Data PDU received from SL-RLC0) regardless of its RRC state, to its parent Relay UE via determined egress RLC channel (e.g. specified PC5 RLC channel). With this mechanism, if the Relay UE is not in RRC Connected state, the Relay UE will firstly forward the first UL RRC message received from Remote UE to its parent Relay UE. And then the Relay UE may send its own first UL RRC message (e.g. RRCSetupRequest) towards its parent Relay UE. Then the parent Relay UE can forward these RRC messages along with the relay path towards the gNB serving the last Relay UE. With this mechanism, the delay for the RRC connection procedure for Remote UE will be significantly reduced.

During the email discussion [POST129][402][Relay] Control plane approach 2. The similar mechanism was proposed for Approach-2: forwarding Remote UE’s RRC message from ingress SL-RLC0 to egress SL-RLC0 (w/o assigned local ID).

We think for Approach-1 we need to support the similar mechanisms for Remote UE’s first UL RRC message.

Proposal-1: Immediate first UL RRC messages forwarding at the first Relay UE and intermediate Relay UE is supported (i.e., forwarding Remote UE’s first UL RRC message from ingress SL-RLC0 to egress SL-RLC0), when they are not in RRC_CONNECTED state.

## Remaining issues of SI acquisition for Remote UE

During RAN2#129 meeting, lots of agreements reached for the system information acquisition for Remote UE for the case of multiple hop based sidelink relay. However there are two FFS as below:

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

It should be noted that the first Relay UE receiving the on demand SIB request cannot request the SIB transmission from the network by itself. The first Relay UE needs to forward the on demand SIB request to its upstream relay node, and the upstream relay node needs to keep forwarding the on demand SIB request until reaching the last Relay UE for the Remote UE. The forwarding is not based on SRAP protocol, but on the PC5 RRC message. In case of one hop based forwarding for SI request (i.e., Rel-17), it does not need to indicate the destination of the SIB request message, since the Relay UE is assumed to terminate this message. In case of multiple hop based forwarding(i.e., Rel-19), the intermediate Relay UE may need to know the destination of such message to make correct message forwarding over PC5 RRC. The destination can be expressed by the identity of the Remote UE.

In addition, the intermediate Relay UE may need to record the routing information for the Remote UE, since the intermediate Relay UE may need to rely on this routing information to forward back the request SIBs when it receives the request SIBs from its parent Relay UE.

Proposal 2:	The destination of on demand SIB request message (i.e., Remote UE ID) is included in the on demand SIB request message over PC5 RRC for Remote UE.

There may be Relay UE(s) in the relaying path that may temporally store a valid version of the SIB(s) requested by the Remote UE. We think that allowing the intermediate Relay UE to provide the available SIBs to Remote UE per its request can reduce signaling overhead over the air and also reduce the relay for the Remote UE to acquire the SIBs. But if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE, it may not terminate the SIB Request from Remote UE and continue to further forward the SIB request message to its parent Relay UE.

Proposal 3:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 4:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.

## SRAP Header Format

Following the discussion of last meeting, there is an FFS on the inclusion of Remote UE local ID.

In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.

We think, adding a remote UE’s L2 ID is important for the case that the Intermediate Relay UE forwards the Child Node’s (e.g. Remote UE’s) uplink RRC message and its own RRC message in parallel. In this case, remote UE’s L2 ID can be used to tell the source node of the RRC message.

Proposal 5: Remote UE’s L2 ID is included in the SRAP header on top of the current agreed fields.

## Link failure handling

In Rel17 U2N relay, the remote UE triggers the RRC re-establishment procedure when detecting PC5 RLF and it can suspend RLM if it is connected to gNB via relay UE. This basic principle of Rel-17 U2N relay PC5 failure handling may be applicable to multi-hop case.

Proposal 6:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

In addition, in Rel17 U2N relay, the radio link failure of relay UE will trigger the notification message transfer or PC5 link release. For the case of multi-hop, we believe, any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

Proposal 7:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Immediate first UL RRC messages forwarding at the first Relay UE and intermediate Relay UE is supported (i.e., forwarding Remote UE’s first UL RRC message from ingress SL-RLC0 to egress SL-RLC0), when they are not in RRC_CONNECTED state.

Proposal 2:	The destination of on demand SIB request message (i.e., Remote UE ID) is included in the on demand SIB request message over PC5 RRC for Remote UE.

Proposal 3:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 4:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.

Proposal 5: Remote UE’s L2 ID is included in the SRAP header on top of the current agreed fields.

Proposal 6:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

Proposal 7:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Annex: RAN2#129 for Control plane aspects for Multi-hop Relay

Agreements:

RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE.  The related FFS can be removed from the stage 2 description.

For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported.  The related FFS can be removed from the stage 2 description.

For the baseline solution, Rel17 SUI message and format is re-used.

No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 from its immediate child node during its own connection establishment and details will be clarified in stage 3 (i.e., follow legacy U2N procedure from Rel-17 to set up the RLC channel for SRB1).  The related FFS can be removed from the stage 2 description.

Agreements:

For system information acquisition in multi-hop, the remote UE:

o	When RRC_CONNECTED, uses end-to-end RRC signaling to obtain its system information directly from its connected cell.

o	When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

o	When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink)

For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

o	when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

o	when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI

The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon:

•	acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE, but the last relay UE is not required to determine which node originated the request)

•	reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1

•	deciding to perform unsolicited SIB1 forwarding

An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE).

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI.

The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE.

Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE.

Agreements:

The remote UE in multi-hop:

o	When RRC_IDLE/RRC_INACTIVE, can request to receive paging by sending its paging information using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

o	When in RRC_IDLE/RRC_INACTIVE, can receive paging record from PC5-RRC signaling (e.g., UuMessageTransferSidelink)?

For a remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

o	when there is a change in the paging information while in IDLE/INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

o	when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to release the paging information

The last relay UE in multihop can forward paging to an intermediate Relay upon receiving paging message related to a multihop remote UE (including the case of an intermediate relay UE functioning as a remote UE).

Agreements:

When the intermediate UE receives a paging message from a parent relay on PC5, it forwards the paging message only to the remote UE being paged or the intermediate relay UE serving a remote UE being paged.

The PC5-RRC message containing the paging information that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least paging UE ID and paging cycle of the remote UE and any intermediate relay UEs serving the remote UE.

Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to provide the paging information.

The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE contains at least one or multiple paging record(s) associated with intermediate relay UE(s) and/or remote UE(s).

Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides paging record to the child UE(s).

Agreements:

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay), following legacy remote UE behaviour:

•	when there is a change in the SI required by the intermediate UE for its own use (as a remote UE)

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request; if a child node requested the SI, the intermediate UE acquires it by dedicated signalling)

The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

•	upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested)

•	FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.

Agreement:

If multiple remote UEs are connected to a single intermediate relay UE, the multiple remote UEs must connect through the same upstream path (same parent relay UE(s)).

Agreements:

An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage).

What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)?

•	FFS: when there is a change in the paging information of the intermediate UE or child UE

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request)

•	upon reception of paging monitoring request from a remote UE/child relay UE

•	FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node

•	Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE)

•	FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE)

Agreements:

In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header.

In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.