3GPP TSG-RAN WG2 Meeting #129bis										 				R2-2501962

Wuhan, China, 7th April. 2025 – 11th April. 2025             		      revision of R2-2500255

Agenda Item:	8.2.2

Source: 	Xiaomi

Title:  	Discussion on A-IOT paging procedure

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#129, RAN2 discussed the paging aspects, and allocated the email discussion [POST129][035][AIoT] Paging (Qualcomm)  to address the remaining paging FFSs, Following issues have been discussed in the email discussion, but based on the summary from Rapporteur, more discussions are still needed.

FFS device behaviour if multiple requests are received in parallel (if needed).

FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

FFS if there is really a need for visibility in the MAC layer

In this contribution, we continue the discussion on above open issues, and the issues related to the content of paging message and PDU design.

# 2	Discussion

## 2.1 Paging duplication detection

### 2.1.1 Device behaviour if a new service request is received while the device is waiting for the response from the reader (035)

RAN2 discussed this issue, and has agreed:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

During the email discussion, companies have different view on what UE haviour should be.

- Option A Ignore all new requests if one procedure is ongoing and not finished: Lenovo, vivo, OPPO (based on Q3), Spreadtrum/UNISOC, Tejas Networks, ZTE, MediaTek, Nokia, ETRI, NEC, Samsung, HONOR, Sharp, III, Sony, Kyocera, Fujitsu (17) - the main argument from the companies in this camp seems to be (a) this is simpler, (b) the RAN2 agreement already implies this. - Option B Terminate the ongoing procedure and respond to the latest request: Huawei/HiSilicon, CMCC, Apple, Xiaomi, Qualcomm, Futurewei, Transssion Holdings (7) - the main concern from the companies in this camp seems to be what if the reader legitimately needs to abort the ongoing service in the middle for whatever reason and start a new service? The devices may be stuck ignoring the new request while waiting for the old procedure to finish.

To our understanding, the device cannot know whether the procedure is ongoing and not finished since the device has no idea on whether it is Inventory only or Inventory+Command, and whether there are multiple commands or single command. In addition, the device has no idea on whether the reader wants to stop the procedure or whether error happens.

Observation 1: it is difficult for a device to know whether the procedure is ongoing and not finished.

The problem can only happen if multi-Readers have no coordination. As commented by companies, the worst case is that different readers send the message simultaneously, and the device cannot decode the message at all. To make it work, the coordination is needed among readers. RAN2 also agreed that we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it. Therefore, we do not see the need for the device to distinguish whether the Msg is from the same or different readers, and the reader ID is not needed in R2D Msg.

Proposal 1: RAN2 confirm that Implementation based coordination among readers is sufficient. The device does not need to distinguish whether a Msg is from same or different reader. The reader ID is not included in R2D Msg.

If implementation-based coordination among readers is supported, the issue mentioned in the email discussion is really a rare case, or intended behavior from the reader. Therefore, the device shall just follow the received R2D message, e.g., the device has sent a Msg1 to respond Paging message with transaction ID 1, and waiting for RAR msg. But it receives Paging message with transaction ID2, the device shall just send a new Msg1 if the paging Msg is for itself.

Proposal 2: The device just follows the request from the reader, and replaces the transaction ID if changed no matter whether the device is waiting for the response from the reader for previous D2R Msg.

### 2.1.2 Transaction ID (035)

[035] discussed how the transaction ID is generated. To our understanding, the only thing RAN2 needs to do is to agree the purpose of the transaction ID, i.e., avoid duplicated response from the same device for the same service and the size of it, no matter whether it is generated by CN or reader, and no matter whether it is generated based on correlation ID or not since the details shall be discussed and decided by RAN3.

Regarding the size, to our understanding, to avoid the case that a device misses paging due to charging, the repetition times should be longer enough. Longer ID size needs more resources for the transmission from the reader side and more power consumption for the reception and decoding from device side. Therefore 4 bits should be sufficient.

Proposal 3: Send LS to RAN3, CC SA2 to inform them of the need of transaction ID (avoid duplicated response from the same device for the same service) and the size (4bits), and ask them to decide how to generate the transaction ID.

### 2.1.3 Association between the paging identifier and the AS-ID (035)

[035] also proposed to discuss how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.). To our understanding, it is related how gNB and CN maintain the UE context, e.g., whether separate NGAP ID for different devices or same NGAP ID for a group of devices. It should be discussed in RAN3 instead of RAN2. RAN2 can send LS to RAN3 once the solution on AS-ID assignment (036) is clear.

Proposal 4: Send LS to RAN3, CC SA2 to inform them of agreements of AS-ID (036), and ask them to decide how to manage the UE context based on AS-ID.

### 2.1.4 Other issues

For the paging duplication times, the reader has no idea how many devices should respond the paging unless CN can provide the number of devices to be paged. Therefore, CN is in the best position to configure it, i.e., it can be controlled by CN, e.g., configure a times of paging duplication and reader can repeat the paging based on CN configured paging duplication times. If the CN consider that the paging duplication is not needed anymore, e.g., the CN can check whether it has received enough response.  If yes, the CN can ask RAN to stop the paging duplication by signaling.

Proposal 5: The paging duplication times should be configured by CN.

Proposal 6: Paging repetition can be stopped by CN via signaling to reader.

For paging triggered CF access, the target device id will be included in paging message, and the reader will know whether the target device has completed the service or not. If not, network can send paging again to trigger the device to re-access with the device ID. The transaction ID is not needed..

Proposal 7: Transaction ID is not needed for paging triggered CF access.

## 2.2 Content of A-IOT paging message

Regarding how to determine CF or CB, RAN2 asked SA2 whether the CN can indicate that the service is targeted for a single or multiple devices; However according to the reply LS from SA2, CN can provide the information to the reader if available. That means this information may not always be available.

Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available.

If the reader can get the information from the CN, i.e., single device, the reader can trigger CF access for paging, otherwise CB based access should be used.

Proposal 8: Reader can decide whether CB or CF access should be used based on the information from CN, i.e., single device ID from CN will trigger CF access, otherwise trigger CB access.

According to RAN1 agreements, the TDM/FDM resource will be used for initial access and the concerned devices can access the network on different TDM/FDM resource. It is common understanding that CB/CF access resource should be indicated to the devices before the device initial the initial access procedure. If CF access is triggered, the reader will also allocate device specific resource, e.g., 1*1 access occasion configured in paging message.  If CB access is triggered, the reader will also allocate multiple access occasion, e.g., X*Y. So, the reader can also know CB or CF access is triggered based on corresponding resource allocation.

In RAN2#127 meeting, RAN2 agreed that some information from CN is visible to the reader. The reader can decide the slot number based on the number of target devices from CN. So, the reader can indicate the slot number for access following this paging.

Agreements - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

From device perspective, the device can know the CF or CB access according to the device id related information and resource allocation for access.

Proposal 9: The device determines whether CF access or CB access according to the device ID information and corresponding resource allocation in paging message, i.e., CF or CB access indication is not included in paging message.

## 2.3 Paging PDU design

In RAN2#129 meeting, RAN2 agreed the two ways to define paging message and R2D message for MSG1 resource determination.

- Agreements - the A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

We listed the format of each option as following.

Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like;

If paging message and R2D message for MSG1 resource determination are different IOT MAC PDU, we need to introduce a new Msg type to indicate R2D message for MSG1 resource determination.

Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

If paging message and R2D message for MSG1 resource determination are same Msg. We need to introduce a bit to indicate whether it is for paging Msg or the R2D message for MSG1 resource determination.

No matter which way to go, the paging ID related fields are not needed for R2D message for MSG1 resource indication.

Proposal 10: The paging ID related fields are not included for R2D message for MSG1 resource indication.

For way-1, a new PDU should be introduced for this R2D message for resource boundary determination. Then the device decodes this message based on this PDU type indication. For way-2, an indication in paging PDU should be introduced to distinguish initial trigger message or resource boundary indication, e.g., purpose field.

In our understanding, there is no much difference. Considering that RAN2 already defined 3 R2D PDU type and 2 bits for PDU type indication. We can use it for the new Msg and do not need to introduce another bit as way 2.

- Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

Proposal 11: introduce a new R2D message other than the paging message which is used to determine the Msg1 resource(s) (Way-1).

## 2.4 Cross layer handling on Paging message

In RAN2#129 meeting, RAN2 agreed that paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Paging ID and device ID are upper layer data and should be handled in upper layer. When IOT MAC receives paging message, the IOT MAC cannot know whether the paging is for itself before checking the paging ID in upper layer. To determine whether the response paging is needed or not, the IOT MAC shall always deliver the paging ID relate fields to the upper layer.  Upper layer should inform IOT MAC if response is needed. Furthermore, the upper layer should also inform IOT MAC about the device ID for MSG3 generation.

Proposal 12: The IOT MAC should deliver paging ID to the upper layer when IOT MAC receives the paging message. The upper layer should inform IOT MAC if the response is needed and provide the device ID.

# 3	Conclusion

In this contribution, we discussed the procedures and signalling for the initial access in A-IOT and have corresponding proposals.

Paging duplication detection:

Observation 1: it is difficult for a device to know whether the procedure is ongoing and not finished.

Proposal 1: RAN2 confirm that Implementation based coordination among readers is sufficient. The device does not need to distinguish whether a Msg is from same or different reader. The reader ID is not included in R2D Msg.

Proposal 2: The device just follows the request from the reader, and replaces the transaction ID if changed no matter whether the device is waiting for the response from the reader for previous D2R Msg.

Proposal 3: Send LS to RAN3, CC SA2 to inform them of the need of transaction ID (avoid duplicated response from the same device for the same service) and the size (4bits), and ask them to decide how to generate the transaction ID.

Proposal 4: Send LS to RAN3, CC SA2 to inform them of agreements of AS-ID (036), and ask them to decide how to manage the UE context based on AS-ID.

Proposal 5: The paging duplication times should be configured by CN.

Proposal 6: Paging repetition can be stopped by CN via signaling to reader.

Proposal 7: Transaction ID is not needed for paging triggered CF access.

Content of A-IOT paging message:

Proposal 8: Reader can decide whether CB or CF access should be used based on the information from CN, i.e., single device ID from CN will trigger CF access, otherwise trigger CB access.

Proposal 9: The device determines whether CF access or CB access according to the device ID information and corresponding resource allocation in paging message, i.e., CF or CB access indication is not included in paging message.

Paging PDU design:

Proposal 10: The paging ID related fields are not included for R2D message for MSG1 resource indication.

Proposal 11: introduce a new R2D message other than the paging message which is used to determine the Msg1 resource(s) (Way-1).

Cross layer handling on Paging message:

Proposal 12: The IOT MAC should deliver paging ID to the upper layer when IOT MAC receives the paging message. The upper layer should inform IOT MAC if the response is needed and provide the device ID.

# Reference

[1] R2-2502211 Email discussion report: [POST129][035][AIoT] Paging