3GPP TSG-RAN WG2 Meeting #129bis				 					R2-2501963

Wuhan, China, 7th April. 2025 – 11th April. 2025               revision of R2-2500254

Agenda Item:	8.2.3

Source: 	Xiaomi

Title:  	Discussion on access procedure for A-IOT

Document for:	Discussion and decision

# Introduction

In RAN2#129, RAN2 discussed random access related issues. However following issues still need further discussion:

Issue 1: Determination of access slot, same as RFID or not?

Issue 2: How to handle RN16 collision?

Issue 3: Message design

Issue 4: The details of NACK

In this paper, we continue the discussion on these open issues.

# Discussion

## Issue 1: Determination of access slot, same as RFID or not?

In RFID, the tag will generate one random value based on network configured Q and the tag will decrease the slot-counter (i.e., random value based on Q) by one upon receiving QueryRep.. The tag considers it is the access slot when the slot-counter is decreased to 0000h. And if the tag failed to access, the slot-counter will be changed to 7FFFh and the tag has to wait until receiving QueryAdjust message with new Q.

In ambient IOT, the similar mechanism can be used to define “Slot”, i.e., the R2D command is used to indicated the boundary of the slot. In last RAN2 meeting, RAN2 has agreed to introduce a R2D trigger message after initial paging message which can be used for this purpose.

- Agreements - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3

Proposal 1: The R2D trigger message is used to determine the beginning of an access slot.

Regarding the random value generation, same as RFID, the paging message is used to indicate the beginning of a paging round and Q, the device shall generate random value based on network configured Q.

Proposal 2: The ambient IOT device generates random value based on Q configured in Paging message.

Regarding how to determine the access slot, RAN2 needs to discuss whether RFID slot-counter decrement mechanism should be used or not.

The simple way could be the reader can indicate the slot index directly in trigger message, e.g., if the reader wants to trigger the slot index 8, the slot index 8 can be included in trigger message, then device with slot counter 8 will initiate the access procedure. This method will not result in frequent removing/storing slot counter. We summary the Pros and Cons of the two methods (Count-down based and Index based) in the below table:

|  | Option 1: The device enters its own slot by decreasing the slot-counter upon receive QueryRep like command as RFID. | Option 2: The reader indicates the target slot index in R2D trigger message and the devices with slot-counter = slot index will initiate the CB access. |
| --- | --- | --- |
| Pros. | - The load of each R2D message is small. | - less power consumption since the device does not need to count-down. - Flexible, the reader can trigger any number in a slot based on index number |
| Cons. | - The device should receive all R2D message to maintain the slot counter decrement and it will waste more device power. - Frequent handling(store/release) of the slot counter that will lead more power consumption. - The device in a paging round only has a single chance to access the network. | - additional signaling overhead especially when the slot number is huge. The worst case is 16bits. |

Observation 1: In RFID, Slot-counter decrement mechanism results in frequent removing/storing slot counter that will increase device’s power consumption.

Observation 2: In RFID, Slot-counter decrement mechanism requires the device to receive lots of R2D command that will increase device’s power consumption.

Observation 3: In RFID, long latency for failure case, the device has to wait for QueryAdjust message, and then count-down again.

Based on above analysis, we think index-based approach is better from power consumption perspective and flexibility perspective although additional R2D overhead is required.

Proposal 3: Slot index-based access slot selection is used for A-IoT, i.e., the reader indicates the target slot index in R2D trigger message and the devices with slot-counter = slot index shall initiate the access.

## Issue 2: How to handle RN16 collision?

In email discussion [036], some companies commented the issue on RN16 collision. If device 1 and device 2 trigger CB access with same RN16 in different FDMA based access occasion in the same access slot, the reader can identify these two devices since the access occasion is different, but it is not clear how to handle this scenario, especially how to identify these two devices in subsequent R2D message (Msg 2). We think it is worth to solve the issue considering there is no ambiguous issue in Reader.

Proposal 4: RAN2 confirm the need to address the RN16 collision issue, i.e., reader shall be able to handle the case that difference devices send MSG1 with same RN16 via different access occasion.

## Issue 3: MSG2 design in CB access

In RAN1#120 meeting, RAN1 agreed that MSG2 supports both option 1 (i.e., separate MSG2) and option 2 (multiplexing MSG2).

Option 1: MSG2 corresponding to each access occasion in one slot is TDM based transmission.

Option 2: MSG2 corresponding to each access occasion in one slot are multiplexed in one MSG2 PDU.

RAN1 is discussing whether Msg2 monitoring window is needed. Given the device shall always monitor the R2D message, the failure can be determined based on the reception of sub-sequent message/signalling, e.g., paging, NACK or Command. The Msg2 monitoring window definition is not needed.

Proposal 5: Msg 2 monitoring window is not needed, no matter whether Msg2 multiplexing is supported or not.

For MSG2 multiplexing, there are 2 options to multiplex MSG2.

Option 1: Access occasion based MSG2 multiplexing.

Option 2: RN16 based MSG2 multiplexing.

For option 1, only Msg2 per access occasion in which msg1 is receiving successfully is included in the multiplexed MAC PDU. The device will determine whether the Msg2 is the response for its Msg1 based on the access occasion. RN16 is not needed.

For option 2, RN16+Msg2 are included in the multiplexed MAC PDU. The device will determine whether the Msg2 is the response for its Msg1 based on the RN16. However, it cannot solve RN16 collision issue.

Option 1 is preferred considering less overhead than option2, and can solve RN16 collision issue very well.

Proposal 6: Access occasion based Msg2 multiplexing PDU design is supported.

## Issue 4: Details of NACK

In RAN2#129 meeting, RAN2 agreed that:

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

It is unclear how to handle the retransmission with this agreements, and whether it is applied for other D2R messages. We analyze the handling of failure scenario in the below table:

| Failure Scenario | Trigger of Re-access or retransmission (Device behavior) |
| --- | --- |
| MSG1 (RN16) Failure | Re-access: no Msg2, Received Subsequent A-IoT paging message; Timer/window is not needed |
| MSG2 (RAR) Failure | Re-access: no Msg2, Received Subsequent A-IoT paging message, Timer/window is not needed |
| MSG3 (Inventory response) Failure | Msg3 Retransmission: Received Msg2 Re-Access: Received NACK; Wait for Subsequent A-IoT paging message Timer/window is not needed |
| NACK failure | Failure: Device assumes “success” |
| Command failure | None |
| Command response failure | Response Retransmission: Received Command again Re-Access: Received NACK; Wait for Subsequent A-IoT paging message Timer/window is not needed |
| Segment of Command response failure | Response Retransmission: Received Command again Segment of Response Retransmission: Received Command with received size Re-Access: Received NACK; Wait for Subsequent A-IoT paging message Timer/window is not needed |

Note 1: For CFRA, Msg1 Inventory response failure (i.e., Msg3 in CBRA) can be handled by Subsequent A-IoT paging message directly without the need of NACK

Note 2: Segment transmission should be stopped upon receiving paging

Note 3: Paging with different transaction ID will trigger the new access

Proposal 7: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for Msg1/2 failure. Timer, window and NACK are not needed for the device to determine the failure.

Proposal 8: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for Msg3 failure. Msg2 transmission is used to trigger the retransmission of Msg 3. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Proposal 9: RAN2 confirms that no special handling is needed for NACK failure although device will assume “success”.

Proposal 10: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for command response failure. Command transmission is used to trigger the retransmission of command response. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Proposal 11: For segment scenario, RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for command response failure. Command transmission is used to trigger the retransmission of command response. Command with received size will trigger the retransmission of the segment. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

To respond the failure of multiple devices, multiplexing is supported for NACK Msg.

Proposal 12: multiplexing is support for NACK message.

# Conclusions

Based on the above discussion, we propose following proposals:

Issue 1: Determination of access slot, same as RFID or not?

Proposal 1: The R2D trigger message is used to determine the beginning of an access slot.

Proposal 2: The ambient IOT device generates random value based on Q configured in Paging message.

Observation 1: In RFID, Slot-counter decrement mechanism results in frequent removing/storing slot counter that will increase device’s power consumption.

Observation 2: In RFID, Slot-counter decrement mechanism requires the device to receive lots of R2D command that will increase device’s power consumption.

Observation 3: In RFID, long latency for failure case, the device has to wait for QueryAdjust message, and then count-down again.

Proposal 3: Slot index-based access slot selection is used for A-IoT, i.e. the reader indicates the target slot index in R2D trigger message and the devices with slot-counter = slot index shall initiate the access.

Issue 2: how to handle RN16 collision?

Proposal 4: RAN2 confirm the need to address the RN16 collision issue, i.e., reader shall be able to handle the case that difference devices send MSG1 with same RN16 via different access occasion.

Issue 3: MSG2 design in CB access

Proposal 5: Msg 2 monitoring window is not needed, no matter whether Msg2 multiplexing is supported or not.

Proposal 6: Access occasion based Msg2 multiplexing PDU design is supported.

Issue 4: Details of NACK

Proposal 7: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for Msg1/2 failure. Timer, window and NACK are not needed for the device to determine the failure.

Proposal 8: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for Msg3 failure. Msg2 transmission is used to trigger the retransmission of Msg 3. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Proposal 9: RAN2 confirms that no special handling is needed for NACK failure although device will assume “success”.

Proposal 10: RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for command response failure. Command transmission is used to trigger the retransmission of command response. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Proposal 11: For segment scenario, RAN2 confirms that Subsequent A-IoT paging is used to trigger the re-access for command response failure. Command transmission is used to trigger the retransmission of command response. Command with received size will trigger the retransmission of the segment. NACK is used to determine whether re-access will be triggered. Timer, window are not needed for the device to determine the failure.

Proposal 12: multiplexing is support for NACK message.

# Reference

[1] RAN2#129 meeting agreement