3GPP TSG-RAN2#129bis 	R2-2501991

Wuhan, China, 7th Apr – 11th Apr, 2025

Agenda item:		8.18 (TEI19)

Source:	LG Electronics Inc.

Title: 	BWP restriction in MT-SDT

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed the issue that TB size of DL transmission in MT-SDT may be small due to CORESET#0 restriction in initial DL BWP and some solutions were suggested to resolve the issue.

During offline discussion, RAN2 downselected two solutions, Solution 1) removing initial BWP restriction and Solution 2) early CQI reporting and concluded to ask RAN1 for RAN1 impacts on Solution 1) and Solution 2).

However, some companies argued further downselection before asking for RAN1 impacts and RAN2 decided to postpone the issue.

In this contribution, we discuss the issue and downselect one between two solutions.

2.	Discussion

Currently, MT-SDT data is transmitted over initial DL BWP only and initial DL BWP is restricted to CORESET#0 in RRC_INACTIVE. Thus, a low number of PRBs can be allocated within CORESET#0.

Meanwhile, when the network receives a request for MT-SDT, i.e. RRCResumeRequest, the network does not know downlink channel quality, so the network may use a conservative MCS (i.e. low MCS). Due to low PRBs and low MCS, the network schedules small TB size for initial DL transmission, which result the issue of a large number of subsequent transmission.

In RAN2#128, many companies had some sympathy for this issue and RAN2 agreed to address this issue, and during offline discussion in RAN2#129, RAN2 downselected two solutions and concluded send LS t RAN1 to ask for RAN1 impacts on two solutions.

However, some companies argued further downselection before asking for RAN1 impacts, and RAN2 finally decided to postpone the issue.

We think that further downselection can do based on RAN1 reply, but if further downselection is really needed before sending LS, RAN2 needs to analyze both solutions first.

Solution 1. Removing initial BWP restriction for MT-SDT

In Solution 1, the UE performs MT-SDT on a separate BWP other than initial BWP. By removing initial BWP restriction, the network can schedule MT-SDT data via the separate BWP that is configured with CORESETs other than CORESET#0 and more number of PRBs can be allocated.

Generally, MT-SDT is considered to be performed on initial BWP only. However, strictly speaking, for RedCap UE, MT-SDT can be performed on a separate BWP other than initial BWP.

For Redcap UE, both RA based MT-SDT and CG-SDT based MT-SDT can be performed using RedCap initial BWP, i.e., initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. Although the name has the format of initial BWP, technically, RedCap initial BWP is additional BWP that is separately configured from initial BWP.

Thus, similar to RedCap, if the separate BWP is allowed for MT-SDT, MT-SDT can be performed using the separate BWP, and if the separate BWP is configured with CORESETs other than CORESET#0, and large TB size can be scheduled.

In details, for RA/CG-SDT based MT-SDT, the configuration for RA procedure and CG-SDT are configured for separate DL and UL BWP. Upon RA/CG-SDT based MT-SDT initiation, UE performs RA/CG-SDT based MT-SDT using separate DL and UL BWP.

Solution 2. Early CQI reporting with RRCResumeRequest

In Solution 2, a new MAC CE to signal channel quality measurement (e.g. RSRQ or CQI) is defined and it is transmitted with Msg3/MsgA (i.e. RRCResumeRequest). In this solution, the network can know downlink channel quality and can use a better MCS at good downlink channel quality.

However, the size of Msg3/MsgA is strictly restricted because the size of Msg3/MsgA is related to the coverage of Msg3/MsgA. Thus, if a new MAC CE introduce, the size of Msg3/MsgA should be increased and this may bring an impact on the deployment of network coverage.

In addition, when MT-SDT is initiated, the UE uses the non-SDT RACH resource. Thus, the network does not distinguish whether PRACH is transmitted for MT-SDT or legacy RRCResume procedure from PRACH alone, so the network may always allocate UL grant for Msg3/MsgA considering both RRCResumeRequest and the new MAC CE. This leads to resource waste in case of the PRACH is transmitted for legacy RRCResume procedure.

Moreover, Solution 2 may impact on RAN4 specification such as the requirement of channel quality measurement upon MT-SDT initiation, e.g., until when the UE should be capable to measure/transmit valid channel quality. Similar requirement is SCell Activation delay requirement and this is specified in RAN4 specification [1] as a time requirement within which the UE should be capable to transmit valid CSI reporting upon receiving SCell activation command.

Moreover, even if the new MAC CE solution is introduced, initial DL BWP is still restricted to CORESET#0 in RRC_INATIVE, so TB size may not be scheduled large enough.

Considering above analysis, therefore, we think that early CQI reporting solution is not desirable.

Proposal 1. In order to address the issue of small size TB in MT-SDT, RAN2 downselect the solution of removing initial BWP restriction in MT-SDT.

If initial BWP restriction is removed for MT-SDT, MO-SDT on the separate BWP can be also considered.  Both RA-SDT and CG-SDT, i.e. Rel-17 MO-SDT, are allowed for RedCap initial BWP, so we think MO-SDT can be performed on the separate BWP.

Proposal 2. RAN2 agrees to remove initial BWP restriction in MO-SDT.

One thing to be confirmed by RAN1 is whether the configuration for legacy RACH resource, RA-SDT and CG-SDT can be configured for the separate BWP. In Rel-17 MO-SDT, RAN2 agreed to introduce separate BWP for MO-SDT, but at that time, RAN1 did not reach to consensus whether CG-SDT can be configured to the separate BWP and RAN2 did not introduce the separate BWP.

However, as mentioned above, for RedCap, the configuration for RA procedure, RA-SDT and CG-SDT is already allowed to be configured for additional BWP, i.e., RedCap initial BWP. Thus, we believe that there is no big technical problem to configure the configuration to the separate BWP.

Anyway, as discussed in the last meeting, RAN2 needs send LS to check RAN1 impact on removing initial BWP restriction.

Proposal 3. RAN2 sends LS to ask RAN1 impacts on removing initial BWP restriction for MT-SDT and MO-SDT.

3.	Conclusion

In this document, we discuss the issue that that TB size of initial DL transmission is small in MT-SDT and made proposals as follows.

Proposal 1. In order to address the issue of small size TB in MT-SDT, RAN2 downselect the solution of removing initial BWP restriction in MT-SDT.

Proposal 2. RAN2 agrees to remove initial BWP restriction in MO-SDT.

Proposal 3. RAN2 sends LS to ask RAN1 impacts on removing initial BWP restriction for MT-SDT and MO-SDT.

4.	Reference

- R2-2410542	Quality Indication in Msg3 for SDT	Ericsson

- TS 38.133	Requirements for support of radio resource management	V18.8.0