3GPP TSG RAN WG2#129bis	R2-2502042

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.4

Source:	OPPO

Title:	Discussion on AIoT data transmission related functionalities

Document for:	Discussion, Agreement

Introduction

In the last RAN2#129 meeting, following agreements regarding A-IOT data transmission have been made:

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer- e.g., number of bits successfully received so far (from the start). FFS this implies that unsegmented packet can also be retransmitted. FFS if this applies to Msg3.

- R2D segmentation is not supported for R19 A-IOT.

- From RAN2 perspective, only the following types of procedures will be considered in the normative phase: “inventory only” and “inventory and command”.

- Aim to design simple MAC PDU format design

- Support multiplexing of information for multiple devices in R2D message for Msg2. FFS others for multicast messages.

- At least the following filed are required for at least for R2D in the MAC header- message type, length for SDU and variable part(s).

- FFS whether for D2R we need message type filed, any length and need for padding.

- Specify message types and contents. As starting point consider the following MAC message types

- R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

- D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not)

- R2D MAC PDU (MSG2)

- D2R MAC PDU (MSG3 and data)

- R2D MAC PDU (R2D data)

Other message types are FFS.  The message types may evolve based on functionality agreements.

In this contribution, we aim to address our view on the aspects e.g., transmission resource determination for A-IOT Msg3 and subsequent messages, segmentation for D2R, MAC PDU format, etc.

Discussion

Determination of Transmission resource for subsequent message transmission

- As the command sent to each A-IOT device can be unique, following bullets are valid:

- Some A-IOT devices may finish the communication process earlier than others, as they may no longer receive any command from the Reader.

- The D2R transmission grants issued to the A-IOT devices may be different, since the expected D2R data size is different (some devices may only need to send a confirmation of the operation ‘write’ to the Reader).

- So similar as we address in the section 2.2 in [1], although the baseline operation is to let A-IOT devices keep using the same resource indices for Msg1 transmission, adjustment of the resource index may be needed in the R2D message, since otherwise if we require one A-IOT device to look for its subPDUs whose rank in the R2D message matches its resource index, some of the subPDUs corresponding to the A-IOT devices no longer receives data from the Reader need to be reserved as padding. Also, the D2R transmission grant could be changed compared with the previous D2R transmission for each A-IOT device, and either the absolute D2R grant or the change of the D2R grant may be reflected the R2D message.

- Observation 1: as the command sent to each A-IOT device can be unique, following bullets are valid:

- Some A-IOT devices may finish the communication process earlier than others, as they may no longer receive any command from the Reader.

- The D2R transmission grants issued to the A-IOT devices may be different, since the expected D2R data size is different (some devices may only need to send a confirmation of the operation ‘write’ to the Reader).

- Proposal 1: adjustment of the resource index, and A: change of the corresponding D2R transmission grant, compared with the one used for previous D2R message transmission or B: indication of the corresponding D2R transmission grant explicitly, shall be allowed to be embedded in the A-IOT msg4 and Subsequent R2D message.

- Also, similar with the A-IOT Msg1 and Msg3 transmission, one R2D message triggering subsequent D2R transmission may not be able to trigger all A-IOT devices in the current set of access occasions to perform D2R message transmission, due to the resource constraining in frequency domain and bad timing-synchronization performance. As a result, similar with our accompany paper [1], more than one R2D messages may be needed, and resource index decrementing method shall be applied to determine whether or not the latest received R2D trigger message can trigger the A-IOT device to perform subsequent D2R message transmission. The only difference is that for subsequent D2R message transmission, the A-IOT device will not randomize the resource index but reuse the latest updated resource index Q (but Q could be adjusted as mentioned in proposal 1). In addition, since there may be more subsequent D2R messages, resource index Q shall be stored, and a copy of Q, Q’, shall be used by the resource index decrementing method.

- One example without resource index adjustment is provided in the following example figure, wherein subsequent R2D message includes a ‘read command’ for the A-IOT device with latest updated Q = 5. Firstly, a copy of Q, Q’, is initialized and to be used as the input by the resource index decrementing method. Then, suppose the first R2D transmission (trigger) message only includes two D2R transmission resource, since Q’>2, the device shall not use any of the triggered resource for D2R transmission but performs resource index decrementing method as Q’ = Q’- 2 = 3. Subsequently, the second R2D transmission (trigger) message comes, which includes three D2R transmission resource. In such case, since Q’ = 3, the A-IOT device shall perform D2R transmission in one of the triggered D2R transmission resource.

- Figure 1: illustration of how a A-IOT device sends its D2R message according to a R2D transmission (trigger) message following a subsequent R2D message (e.g., Msg4 in this case)

- As a result, following proposals have been provided by us.

- Proposal 2: resource index decrementing method shall be applied to determine whether or not the latest received R2D transmission (trigger) message can trigger the A-IOT device to perform subsequent D2R message transmission:

- A copy of resource index Q with adjustment applied, i.e., Q’ = Q, is used for the resource index decrementing to determine whether or not the latest received R2D (trigger) message can trigger the D2R message transmission, while Q is maintained in the A-IOT device for possible further D2R transmission.

- upon reception of one R2D transmission (trigger) message, A-IOT device makes the judgement whether or not such message can trigger its D2R message transmission by comparing Q’ value with the number of D2R transmission ocassions triggered by this message: if Q’ is larger than or equeal to the number of resources, such message cannot trigger the A-IOT device to perform D2R message transmission ; Otherwise, yes.

- if Q’ is larger than or equal to the number of D2R message transmission resource, calculation of Q’ = Q’ – No. D2R resources shall be made by the A-IOT device with such parameter(s) received from R2D transmission (trigger) message, wherein No. FDM and No. TDM denote the number of FDMed access ocassions and TDMed ones triggered by one particular R2D transmission (trigger) message message.

Consideration on segmentation retransmission

As mentioned in the introduction part, in the last RAN2#129 meeting, agreement on how retransmission is performed when segmentation is applied has been achieved as follows:

For segment retransmission, reader explicitly indicates an offset in the MAC layer- e.g., number of bits successfully received so far (from the start). FFS this implies that unsegmented packet can also be retransmitted. FFS if this applies to Msg3.

Regarding the first FFS, our answer is no. Previously, the reason why we approve retransmission shall be applied in the case of segmentation is that there could be many segments that have already been sent, and if, occasionally, one subsequent D2R segment is lost by the Reader, discarding all segments that have been already received by the reader successfully may lead to the result of waste of huge amount of the air-interface resource. In other words, a compromise on the retransmission when segmentation is applied was made. Regarding the unsegmented packet, in the very beginning of the SI phase, we have already made the decision to exclude RLC-like ARQ mechanism, i.e., retransmission, from using in the A-IOT system, which reduces the complexity of A-IOT devices. In our opinion, such principle should not be broken. The correct Reader behaviour should be that it will not schedule retransmission grant to the A-IOT device which sets 1-bit flag indicating that no more data will come in the D2R message, i.e., one unsegmented packet being sent.  Correspondingly, an A-IOT device which has already set 1-bit flag indicating that no more data will be transmitted in the D2R message should not re-transmit any data to the Reader, even if it receives a re-transmission grant from the Reader.

Observation 2: the reason why retransmission shall be applied in the case of segmentation is that there could be many segments that have already been sent, and if, occasionally, one subsequent D2R segment is lost by the Reader, discarding all segments that have been already received by the reader successfully leads to the result of waste of huge amount of the air-interface resource.

Proposal 3: no re-transmission on unsegmented packet should be pursed, which implies that an A-IOT device which has already set 1-bit flag indicating in the preceding D2R message that no more data will be transmitted should not retransmit any data to the Reader, even if it receives a retransmission grant from the Reader.

Regarding if re-transmission should be applied to Msg3, in our opinion, it depends on the size of the Msg3. For CFRA, since the A-IOT device/temp ID is sent in the Msg1, it is possible that a read command is sent to the A-IOT device and correspondingly, the A-IOT device may need to send a large amount of data to the Reader using Msg3. Therefore, Msg3 segmentation is needed. On the contrary, for the CBRA, A-IOT devices are only allowed to send device ID/temp ID in the A-IOT Msg3, regardless inventory or inventory+ command case. Because the data size of device ID/temp ID is limited, segmentation on Msg3 seems redundant.

Proposal 4: segmentation on Msg3 is allowed only for CFRA.

Alleviate the need of remaining D2R message size report

Although, in some cases, the CN node could indicate an expected D2R message size in approximate amount to the Reader for it to issue a proper D2R transmission grant to A-IOT devices to finish transmission of all data, still it is possible that the grant does not fulfil the need of A-IOT device, and therefore we have agreed in the last RAN2#129 meeting that a 1-bit indication is introduced to indicate whether there is more data to be transmitted towards the Reader. In our opinion, in such cases, for the convenience of the Reader to issue proper D2R transmission resource to A-IOT devices, A-IOT devices should be allowed to send remaining D2R message size report to the Reader. However, reporting the remaining data message size in the format of exact length in bits demanded by itself may occupy a lot of data, for instance, a decimal number ‘100’ is transformed to ‘1100100’ in binary format, which is neither air-interface resource efficient nor energy-friendly for A-IOT devices.

Observation 3: reporting the remaining data message size in the format of exact number of bits demanded by itself may occupy a lot of data, which is neither air-interface resource efficient nor energy-friendly for A-IOT devices.

In our opinion, it is highly likely that the difference between the expected D2R message size given by the CN node and the size of the message demanded by A-IOT devices in practice is small, so to save the air-interface resource and energy consumed by the A-IOT device, a D2R transmission grant accommodating up to the number of default amount of bits could be issued to the A-IOT device which indicates more data is to be transmitted to the Reader but remaining D2R message size report is not carried in the message. Correspondingly, a threshold equal to the default amount of the D2R transmission grant could be provided to A-IOT devices by the Reader. In such way, A-IOT devices determine whether or not to send remaining D2R message size report to the network, depending on the comparison result of the remaining data size and the set thresholds.

Observation 4: it is highly likely that the difference between the expected D2R message size given by the CN node and the size of the D2R data demanded by A-IOT devices in practice is small.

Proposal 5: an A-IOT device is required to send the remaining D2R message size report, only if the demanded message size is higher than a certain threshold set by the Reader or defined in the spec.

Proposal 6: the Reader will issue a D2R transmission grant accommodating up to the number of default amount of bits to the A-IOT device, if no remaining D2R message size report but only the flag indicting more data need to be sent to the Reader.

To further reduce size of the remaining data size report, in our opinion, quantization method should be applied. Specifically, instead of reporting the exact number of the amount of remaining data in bits to be sent, employing limited length of bitstream to represent the data size to be reported to the Reader saves a lot of air interface and A-IOT energy. For example, using 01 to represent a data demand in the range between the default amount of the D2R transmission grant and 100 bits; using 10 to represent a data demand in the range between 100 to 200 bits; using 11 to represent a data demand in the range between 200 to 300 bits data demand, and so on so forth. Similar as above, the thresholds for quantization could be configured to A-IOT devices by the Reader in the R2D message or defined in the spec.

One such example is provided in Figure 2, wherein three A-IOT devices employ only a limited size of bitstream or nothing at all for reporting remaining message size to ask for the Reader to issue additional proper D2R transmission grants to them

Figure 2: A-IOT devices only employ a limited size of bitstream or nothing at all for reporting the remaining message size to the Reader

Proposal 7: a default threshold is provided to a A-IOT device, so that the device sends the remaining message data size report to the Reader, only when data demanded by the A-IOT device is higher than the threshold.

Proposal 8: quantization method quantizing the number of bits of the remaining D2R message size to a limited set of numerical values is employed for reporting remaining message size to the Reader.

Further details of NACK based mechanism

In the last RAN2 #129 meeting, agreement on applying NACK based mechanism on D2R message transmission to determine re-access for at least msg3 was achieved, but still FFS details including whether we need a timer or explicit message and when the reader sends the feedback.

In our opinion, even for an explicit NACK-based solution, somehow, at some time moment later, the device needs to make a judgement that it has already successfully finished the D2R transmission without any R2D message being received, which is illustrated as follows.

Figure 3: illustration of how explicit NACK message-based NACK mechanism works

Otherwise, timer-based NACK mechanism with explicit ACK message used is illustrated as follows:

Figure 4: illustration of how implicit NACK mechanism works

Obviously, both option 1 and 2 can work. However, according to discussion on the last RAN2 meeting, majority of views are that the explicit ACK based solution can cause more signalling overhead than the explicit NACK based solution, therefore explicit NACK based solution as indicated in Figure 3 is suggested by us.  Regarding how to define the time duration after which the device can decide that the previous D2R message was successful, in our opinion, two options are on the table.

The first one is of course the timer-based. The problem is the bad timing synchronization performance at the A-IOT device may cause the misalignment between the device and the Reader on when the timer shall be expired. Another approach is based on the threshold of the following R2D messages. For example, after performing the D2R transmission, if the device finds that, subsequently, a certain number of R2D messages are transmitted by the reader but none of them includes the content for itself, it will make a judgement that the previous D2R message was successful.

Observation 5: generally speaking, in majority of cases, the channel status shall be good, explicit NACK based approach can save the signalling overhead.

Proposal 9: explicit NACK message-based NACK mechanism is supported.

Observation 6: the default A-IOT behaviour, i.e., considering the latest D2R message transmission was successful, shall be defined regarding the explicit NACK based solution to let the A-IOT device not to respond to a subsequent Paging message with the same transaction ID.

Proposal 10: discuss which option shall be adopted for the A-IOT device to make a judgement that if the previous D2R transmission was successful:

- Timer-based

- No content for the device is contained in subsequent certain number of following R2D messages

In addition, regardless explicit NACK message-based NACK mechanism or explicit ACK message-based mechanism, it is possible for the A-IOT device to miss the feedback sent by the Reader. In such cases, the A-IOT device will act wrongly from the Reader’s demand. We propose RAN2 to discuss how to solve this problem.

Proposal 11: RAN2 to discuss how to resolve the problem of A-IOT devices will act wrongly from the Reader’s demand, if explicit ACK/NACK message is missed by the A-IOT device.

Design of MAC PDU format

- 2.5.1 Byte-alignment and the need of padding

For the 5G NR MAC PDU, if the TB size issued by the network is larger than a UE’s need, the UE may include a MAC subPDU with padding inside it. In addition, if coincidentally, there is no more room for accommodating padding with any length, i.e., only one byte is left, a MAC subPDU with only the sub-header field set to be in 1-byte length shall be included. In addition, the case of remaining space is less than 1 byte is not considered for padding in the MAC layer but processed in the PHY layer by appending filter bits at the end of the MAC PDU. The basic principle behind is that the RLC layer always ensure the RLC PDU/MAC SDUs provided to the MAC layer are always byte-aligned, and both of the sub-headers and MAC CEs are always byte-aligned.

For the A-IOT system, to ensure a simple MAC PDU format design, the principle of byte-alignment should be followed, i.e., each element such as the common header to be mentioned later in the section 2.4.2, sub-header, SDU, MAC CE, padding, etc, should be byte-aligned. In such case, the length field in sub-header could be indicated in the unit of bytes, e.g., it is set to 1 implying the length of the SDU is 1 byte.

However, since there is no RLC layer resided in the A-IOT Au interface, rather than RLC layer, in our opinion, NAS layer may need to take the task to make sure the MAC SDU sent to the MAC layer is always byte-aligned. A related LS may need to be sent to SA2 to inform them of our request.

Observation 7:  byte-alignment principle is critical for simple MAC PDU design for example for design of padding MAC sub-PDU, and for saving the bit space for the length field in A-IOT system.

Proposal 12: for a simple MAC PDU format design, the byte-alignment principle should be followed when designing the format of each element in the 5G A-IOT system, including sub-header field, MAC CE, padding filed, etc.

Proposal 13: a LS regarding request of the NAS layer to ensure the byte-alignment of MAC SDUs sent to the MAC layer shall be sent to SA2.

- 2.5.2 the overall R2D message MAC PDU format design principle

The first thing we would like to discuss is whether or not R2D message for different A-IOT devices should be multiplexed in the same piece of R2D message, in the CBRA scenario. Similar with the reasons given in the section of 2.5 in our accompany paper [1], contents for different A-IOT devices shall be allowed to be included in the same R2D message after the Msg2 in the CBRA scenario.

Observation 8:  in the CBRA scenario, if R2D message is needed to be sent to each A-IOT device individually, then messages should be sent one-by-one in time domain. Correspondingly, each A-IOT device shall parse these messages one-by-one to see if one of the messages is destined to itself, which causes heavy burden to these A-IOT devices.

Proposal 14: allow packaging contents for different A-IOT devices in the same R2D message, besides A-IOT Msg2.

In the 5G NR MAC PDU format, Logical channel ID (LCID) in the sub-header is used to identify the message type of the corresponding SDU, MAC CE, etc. For the A-IOT system, since in the last RAN2 #129 meeting, we agree to have several message types, and different A-IOT devices shall experience exactly the same R2D message, e.g., they shall receive A-IOT paging message before making access, they shall receive A-IOT Msg4 after performing Msg3 transmission, etc. A common header field consisting of the message type shall be placed in the beginning of the R2D MAC PDU for A-IOT devices to know the purpose of received R2D messages. For AIOT devices not involved in the current access procedure, parsing such common header field only can help them skip this message content part.

Observation 9: setting a common header field could let A-IOT devices not involved in the current access procedure skip parsing the message content part.

Proposal 15: place a common header field, consisting of the message type to indicate the purpose of the message, in the beginning of the 5G A-IOT R2D MAC PDU.

Recalled In 5G NR system, instead of putting all sub-header fields in the beginning of the MAC PDU, an alterative way interleaving the sub-header fields in-between different SDUs, MAC CEs was adopted. The reasons are summarized as follows:

- Flexible assembly of the PDU: prioritizing to assembly the one with higher priorities or timing constraints. For example, a MAC CE that needs urgent attention can be placed early in the PDU, followed by its sub-header and payload, without waiting for other elements to be processed.

- Reduced Delay: delay could be introduced to have all sub-headers at the beginning, which requires the transmitter to collect all necessary information about each element (e.g., length) before constructing the header block. By interleaving headers and payloads, the transmitter can build the PDU incrementally, which is more efficient for real-time processing. In turn, the receiver can process each element sequentially without waiting for the entire header block.

- Simplified segmentation and padding:

  - Segmentation: Enables on-the-fly segmentation without restructuring the entire PDU, as each sub-header directly precedes its payload.

  - Padding: Padding can be appended at the end with a dedicated subheader, avoiding complex adjustments to a centralized header block.

For A-IOT R2D message, sub-header field is needed for each A-IOT device to at least embrace the length(s) of the variable part(s). the Pros brought by interleaving the sub-header fields in-between different SDUs, such as reduced processing delay, simplified padding, is still needed for the 5G A-IOT system, in our opinion. Also, unify between A-IOT system and 5G NR system regarding MAC PDU processing will ease the Reader, likely to be a gNB, implementation complexity.

As a result, we propose that similar with 5G NR system, MAC subPDU destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order

Proposal 16: MAC subPDUs destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order in the 5G A-IOT R2D MAC PDU.

Putting Proposals 16 and 17 together, the MAC PDU format for multiplexing the MAC SDUs for different A-IOT devices in the same R2D message is indicated as follows:

Figure 3:  the suggested MAC PDU format for the R2D message multiplexing the MAC SDUs for different A-IOT devices in the process.

Regarding the length field, it has been agreed to be introduced for SDU and variable parts in the last RAN2#129 meeting. In our opinion, with the aim of designing simple MAC PDU format, the number of variable part(s) should be reduced as much as possible. In such way, we can alleviate the need of length fields.

For instance, a R2D command message may consist of AS ID and the data to be wrote by targeted A-IOT devices in their memory. On the top of variable bit stream of the data, if AS ID is also a variable-length bit-stream, then we will end up with using two length fields to define the length of the AS ID and data field, respectively, which will lead to large signalling overhead and additional burden of A-IOT devices to monitor and decode the R2D message. As a result, we propose RAN2 to employ fixed length of bitstream to represent parameters to be included in the A-IOT R2D message, e.g., No. of total access occasions, No. of FDM for D2R transmission, AS ID, etc.

Observation 10: the number of variable part(s) included in the R2D message MAC PDU should be reduced as much as possible. In such way, the need of length fields can be alleviated, which also reduces the decoding complexity for A-IOT devices.

Proposal 17: employing fixed number of bitstream to represent variable parts, e.g., number of total access occasions, number of FDM for D2R transmission, AS ID, etc, for all kinds of R2D message, as much as possible.

2.4.2.1 sub-header format

The sub-header of MAC subPDU format of 5G NR is provided as follows:

- Sub-header for Fixed-Size MAC Control Elements (CEs), Padding, and a MAC SDU containing UL CCCH :

  - Format: 1 octet (8 bits)

  - Fields:

    - R (Reserved): 2 bits (set to 0)

    - LCID (Logical Channel ID): 6 bits (identifies the CE type or padding)

- Variable-Size Subheader: used for variable-length elements such as MAC SDUs and some MAC CEs with variable payloads (e.g., Long BSR):

- Format: 3 octets (24 bits)

- Fields:

    - R (Reserved): 2 bits (set to 0)

    - F: indicates the size of the Length field; the value 0/1 indicates 8/16 bits of the Length field。

    - LCID (Logical Channel ID): 6 bits. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field; if the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field.

    - eLCID: the extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

    - Length (L): 8/16 bits (specifies the payload size in bytes).

For the Ambient IOT R2D message, in the last RAN2 meeting, we have achieved the agreement that at least the following field are required for at least for R2D in the MAC header – message type, length for SDU and variable part(s). For message type, we have already proposed to indicate in the common header for the R2D message. So, we would like to discuss the other potential fields one by one.

- Length: as mentioned in section 2.4.1, byte-alignment of payload, e.g., MAC SubSDU and other variable part(s) included in the MAC PDU for 5G A-IOT system is proposed by us, so the usage of length field should be similar with the current 5G NR, i.e., specifying the payload size in bytes. Regarding the bit-stream length of the length field, 8 bit can indicate SDU length varies in the range of from 0 to 2^8*8 = 2048 bit. For a typical MAC SDU addressed to single A-IOT device, such implementation, from our perspective of view, is enough.

- F: since the length field is fixed, F shall be not included in the R2D messages.

- LCID: the detailed command type is included in the upper layer data packet, so this field is not needed.

- ACK/NACK to the previous transmitted D2R message for performing re-access: this is needed to identify if the R2D message sent to one particular A-IOT device is to confirm the success/unsuccess of the previous D2R message to perform re-access. Only 1 bit is enough, setting to 1 is to indicate ACK/NACK (pending on the further agreement) and setting to 0 means ACK/NACK feedback is disabled.

- AS ID: if AS IDs of targeted A-IOT devices are agreed to be included in the multiplexed R2D message addressed to them, we suggest AS ID to be included in the sub-header field, since each R2D message shall include the information.

- R: could be included for future-proof

- Segment retransmission Offset indicator: this reflects the last RAN2 meeting agreement that network instructs to the A-IOT device regarding from which position in the D2R message the device shall perform transmission in the case that segmentation is applied. For example, such indicator is set to 1 if segment retransmission offset is sent to the A-IOT device, otherwise set to 0.

Proposal 18: following fields are considered to be included in the sub-header fields for the R2D message in A-IOT system:

- the Length field for SDU:  set to 1 byte

- the ACK/NACK to the previous transmitted D2R message for triggering re-access (optional): wherein the 1-bit flag being setting to 1 is to indicate ACK/NACK (pending on the further agreement) and setting to 0 means ACK/NACK feedback is disabled. For A-IOT paging, such field shall be ignored.

- AS IDs for the targeted A-IOT devices are included in the R2D message except the A-IOT paging message, if AS ID of the targeted A-IOT device is agreed to be included in the R2D message addressed to them.

- the Reserved field for future-proof purpose.

- Segment retransmission enabling indicator

    - the overall D2R message MAC PDU design principle

In our opinion, since only 1 command is included in one R2D message sent to a A-IOT device, only one response is sent to the Reader in the subsequent D2R message. As a result, concatenation of MAC subPDUs for different command is not needed.

Proposal 19: confirm that concatenation of MAC subPDUs including the response for different commands is not needed for the D2R message.

Next, regarding the fields to be included in the sub-header, we discuss them one-by-one, as follows:

- Length: since MAC subSDU is a variable part, so the length field is needed in the sub-header field also. For D2R direction, 8-bit length indication could be enough since a typical D2R message size is less than 1000 bit, which is only one half of 2048 bit, the longest length of bitstream can be represented by the 8-bit length indication. As a result, from our point of view, 8-bit length indication is enough.

- Format (F) field: regarding Format (F) field, since the suggested length field is static, ‘F’ shall not be included in D2R message sub header field.

- LCID/ eLCID: the message type indication is not needed, since the Reader knows exactly which message is scheduled by itself

- AS ID: if the AS ID of sourcing A-IOT device is agreed to be included in the D2R message for the Reader to identify the A-IOT device, especially for CBRA, we suggest AS ID to be included in the sub header field of D2R message except A-IOT Msg1.

- padding indication: since the Reader may issue a D2R transmission grant more than a device’s need, then the device needs to append the padding field at the end of the MAC subPDU. Correspondingly, for the Reader to identify if some bits are really a content or a padding, a padding indication is required to mark the presence of padding bits. Similar with 5G NR, for convenience of decoding/coding, the padding bits are suggested to be place in the end of the A-IOT D2R MAC PDU.

- Segmentation indication: since in the last RAN2 meeting, we have already agreed to introduce a 1-bit indication to indicate whether there is more data or not for the D2R transmission, a related flag is needed to be included in the sub header for the MAC SDU.

- Remaining data size report indication: such indication shall be included when only a segment of a piece of D2R message is sent to the Reader, and there is a need to report the remaining data size to the network for subsequent R2D scheduling

Proposal 20: following fields are considered to be included in the sub-header fields for the D2R message in A-IOT system:

- length field: this field indicates the length of the MAC subSDU in bytes

- AS ID:  if the AS ID of sourcing A-IOT device is agreed to be included in the D2R message for the Reader to identify the A-IOT device, especially for CBRA.

- Padding indication: marking the presence of padding bits

- Indication of the segmentation, i.e., whether there is more data or not for the D2R transmission is needed.

- Remaining data size report indication: when there is a need to report the remaining data size to the network for subsequent R2D scheduling.

- Conclusion

We have the following proposals:

Determination of Transmission resource for subsequent message transmission:

- Observation 1: as the command sent to each A-IOT device can be unique, following bullets are valid:

- Some A-IOT devices may finish the communication process earlier than others, as they may no longer receive any command from the Reader.

- The D2R transmission grants issued to the A-IOT devices may be different, since the expected D2R data size is different (some devices may only need to send a confirmation of the operation ‘write’ to the Reader).

- Proposal 1: adjustment of the resource index, and A: change of the corresponding D2R transmission grant, compared with the one used for previous D2R message transmission or B: indication of the corresponding D2R transmission grant explicitly, shall be allowed to be embedded in the A-IOT Msg4 and subsequent R2D message.

- Proposal 2: resource index decrementing method shall be applied to determine whether or not the latest received R2D transmission (trigger) message can trigger the A-IOT device to perform subsequent D2R message transmission:

- A copy of resource index Q with adjustment applied, i.e., Q’ = Q, is used for the resource index decrementing to determine whether or not the latest received R2D (trigger) message can trigger the D2R message transmission, while Q is maintained in the A-IOT device for possible further D2R transmission.

- upon reception of one R2D transmission (trigger) message, A-IOT device makes the judgement whether or not such message can trigger its D2R message transmission by comparing Q’ value with the number of D2R transmission ocassions triggered by this message: if Q’ is larger than or equeal to the number of resources, such message cannot trigger the A-IOT device to perform D2R message transmission ; Otherwise, yes.

- if Q’ is larger than or equal to the number of D2R message transmission resource, calculation of Q’ = Q’ – No. D2R resources shall be made by the A-IOT device with such parameter(s) received from R2D transmission (trigger) message, wherein No. FDM and No. TDM denote the number of FDMed access ocassions and TDMed ones triggered by one particular R2D transmission (trigger) message message.

Consideration on segmentation retransmission:

Observation 2: the reason why retransmission shall be applied in the case of segmentation is that there could be many segments that have already been sent, and if, occasionally, one subsequent D2R segment is lost by the Reader, discarding all segments that have been already received by the reader successfully leads to the result of waste of huge amount of the air-interface resource.

Proposal 3: no re-transmission on unsegmented packet should be pursed, which implies that an A-IOT device which has already set 1-bit flag indicating in the preceding D2R message that no more data will be transmitted should not retransmit any data to the Reader, even if it receives a retransmission grant from the Reader.

Proposal 4: segmentation on Msg3 is allowed only for CFRA.

Alleviate the need of remaining D2R message size report:

Observation 3: reporting the remaining data message size in the format of exact number of bits demanded by itself may occupy a lot of data, which is neither air-interface resource efficient nor energy-friendly for A-IOT devices.

Observation 4: it is highly likely that the difference between the expected D2R message size given by the CN node and the size of the D2R data demanded by A-IOT devices in practice is small.

Proposal 5: an A-IOT device is required to send the remaining D2R message size report, only if the demanded message size is higher than a certain threshold set by the Reader or defined in the spec.

Proposal 6: the Reader will issue a D2R transmission grant accommodating up to the number of default amount of bits to the A-IOT device, if no remaining D2R message size report but only the flag indicting more data need to be sent to the Reader.

Proposal 7: a default threshold is provided to a A-IOT device, so that the device sends the remaining message data size report to the Reader, only when data demanded by the A-IOT device is higher than the threshold.

Proposal 8: quantization method quantizing the number of bits of the remaining D2R message size to a limited set of numerical values is employed for reporting remaining message size to the Reader.

Further details of NACK based mechanism:

Observation 5: generally speaking, in majority of cases, the channel status shall be good, explicit NACK based approach can save the signalling overhead.

Proposal 9: explicit NACK message-based NACK mechanism is supported.

Observation 6: the default A-IOT behaviour, i.e., considering the latest D2R message transmission was successful, shall be defined regarding the explicit NACK based solution to let the A-IOT device not to respond to a subsequent Paging message with the same transaction ID.

Proposal 10: discuss which option shall be adopted for the A-IOT device to make a judgement that if the previous D2R transmission was successful:

- Timer-based

- No content for the device is contained in subsequent certain number of following R2D messages

Proposal 11: RAN2 to discuss how to resolve the problem of A-IOT devices will act wrongly from the Reader’s demand, if explicit ACK/NACK message is missed by the A-IOT device.

Design of MAC PDU format

Observation 7:  byte-alignment principle is critical for simple MAC PDU design for example for design of padding MAC sub-PDU, and for saving the bit space for the length field in A-IOT system.

Proposal 12: for a simple MAC PDU format design, the byte-alignment principle should be followed when designing the format of each element in the 5G A-IOT system, including sub-header field, MAC CE, padding filed, etc.

Proposal 13: a LS regarding request of the NAS layer to ensure the byte-alignment of MAC SDUs sent to the MAC layer shall be sent to SA2.

Observation 8:  in the CBRA scenario, if R2D message is needed to be sent to each A-IOT device individually, then messages should be sent one-by-one in time domain. Correspondingly, each A-IOT device shall parse these messages one-by-one to see if one of the messages is destined to itself, which causes heavy burden to these A-IOT devices.

Proposal 14: allow packaging contents for different A-IOT devices in the same R2D message, besides A-IOT Msg2.

Observation 9: setting a common header field could let A-IOT devices not involved in the current access procedure skip parsing the message content part.

Proposal 15: place a common header field, consisting of the message type, to indicate the purpose of the message, in the beginning of the 5G A-IOT R2D MAC PDU.

Proposal 16: MAC subPDUs destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order in the 5G A-IOT R2D MAC PDU.

Observation 10: the number of variable part(s) included in the R2D message MAC PDU should be reduced as much as possible. In such way, the need of length fields can be alleviated, which also reduces the decoding complexity for A-IOT devices.

Proposal 17: employing fixed number of bitstream to represent variable parts, e.g., number of total access occasions, number of FDM for D2R transmission, AS ID, etc, for all kinds of R2D message, as much as possible.

Proposal 18: following fields are considered to be included in the sub-header fields for the R2D message in A-IOT system:

- the Length field for SDU:  set to 1 byte

- the ACK/NACK to the previous transmitted D2R message for triggering re-access (optional): wherein the 1-bit flag being setting to 1 is to indicate ACK/NACK (pending on the further agreement) and setting to 0 means ACK/NACK feedback is disabled. For A-IOT paging, such field shall be ignored.

- AS IDs for the targeted A-IOT devices are included in the R2D message except the A-IOT paging message, if AS ID of the targeted A-IOT device is agreed to be included in the R2D message addressed to them.

- the Reserved field for future-proof purpose.

- Segment retransmission enabling indicator

Proposal 19: confirm that concatenation of MAC subPDUs including the response for different commands is not needed for the D2R message.

Proposal 20: following fields are considered to be included in the sub-header fields for the D2R message in A-IOT system:

- length field: this field indicates the length of the MAC subSDU in bytes

- AS ID:  if the AS ID of sourcing A-IOT device is agreed to be included in the D2R message for the Reader to identify the A-IOT device, especially for CBRA.

- Padding indication: marking the presence of padding bits

- Indication of the segmentation, i.e., whether there is more data or not for the D2R transmission is needed.

- Remaining data size report indication: when there is a need to report the remaining data size to the network for subsequent R2D scheduling.

Reference:

[1] R2-2502043, Discussion on random access for A-IOT, OPPO