3GPP TSG-RAN WG2 Meeting #129bis	R2-2502053

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on RAN2 impacts for the support of S&F operation

Document for: 	Discussion and Decision

Introduction

Based on progress of RAN2 [1] and SA2, this contribution discusses the leftover issues on S&F operation, including the mode transition time, MME-configured satellite list and other aspects.

Discussion on MME-configured satellite list: AS aspects

SA2 replied our LS [2] in [3] and approved the CR [4]. Based on SA2’s progress, this section further discuss the possible RAN2 impact due to the MME-configured satellite list.

2.1.1		Cell (re)selection enhancement

As illustrated by the coversheet and changes of the agreed CR [4], SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling. According to the latest TS 23.401 [5] which already implemented the agreed SA2 CR, although it says that how the UE behaves upon reception of this MME-configured satellite list, it was also captured that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Citation from coversheet part [4] Reason for change:; As described in the discussion paper (S2-2500780), this contribution removes this EN: Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources. While how UE behaves when receiving the satellite list from MME is UE implementation. It is difficult to define the UE behaviour to only follow the received satellite list to access the network. This contribution proposes MME sends S&F Monitoring list to UE and UE behaviour is based on UE implementation and it may use it to determine which satellites to access. For example, if the UE determines a satellite is operating in S&F Mode, the UE may follow the S&F Monitoring List to ensure a successful access; if the UE determines a satellite is not operating in S&F Mode, the UE can access to this satellite even it is not in the S&F Monitoring List..

Citation from TS 23.401, 4.13.9.1 [5] […] - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. […]

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, there is some room left for the UE to decide which S&F satellite to access and it is specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

However, based on the current cell (re)selection mechanism in [6], the UE shall select the cell relying on frequency priority and RSRP/RSRQ, as specified in the following excerption from. This means that UE has to camp/reselect to the corresponding cells meeting the RSRP/RSRQ conditions, irrespective of whether the cells are provided by satellites included in the MME-configured satellite list. That is to say, current cell reselection procedure leaves no such room for UE implementation to further consider the MME-configured satellite list as specified in SA2 procedure. The consequence, as what is also highlighted above in TS 23.401, is the increasing chance for UE access failure, if the current cell reselection procedure is followed without considering the MME-configured satellite list.

#### 5.2.4 Cell Reselection evaluation process ##### 5.2.4.5 E-UTRAN Inter-frequency and inter-RAT Cell Reselection criteria [unrelated part is omitted] In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in clause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows: - If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6; - If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempted to a satellite not in the MME-configured list).

To align with the design of SA2 Spec, we propose to leave some room for the UE to consider the MME-configured satellite list during cell reselection procedure. Specifically, during cell reselection procedure, the UE may prioritize the frequency(ies)/cell(s) corresponding to the satellites that are included in the MME-configured satellite list. With the "may", whether/how the UE prioritizes the frequency(ies)/cell(s) belonging to the MME-configured satellites are still left to UE implementation, and this is also in line with the SA2 Spec that leaves how the UE behaves upon reception of the MME-configured satellite list to UE implementation.

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list.

To support Proposal 1, assistance information which provides the frequency list and/or cell list associated with each satellite ID (indicated by MME) can be provided to the UE in system information.

Proposal 1a: A frequency list and/or cell list associated with each satellite ID (indicated by the MME) are provided to the UE in system information.

2.1.2	Relaxation on IDLE mode tasks

Besides selecting/accessing to the proper satellites, another intention of SA2 to introduce this MME-configured satellite list is for UE power saving purpose, as seen above in the approved CR cover page [4]:

The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources.

If there is an intention to further pursue this power-saving intention, we may take the discontinuous coverage design as a reference. Considering that the MME-configured satellites are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage), if the UE start/stop idle mode tasks upon the indicated satellite coming and leaving, it is beneficial for UE power saving. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by MME, the UE operating in S&F mode may relax its IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.). How the UE determines whether it is in coverage or out of coverage of the indicated satellite(s) using satellite assistance information can be left up to UE implementation (similar to legacy discontinuous coverage mechinism).

Note that our intention is to allow UE some flexibility to relax IDLE mode tasks for power saving (i.e. using "the UE may relax"), but not to prohibit the UE from doing these tasks when there is no coverage of satellite configured by MME. Then whether the UE really do this relaxation is eventually still left to UE implementation, and this can address companies' concern whether UE will miss the satellite outside the MME-configured list which is still allowed for UE's access: if the UE cannot accept such missing, it simply does not do the relaxation. Anyway, nothing is mandated, but everything is eventually up to UE implementation, which is in line with SA2 design.

Proposal 2: RAN2 discusses whether the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.), if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines out of coverage of all target satellites is up to UE implementation.

In case Proposal 2 is pursued, some more assistance information related to the satellites in the MME-configured list needs to be provided by the system information for this power-saving operation. We can also take information in SIB32 introduced for the discontinuous coverage as a reference. In Rel-17, SIB32 was introduced to predict of discontinuous coverage, offering ephemeris info and footprint info/start serving time [7]. Since the satellites indicated by the MME to provide the subsequent services may not be the next ones providing the same coverage as the current satellite, the satellite assistance information needed for S&F operation is logically similar to that used for discontinuous coverage, i.e., footprint info/start serving time is provided and the ephemeris should be long-term valid format (TLE set format), so that similar signalling format can be reused for the UE to determine the coverage of the satellites provided by the MME-configured satellite list.

SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } }

Observation 4: To support the power saving operation for S&F operation in Proposal 2, satellite assistance information similar to that used for discontinuous coverage (i.e., footprint info/start serving time and the long-term valid ephemeris format (TLE set format)) needs to be provided by system information.

However, it is better to introduce a separate list specifically for this S&F power saving purpose, instead of reusing the existing list in SIB32 directly. This is to avoid confusion to the UEs whether, if the existing satellite list in SIB32 is reused, the list serves the discontinuous coverage purpose, the S&F power saving purpose or both purposes, because such confusion led by reusing the existing satellite list in SIB32 will cause unexpected UE behaviour.

Hence, if Proposal 2 is pursued, we propose to introduce a separate list to indicate the assistance information of the satellite(s) indicated by MME for S&F power saving purpose. But the format of SatelliteInfo-r17 in SIB32 can be taken as the baseline for the signalling design.

Proposal 2a: If Proposal 2 is agreed, introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Proposal 2b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

Discussion on transition time

- S&F mode  normal mode transition time

In RAN2#128 [8], RAN2 agreed to introduce time information to indicate mode transition from S&F mode to normal mode. The transition time can be used to delay some NAS procedure until the feeder link is resumed.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

In RAN2#129 [1], RAN2 had a further discussion on the signalling format of this transition time and achieved the following agreements.

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

RAN2 can further discuss how the UE uses the S&F mode to normal mode transition time. As per the agreement of RAN2#128, the transition time can delay NAS procedure for UE operating in S&F mode. There were also some contributions proposing to use this transition time information to determine whether to initiate the service immediately or postpone the initiation a bit. Irrespective of how it is used, i.e. impacting NAS procedure or service initiation, this transition time is used to impact the NAS layer/upper layer behaviour of the UE. The only impact on AS layer is to forward the time information to upper layer when received. Whether/how the time information is used by upper layers is up to CT1.

Proposal 3: The UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. No other AS impact is expected.

Note that to delay the NAS procedure and/or service initiation was an assumption made by RAN2 ourselves. However, finally how/whether it is really used in NAS and captured in the Spec needs to be determined by CT1. Therefore, LS needs to be sent to CT1, to inform them of RAN2 conclusion/assumption, and ask CT1 to work on the related Spec change whenever necessary.

Proposal 4: Send LS to CT1, informing CT1 of RAN2 agreements related to the "S&F operation mode" to "normal mode" transition time. Take the draft LS in Annex as baseline.

- Normal mode  S&F mode transition time

There were also proposals to introduce time information to indicate when the current satellite operation mode will transit from normal operation to S&F operation. At present, we do not identify the need for the transition time in this direction. The RRC_CONNECTED UE is under NW control, and can be handed over to another cell or be released to RRC_IDLE, in case this transition happens. The RRC_IDLE UE which is camping on the cell operating in normal mode (no matter whether it support S&F mode or not) can start perform measurements for cell reselection before t-Service, and in this case it seems better for the UE to reselect to a cell in normal mode (if any). So we fail to find any benefit/necessity to introduce such transition time information from normal mode to S&F mode, and tend to think an additional transition time is not needed.

Proposal 5: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

Discussion on new release cause

Another FFS is whether a new release cause is needed when the NW releases UE to RRC_IDLE.

Proposal 8 When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE with full S&F support. FFS whether a new release cause is needed. - ZTE is fine with this and thinks we could have a new case - Ericsson wonders what is meant by suspend RRC - vivo thinks there is no need to suspend RRC - CATT agrees on this under the assumption that we are not introducing a new RRC state. - Samsung thinks this is not needed It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

When the feeder link becomes unavailable, the satellite transits from normal mode to S&F mode. Under this situation, it is up to the NW to hand over the RRC_CONNECTED UE to another cell or release the UE to RRC_IDLE. If the NW chooses to release the UE to RRC_IDLE, the cell (re-)selection procedure can be triggered as in legacy for the UE to choose one cell to camp on. Since the UE is aware of the current operating mode of the selected cell, its NAS can determine how to trigger the following NAS procedure and behaves properly. Hence the UE does not need to know the specific release cause that feeder-link becomes unavailable.

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the feeder link becomes unavailable.

For the case that service link of an S&F mode cell becomes unavailable, when the NW releases the UE to RRC_IDLE, the UE operating in S&F may need to hold the NAS procedure and context. For example, under the architecture of split MME, the UE needs to hold the process status during the attach procedure, e.g., wait timer and assistance info of satellite ID list. However, such UE NAS behaviour has already been designed by SA2 with necessary indication from MME (as shown below), so a new RRC release cause is not needed. The NW can release the RRC connection of UE with RRC release cause ‘other’. After the UE AS forwards the RRC release cause ‘other’ to NAS layer, it is up to NAS layer to keep the NAS procedure and context.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

Proposal 7: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

Conclusion

In this contribution, some leftover issues on support of S&F operation are discussed with corresponding proposals listed as follows:

On MME-configured satellite list: Cell (re)selection enhancement

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, there is some room left for the UE to decide which S&F satellite to access and it is specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempted to a satellite not in the MME-configured list).

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list.

Proposal 1a: A frequency list and/or cell list associated with each satellite ID (indicated by the MME) are provided to the UE in system information.

On MME-configured satellite list: Relaxation on IDLE mode tasks

Proposal 2: RAN2 discusses whether the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.), if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines out of coverage of all target satellites is up to UE implementation.

Observation 4: To support the power saving operation for S&F operation in Proposal 2, satellite assistance information similar to that used for discontinuous coverage (i.e., footprint info/start serving time and the long-term valid ephemeris format (TLE set format)) needs to be provided by system information.

Proposal 2a: If Proposal 2 is agreed, introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Proposal 2b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

On transition time

Proposal 3: The UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. No other AS impact is expected.

Proposal 4: Send LS to CT1, informing CT1 of RAN2 agreements related to the "S&F operation mode" to "normal mode" transition time. Take the draft LS in Annex as baseline.

Proposal 5: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

On new release cause

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the feeder link becomes unavailable.

Proposal 7: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

Reference

- RAN2#129 ChairNotes.

- R2-2411197	LS on Satellite IDs for store-and-forward operation

- S2-2502450	Reply LS on Satellite IDs for store-and-forward operation

- S2-2502513	Support of S&F Monitoring List in S&F Mode

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.2.0)

- 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

- RAN2#128 ChairNotes.

- Annex: Draft LS to CT1

Title:	LS on S&F mode to normal mode transition time

Response to:	-

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN2

To:	CT1

Cc:	-

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN2 discussed the scenario that an S&F-capable satellite transit from "S&F operation mode" to "normal mode", and achieved the following agreements.

RAN2#128

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

RAN2#129

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

RAN2 agreed that upon reception of this transition time from "S&F operation mode" to "normal mode" from SIB31 UE AS indicates this transition time to the upper layers. It is up to upper layers how to use this time information (e.g. delaying NAS procedure, delaying service initiation, etc.)

# 2	Actions

To CT1

ACTION: 	RAN2 respectfully asks CT1 to take above information into account and provide feedback (if any concern)

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #130	2025-05-19 - 2025-05-23 	Malta, MT

TSG RAN WG2 Meeting #131	2025-08-25 - 2025-08-29 	India, IN