3GPP TSG-RAN WG2 #129bis 	R2-2502501

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	8.7.5  RLC AM Enhancements

Source:	NEC

Title:	Remaining details on RLC AM enhancements

Document for:	Discussion, Decision

# 1.Introduction

Main aspects of RLC AM enhancement has been concluded but there are some specification details requiring clarification. In this document, we discuss following remaining details:

Unnecessary retransmissions avoidance

- Whether Tx window should move forward considering discarded in addition to ACKed RLC SDUs

- Discard the outdated PDUs even if its transmission has been attempted, for specification clarity

- New trigger of RX to TX STATUS Report

- independent or joint capability and configuration for DL and UL

Autonomous retransmissions and polling enhancements

- Whether to use new RLC timer or PDCP discard timer

- Extra Auto-retransmission conditions

- RETX-COUNT and RLF

- Efficient polling

# 2 Avoid unnecessary retransmissions

## 2.0 Tx window operation

If the transmitting side of an AM RLC entity discards an RLC SDU, as captured in running CR as follows, this RLC SDU will not be considered for retransmission further. However the discarded RLC SDU is not taken into account to advance the TX window:

### 5.2.3 AM data transfer #### 5.2.3.1 Transmit operations ##### 5.2.3.1.1 General /////////////////////////////////////////////////// unrelevant part is skipted///////////////////////////////////////////////////////////////////////////////// When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet. If stopReTxObsoleteSDU is set to enabled, when receiving a discard indication for an RLC SDU from the upper layer (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment for transmission or retransmission.

The lower edge of the AM RLC TX window holds the smallest SN value whose successful delivery has not been confirmed/finished, and it is supposed to wait for either retransmission or a positive acknowledgement. The lower edge also determine the biggest SN value TX_Next_Ack + AM_Window_Size, which Tx side can use to send new RLC SDU.

With current running CR, even though the discarded RLC SDUs are not waiting for retransmission and also not care about to receive a negative acknowledgement, but it will still likely hold the lower edge of TX window back, and potentially cause the Tx window stalling. Even though we understand, based on our discussion in next section, that eventually RX side will send a positive ACK to TX side for the discarded RLC SDUs and TX window will move forward, but it does not need to. Specifically, the RLC entity can move the TX window forward considering both ACKed and discarded RLC SDUs. Here is an example TP:

When receiving a positive acknowledgement for an RLC SDU with SN = x or if stopReTxObsoleteSDU is configured and discard indication for an RLC SDU with SN = x has been received from upper layer, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU in case of a positive acknowledgement is received; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet and has not been discarded.

Proposal 0: The transmitting side of an AM RLC entity update the value of TX_Next_Ack to advance the transmitting window when discarding an RLC SDU.

## 2.1 Discard the outdated PDUs

Because legacy RLC AM never give up retransmission unless it is ACKed or RLF is triggered, as shown below, when discard indication of a SDU is received from PDCP, the transmitting side cannot discard the SDU if its transmission has been attempted. However, if skip retransmission for outdated SDU is enable, this condition shall be lifted.

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall: If [x] is not configured: - discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU. Else if [x] is configured - discard the indicated RLC SDU (consequently, this RLC SDU and any segment thereof are not pending for initial transmission or retransmission)

Once these outdated PDUs are discarded, they will not be considered for transmission/retransmission, polling, data volume calculation for BSR/DSR report. Otherwise, many places in specification have to be updated with exceptional sentence, one example is as follows:

## 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

With the understanding that RLC AM will not retransmit the discarded/outdated RLC SDU, These are not SDUs pending for retransmission. They will not be taken into account for data volume calculation, polling, retransmission etc.

Proposal 1: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU without exception, consequently outdated SDU is discarded, not pending for retransmission, and not for triggering polling etc.

## 2.2 RX to TX feedback

It is agreed “The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report” .We look into the existing specification, A STATUS PDU is constructed as shown in below:

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

As above, in a STATUS PDU, Rx side will list all RLC SDUs which has not been completely received, start from RX-NEXT till ACK_SN.

When skip retransmissions for outdated PDU is configured, upon RLC SDUs become outdated/abandoned, the variable RX-NEXT which indicates the lower edge of the receiving window shall be updated to a higher value (i.e., to advance the RX window) than the SN of the outdated SDUs. Hence the outdated RLC SDUs will be moved out of the Rx window and not included in STATUS PDU, which will be interpreted as ACKed at Tx side.

However, similar to t-reassembly timer expires case, to ensure more timely synchronization of the Tx and Rx windows, a new trigger for STATUS report could be introduced upon the new timer (t-RxDiscard) expires.

Proposal 2: RAN2 confirm that STATUS PDU will ACK abandoned SDUs, and consider additional STATUS Report trigger upon t-RxDiscard expires.

## 2.3 Capability and configuration

It is obvious this feature is only for RLC AM DRB with very short latency requirements, and a UE usually can be configured with multiple DRBs. So the feature should be enabled per AM DRB under NW control.

One question is whether we should report and enable this feature in DL and UL independently or jointly? With independent way:

- In DL direction, UE first reports if it supports this feature in DL direction, NW then configures the new timer for Rx operation at UE. This will enable the feature in DL direction.

- In UL direction, UE first reports if it supports this feature in UL direction. Because there is neither new timer nor new threshold configuration is needed, as captured in running CRs, a enable bit for UL direction is added.

If joint enabling is agreed, there is no need to have enable bit. Presence of the new timer would enable the feature in both DL and UL direction, and also one capability bit would be enough.

Notice that RLC AM entity is bi-direction entity, but the configuration of UL and DL direction are independently currently. So independent way is align with other configurations, while joint way will be simpler. The question would be: is it possible to enable this feature in one direction but not in another direction for the same AM DRB? A step further into essence, is it a possible case that the short delay requirement outweighs lossless transmission in one direction, while lossless transmission outweighs short delay requirement in another direction for the same AM DRB?

Proposal 3: RAN2 discuss if the feature “skip outdated/abandoned SDU” should be supported by UE/enabled by NW jointly or independently for DL and UL directions:

- If go with independent way: a enable bit is needed for UL direction, two capability bits are needed

- If go with joint way: enable bit is not needed, presence of new timer configuration will implicitly enable the feature in both DL and UL directions and one capability bit is enough

# 3 Timely retransmissions

To achieve timely RLC retransmission for XR, RAN2 agreed to have both autonomous retransmission and polling enhancement and NW can configure either or both of them.

## 3.1 New RLC timer or PDCP discardTimer

For both solutions, it is questioned whether remaining time is determined based on discardTimer at PDCP or a new timer at RLC. The main argument to have RLC layer new timer is to reduce the interaction between PDCP layer and RLC layer. We do agree the motivation.

However, unfortunately we have already used PDCP layer discard timer for other functions like delay-critical data calculation, SDU discarding and terminating out-dated data transmission. Besides, if we introduce RLC layer timer, the timer will starts when the packet arrives to RLC layer. However, a packet might be delivered from PDCP layer immediately or after a while upon arrival from application layer. Different arriving time happens even among RLC SDUs within a data flow, and this results in different auto-retransmission/polling time within its PDB. This difference happens due to internal implementation and unknown to network.

Proposal 4: PDCP discardTimer is reused for triggering auto-retransmission and polling

## 3.2 extra Auto-reTx conditions

Generally, ARQ retransmission is triggered when a NACK is received. However the details specification has to consider an extra condition that the SN falls within the range of Tx window as following. Similarly, we also need to check the details of how to specify auto-retransmission:

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

Generally, autonomous retransmission could be triggered once when remaining time is less than a configured threshold. however as shown in following figure, within the Tx window, each RLC SDU may be in different status such as

- RLC SDU has been ACKed

- RLC SDU pending for feedback after initial transmission or retransmission

- RLC SDU pending for retransmission after NACKed

Among all these RLC SDUs in TX window, if a RLC SDU has been positively acknowledged, there is no need to trigger auto-retransmission anymore; if a RLC SDU has been pending for retransmission already, there is as well no need to trigger auto-retransmission once again. Besides, for both NACK-based retransmission and remaining time based retransmission, same retransmission action should be done once transmission opportunity (UL grant) comes.

With above analysis, following TP could be considered:

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. If [thresholdforAutoTx] is configured for the transmitting side of the AM RLC entity, when [timer] become less than [thresholdforAutoTx] for an RLC SDU, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer, and if the corresponding RLC SDU or RLC SDU segment(s) is neither pending for retransmission already nor positively acknowledged by a STATUS PDU: - consider the corresponding RLC SDU or RLC SDU segment(s) for retransmission.

Proposal 5: Auto-Rx is triggered only for SDU pending for feedback, i.e., it should be neither pending for retransmission already nor positively acknowledged by a STATUS PDU. RAN2 consider the TP as shown in section 3.2 to merge auto-retransmission into retransmission.

## 3.3 RETX_COUNT and RLF

RLF could be tiggered when RETX_COUNT reaches maxRetxThreshold. RETX_COUNT is maintained per RLC SDU. RETX_COUNT is initiated as 0 when the RLC SDU is considered for retransmission for the first time, and will increment whenever another retransmission is triggered/considered as following:

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Following two open issues are relevant to each other:

- Whether RLF upon maximum retransmission should be disabled for enhanced RLC AM

- whether RETX_COUNT should increment as well when auto-retransmission is triggered/considered.

Upon maximum times of ARQ retransmissions reaches, it means a RLC PDU cannot be successfully delivered after (x times of ARQ retransmissions * y times of HARQ retransmissions) . This was considered as a rare case and RLF can be triggered, because firstly we assume that x * y would not be a small number, secondly RLC AM never advance transmission window unless reception of the RLC PDU is confirmed i.e. aim for lossless delivery. These assumptions have been changed with enhanced AM RLC: firstly x * y could be a small value for the short PDB service which we are working on; secondary, RLC AM may advance the TX window when PDB of a RLC SDU runs out. It becomes questionable whether RLF should be triggered at all for enhanced AM RLC entities.

If RLF is skipped then we do not need to maintain RETX_COUNT if enhanced AM RLC featured is configured.

If RLF is still considered, then we need to discuss the second issue. Auto retransmission will speed up retransmission, it impacts how fast the maximum retransmissions will reaches, or in another word it will impact how many retransmissions it can do within its PDB. NW could configured a higher maximum retransmission times if auto retransmission is counted. Because we agreed auto retransmission is only triggered once, it is simple to adjust the configuration. If auto-retransmission is not counted, and maximum retransmission times configuration could be kept the same, we slightly prefer later way for simplicity.

Proposal 6: RAN2 discuss following options regarding RLF and RETX_COUNT for enhanced AM RLC entity

Option1: RETX_COUNT and RLF trigger part is skipped

Option2: keep RETX_COUNT and RLF trigger part, auto-retransmission is not counted into RETX_COUNT

Option3: keep RETX_COUNT and RLF trigger part, auto-retransmission is counted into RETX_COUNT

## 3.4 Efficient polling

Some companies concerned excessive polling based on remaining time e.g.:

- Each RLC SDU in TX window maintains a different remaining time, so they may trigger polling one after another.

- Remaining time of a RLC SDU will keep being lower than the threshold once it happens, this RLC SDU can keep on polling

Excessive polling itself will not cause extra signalling overhead, because it is just a bit setting between 0 and 1. However it may result in excessive SR. Currently SR frequency is limited by two factors a) HARQ reordering time and b) t-StatusProhibit:

Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size. NOTE 1: This ensures that the RLC Status report is transmitted after HARQ reordering. ///////////skip irrelevant//// When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

Assuming Tx side keep polling as long as any RLC SDU’s remaining time is lower than the threshold, and polling bit will be sent for every AMD PDU transmissions, but SR will not be sent more frequently than every prohibit time or every HARQ reordering time. it seem ok to keep polling as long as a faster SR is required:

Observation1: excessive polling will lead to neither excessive SR nor excessive signalling overhead

Besides, not all RLC SDU or RLC SDU segment having remaining time lower than the polling threshold should trigger polling. For these RLC SDU or RLC SDU segments which has been ACKed or already pending for retransmission, a further feedback is not necessary, hence:

Observation2: only for these RLC SDUs or RLC SDU segments waiting for acknowledgment in the TX buffer, remaining time based polling could be triggered

One the other hand, as following, existing polling triggers are classified into two categories:

- As highlighted in yellow: include a poll when newly transmitted PDU number or bytes excesses corresponding threshold. In this case, polling will be included with additional condition that AMD PDU submitted for transmission contains new data. This condition should not be applied to the remaining time based polling.

- As highlighted in green: include a poll when TX buffer is in an extreme case, the buffer has neither new RLC SDU for initial transmission nor RLC PDU pending for retransmission. In this case, polling will be included without additional condition.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below.

We recommend to categorize remaining time-based polling into the second case:

Observation3: the remaining time based polling if triggered can be included in any AMD PDU submitted for transmission, but not limit to AMD PDU containing new data

In summary, our proposal is:

Proposal 7: include a poll in any AMD PDU submitted for transmission, “if any RLC SDU or RLC SDU segment waiting for acknowledgment in the TX buffer has remaining time less the configured threshold”.

# 4 Conclusion

In this contribution, we provided following observations and proposals regarding the details of RLC AM enhancement, TP of how to specify many of these proposals are also provided in corresponding discussion sections:

Proposal 0: The transmitting side of an AM RLC entity update the value of TX_Next_Ack to advance the transmitting window when discarding an RLC SDU.

Proposal 1: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU without exception, consequently outdated SDU is discarded, not pending for retransmission, and not for triggering polling etc.

Proposal 2: RAN2 confirm that STATUS PDU will ACK abandoned SDUs, and consider additional STATUS Report trigger upon t-RxDiscard expires.

Proposal 3: RAN2 discuss if the feature “skip outdated/abandoned SDU” should be supported by UE/enabled by NW jointly or independently for DL and UL directions:

- If go with independent way: a enable bit is needed for UL direction, two capability bits are needed

- If go with joint way: enable bit is not needed, presence of new timer configuration will implicitly enable the feature in both DL and UL directions and one capability bit is enough

Proposal 4: PDCP discardTimer is reused for triggering auto-retransmission and polling

Proposal 5: Auto-Rx is triggered only for SDU pending for feedback, i.e., it should be neither pending for retransmission already nor positively acknowledged by a STATUS PDU. RAN2 consider the TP provided in section 3.2 to merge auto-retransmission into retransmission.

Proposal 6: RAN2 discuss following options regarding RLF and RETX_COUNT for enhanced AM RLC entity

Option1: RETX_COUNT and RLF trigger part is skipped

Option2: keep RETX_COUNT and RLF trigger part, auto-retransmission is not counted into RETX_COUNT

Option3: keep RETX_COUNT and RLF trigger part, auto-retransmission is counted into RETX_COUNT

Observation1: excessive polling will lead to neither excessive SR nor excessive signalling overhead

Observation2: only for these RLC SDUs or RLC SDU segments waiting for acknowledgment in the TX buffer, remaining time based polling could be triggered

Observation3: the remaining time based polling if triggered can be included in any AMD PDU submitted for transmission, but not limit to AMD PDU containing new data

These leads to:

Proposal 7: include a poll in any AMD PDU submitted for transmission, “if any RLC SDU or RLC SDU segment waiting for acknowledgment in the TX buffer has remaining time less the configured threshold”.