3GPP TSG-RAN WG2 Meeting #129bis	                 R2-2502991 (revision of R2-2502865)

Wuhan, China, Apr. 7th – 11th , 2025

Source:        ZTE Corporation

Title:           Consideration on Introduction of SRS capability reporting for SRS-only cell

Agenda item:   5.1.3.2

Document for:  Discussion and Decision

- Introduction

In the last meeting, an agreement has been made for the SRS capability on the SRS only cell.

Agreements - Allow SRS switching capability reporting for a band without FeautureSetUplink by introducing the following capabilities in FeatureSetDownlink: - zeroSlotOffsetAperiodicSRS; - offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16; - offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16; - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16; - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16; - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16; - srs-AntennaSwitching2SP-1Periodic-r17; - srs-ExtensionAperiodicSRS-r17; - srs-OneAP-SRS-r17 CRs to be seen next meeting, including discussing which release.

In this contribution, we’d like to share our views on how to introduce these capabilities.

- Discussion

As shown above, these capabilities are from the different versions from the R15. There are 2 options to introduce these capabilities:

- Option 1: Introduce each capability from the corresponding release version

- Option 2: Introduce these capabilities from the R18 and allow the early implementation.

For the option 1, since there have been many extension brunches these capability need to be added as highlighted in Red in the Fig 1, obviously, it would introduce more brunches.

Fig 1: UE Capability Extension Brunches

For the option 2, it would be quite easy and simple. Though these capabilities are from the different versions and thus the earliest implementable release version would be different, a NOTE can be added to the “Additional Information” as in the 38331 Annex C” (e.g. the earliest implementable release depends on the release version of each capability) or it can be specified in the cover page.

Table C-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): : CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| xxxxx | xxxx | xxxx | 15 | For the capabilites that late than the R15, the earliest implementable release depends on the release version of each capability |

Proposal 1: Introduce the below SRS capabilities on the FeatureSetDownlink from the R18 and allow the early implementation.

  - zeroSlotOffsetAperiodicSRS;

  - offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16;

  - srs-AntennaSwitching2SP-1Periodic-r17;

  - srs-ExtensionAperiodicSRS-r17;

  - srs-OneAP-SRS-r17

The other issue is that these capabilities are for the SRS carrier switch from the SRS-only cell, and there is another capability to indicate whether the UE support SRS carrier switch. Thus these capabilities can only be reported on the band that support SRS carrier switching.

BandParameters-v1540 ::= SEQUENCE { srs-CarrierSwitch CHOICE { nr SEQUENCE { srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR }, eutra SEQUENCE { srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA } }

srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on

Proposal 2: Add a precondition to the newly introduced capability in the FeatureSetDownlink, e.g. it is only present when the UE supports SRS carrier switching from this band.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Introduce the below SRS capabilities on the FeatureSetDownlink from the R18 and allow the early implementation.

  - zeroSlotOffsetAperiodicSRS;

  - offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16;

  - offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16;

  - srs-AntennaSwitching2SP-1Periodic-r17;

  - srs-ExtensionAperiodicSRS-r17;

  - srs-OneAP-SRS-r17

Proposal 2: Add a precondition to the newly introduced capability in the FeatureSetDownlink, e.g. it is only present when the UE supports SRS carrier switching from this band.