3GPP TSG-RAN WG2 Meeting #95	R2-165626

# Göteborg, Sweden, Aug. 22 – 26, 2016

Agenda item:	8.11.3 (NB_IOTenh-Core)

Source:	LG Electronics Inc.

Title: 	Fast RRC connection re-establishment in NB-IoT

Document for:	Discussion and Decision

Introduction

In the RAN #72 meeting, a proposed WID [1] titled “Enhancements of NB-IoT” was approved. The objective of this WI includes:

Mobility and service continuity enhancements

Enhancement(s) to connected mode mobility in order to improve service continuity and avoid NAS recovery for 	both CP and UP solutions without the increasing of UE power consumption.

In this contribution, we propose fast RRC connection re-establishment as a potential solution to avoid NAS recovery without the increasing of UE power consumption.

Discussion

In the current LTE specification, fast RRC connection re-establishment provides fast AS level recovery and is triggered only when a UE is in RLF. RRC connection re-establishment procedure can be used to support UE centric mobility in RRC_CONNECTED in NB-IOT not only for handling abnormal situation e.g. RLF but also for supporting connected mobility [2]. When a NB-IoT UE finds a better cell, UE performs connection re-establishment towards the target cell, regardless of RLF.

Proposal 1: Fast RRC connection re-establishment in LTE is also applied to NB-IOT for connected mobility.

If proposal 1 is applied, there will be short service interruption before success of the fast RRC connection re-establishment. However, UE may unnecessarily move to a target cell even though a source cell quality is sufficient. In order to avoid such undesired situation, we propose that a NB-IoT UE initiates RRC connection re-establishment towards the target cell regardless of RLF only when the entering condition of A3 event (e.g. when a target cell quality is better than a threshold or a source cell quality) is fulfilled only if a service cell quality is lower than a certain threshold. Since dedicated measurement configuration will cause additional UE battery consumption, the configuration needs to be broadcast via system information.

Proposal 2: When the entering condition of A3 event is fulfilled only if a service cell quality is lower than a certain threshold, a NB-IoT UE initiates RRC connection re-establishment towards the target cell regardless of RLF.

Proposal 3: The configuration related to fast RRC connection re-establishment (e.g. offset for A3 event) needs to be broadcast via system information.

Conclusion

Proposal 1: Fast RRC connection re-establishment in LTE is also applied to NB-IOT for connected mobility.

Proposal 2: When the entering condition of A3 event is fulfilled only if a service cell quality is lower than a certain threshold, a NB-IoT UE initiates RRC connection re-establishment towards the target cell regardless of RLF.

Proposal 3: The configuration related to fast RRC connection re-establishment (e.g. offset for A3 event) needs to be broadcast via system information.

References

[1] RP-161324, New work item proposal: Enhancements of NB-IoT, Vodafone

[2] R2-165625, Need for UE centric mobility for service continuity, LG Electronics