3GPP TSG-RAN WG2 Meeting #97bis	R2-1703490

Spokane, USA, 3rd – 7th April 2017	Revision of R2-1701546

Agenda Item	: 9.2.2 (LTE_STTIandPT-Core)

Source	: LG Electronics Inc.

Title	: DRX with short TTI length

Document for	: Discussion and Decision

1.	Introduction

In RAN2#97, RAN2 agreed that Common DRX configuration per MAC entity is applied. Enhancements for sTTI sPDCCH monitoring can be considered. However, it has not been decided yet how to count the DRX timers considering sTTI.

In this contribution, we further discuss the DRX timers by considering the shortened TTI length.

2.	Discussion

WI on short TTI and processing time for LTE have its objective including short TTI operation and reduced processing time for both legacy (1ms). While RAN1 is still discussing the detailed structure for sTTI, the implication of sTTI is that timers may need to be scaled based on the short TTI length. Therefore, it seems that RAN2 can start to discuss DRX aspect.

There are several DRX timers, which are summarized in the Table below:

Table 1 DRX timers in MAC

| Timer Type | Value in number of… | Timers |
| --- | --- | --- |
| Type 1 | PDCCH-subframe | onDurationTimer drx-InactivityTimer drx-RetransmissionTimer drx-ULRetransmissionTime |
| Type 2 | subframe | HARQ RTT Timer UL HARQ RTT Timer ongDRX-CycleStartOffset shortDRX-Cycle |
| Type 3 | shortDRX-Cycle | drxShortCycleTimer |

The question here is how to calculate the DRX timers for differnet TTI length. Considering that there are reasons in having different counting rule for each type of timer, it would be good to discuss them seperately.

Type 1 DRX timers counting PDCCH-subframes

The intention of setting the DRX timers in number of PDCCH subframes is to offer a specific number of scheduling opportunity, i.e., certain number of PDCCHs. Therefore, there could be two approaches in calculating these timers:

- Option 1. Count the number of PDCCH-subframe regardless of TTI length.

- Option 2. Count the number of PDCCH opportunity by considering TTI length.

For better understanding, let’s practice by assuming that DRX timer is set to psf4, and scheduling can be done based on 1 slot of 0.5ms. In Option 1, a DRX timer counts 4 PDCCH subframes, whereas in Option 2, a DRX timer counts 4 slots, i.e., 2 PDCCH subframes.

In Option 1, if the scheduling is based on 1 slot of 0.5ms TTI length, the UE may have more scheduling opportunities during the Active Time, e.g., 8 PDCCH opportunities, compared to Option 2, e.g., 4 PDCCH opportunities. Considering that the intention of short TTI is to offer more scheduling opportunities, counting the timer as in the legacy (Option 1) would be acceptable even for the scheduling with short TTI.

On the other hand, the intention of short TTI is to reduce the latency in scheduling, hence, it would also make sense only to offer the same scheduling opportunities by counting the actual scheduling opportunities. However, in case the network wants to schedule the UE based on 1 ms TTI for some reason even when short TTI scheduling is possible, Option 2 may take scheduling opportunities away from the UE/eNB.

As each DRX timer plays a different role in PDCCH monitoring, different option may need to be applied to each DRX timer.

For example, drx-InactivityTimer and onDurationTimer could keep counting the number of PDCCH subframes (Option 1) because those timers are defined per UE and should guarantee sufficient scheduling opportunities when multiple TTI lengths are used simultaneously.

Proposal 1. For drx-InactivityTimer and OnDurationTimer, the UE counts PDCCH-subframe.

In the meanwhile, drx-RetransmissionTimer and drx-ULRetransmissionTimer may need to count the number of PDCCH opportunities (Option 2) because those timers are defined per HARQ process and depend on the TTI length that is to be used for scheduling of retransmission.

On the other hand, in case TTI length switching between new transmission and retransmission is supported, the situation is similar to drx-InactivityTimer and onDurationTimer. In other words, the UE should be ready to receive scheduling with any TTI length. Thus, drx-RetransmissionTimer and drx-ULRetransmissionTimer need to count the number of PDCCH-subframe as in the legacy.

In RAN1, however, it is still under discussion whether to support switching between new transmission and retransmission. Thus, RAN2 can discuss how to count retransmission related timers only after deciding support of TTI length switching between new transmission and retransmission.

Proposal 2. For drx-RetransmissionTimer and drx-ULRetransmissionTimer, RAN2 discuss how to count the timer after RAN1 decides whether to support TTI length switching between new transmission and retransmission.

3.	Conclusion

In this contribution, we discuss the DRX timers in supporting short TTI and propose that:

Proposal 1. For drx-InactivityTimer and OnDurationTimer, the UE counts PDCCH-subframe.

Proposal 2. For drx-RetransmissionTimer and drx-ULRetransmissionTimer, RAN2 discuss how to count the timer after RAN1 decides whether to support TTI length switching between new transmission and retransmission.