3GPP TSG-RAN WG2 #98	R2-1704365

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.2.5

Source:	Ericsson

Title:	RLC UM receiver window operation

Document for:	Discussion, Decision

# Introduction

In RAN2#97bis, it was agreed that RLC UM receive window operation is maintained similar to LTE:

=> Duplicate detection functionality is kept as a baseline. FFS if duplicate detection can be removed. => RLC UM receive window operation is maintained similar to LTE. If duplicate detection is removed from RLC UM then the need for the window will depend on the mechanism use to discard.

This contribution reviews how the LTE RLC receive window operates and proposes NR to adopt similar mechanism. Furthermore, we discuss reasons why SN should be included in all PDUs, and T-reordering, or a similar timer, should be maintained in NR RLC UM.

# Discussion

## On the need of RLC UM PDU Sequence Number

It has been agreed in RAN2 that Sequence Numbers (SNs) are used when RLC segments the RLC SDU, that is, the same SN is assigned to all segmented SDUs.

As baseline the NR RLC UM receiver window operation should follow that of LTE. To be able to maintain a receiver window and update these variables, sequence numbers must be contained in all PDUs.

All RLC UM PDUs must have a SN

## On the need of duplicate detection and discard function

If SN is included for all RLC SDUs, detection of duplicate SDUs becomes trivial and can be implemented similarly as in LTE (see text proposal in Annex). In any case, RLC SDUs with duplicate SN will be monitored for reassembly of segmented SDUs. If none of the detected duplicate SDUs would not have SO-field and FI-field in the corresponding PDU, then the SDU could be discarded.

UM receiver window, similarly as in LTE, should be implemented to keep track of missing or late SDUs, e.g., to track the range of SNs for duplicate detection. The LTE UM receiver window operation, which should be adopted for NR as well, is described in more detail in the next section.

Having a duplicate detection and receiver window functionality in RLC would be beneficial especially in deployments where PDCP and RLC would be placed in different locations. In this case, it would be of value to not immediately send all RLC SDUs to PDCP but use a UM receiver (or reordering) window.

An additional advantage for having the receiver window and discard functionality in RLC is to lower the probability for desynchronized HFN in PDCP.

Duplicate detection and discard functions are maintained in RLC UM

## Detection of missing RLC SDU segments

Missing RLC SDU segments, and their relative position in the complete SDU, can be detected by tracking the SO-fields of the received segments. Additionally, RAN2 has agreed to include a “FI-like” field to distinguish between first, middle and last SDU segments. The SO field is needed only for the middle and last segment. Together with SO, FI and duplicate SN, the RLC entity can detect missing segments and reassemble complete RLC SDUs.

Missing RLC SDU segments are detected by means of SN, FI field and SO field.

## Details of UM receiver window operation

In LTE, the RLC UM receiver window is maintained as (Figure 1):

- VR(UH) is updated when the received PDU contains a larger SN than the value in VR(UH), i.e the highest received state variable.

- VR(UR) is updated by three different mechanisms (receive state variable):

  - Reception of a RLC PDU or all segments of an RLC PDU,

  - Pulled by the update of the VR(UH),

  - Expiration of T-reordering.

- UM_Window_Size - this constant is used by the receiving UM RLC entity to define SNs of those UMD PDUs that can be received without causing an advancement of the receiving window

- Reordering window is updated when VR(UH) is updated, i.e. the upper part of the reordering window. The window sets the range of SNs which the receiver can accept.

Figure 1. LTE RLC UM receiver window.

Similar functionality should be adopted for NR, but the procedure should operate on received RLC SDUs or SDU segments instead of PDUs.

In NR, also here a window should be used to keep track of the missing/late SDUs and treat (discarding or buffering) these packets if they are received within period denoted by the window. The reason to keep track of missing/late SDUs is because of how the receiver window is updated as stated above. Hence, VR(UH) is updated when a “newer” SN is received.

There may be cases in which the transmitted SDUs are lost and the transmitter window wraps around. When the UE receives again a SDU, it might not be able to distinguish if the SDU was an “old” SDU (which would update VR(UR)) or it is a new SDU (which would update VR(UH)). This functionality is especially useful in cases when the user plane flow is interrupted (e.g. losses) for a short period. This could happen during handovers, or when there is a deep fade which might be common at high frequencies. If the VR(UR) is not moved forward, a “new” SDU may be a missing SDU which will likely cause a HFN desync in PDCP.

There should thus be a timer in receiver RLC UM entity to handle the issues discussed above, for both SDUs and SDU segments. The timer allows for a period before VR(UR) is moved forward. This limits the possibility of receiving a new SDU or a SDU segment, treating it as an old SDU, and the other way around. It should be noted that such timer should not be confused with the in-order-delivery functionality of SDUs, although in LTE the corresponding timer is named “T-reordering”. In-order-delivery of SDUs should be considered as a different functionality which is not directly related to the timer “T-reordering”. We propose to maintain the “reordering window” and “T-reordering” which are used in the baseline LTE specification, however a naming to better distinguish the NR RLC functions could be “receiver window” and for example “T-receiver”, to avoid misconceptions.

In NR, RLC UM entity shall keep:

  - A receiver window (reordering window in LTE), and receiver window state variables VR(UR), VR(UH), VR(UX) and UM_Window_Size

  - RLC UM should keep a timer, T-receiver, with the similar functionality and operation as T-reordering in LTE

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	All RLC UM PDUs must have a SN

Proposal 2	Duplicate detection and discard functions are maintained in RLC UM

Proposal 3	Missing RLC SDU segments are detected by means of SN, FI field and SO field.

Proposal 4	In NR, RLC UM entity shall keep:

a.	A receiver window (reordering window in LTE), and receiver window state variables VR(UR), VR(UH), VR(UX) and UM_Window_Size

b.	RLC UM should keep a timer, T-receiver, with the similar functionality and operation as T-reordering in LTE

# Annex for information (based on 36.322)

### 5.1.2	UM data transfer

#### 5.1.2.2	Receive operations

##### 5.1.2.2.1	General

The receiving UM RLC entity shall maintain a receiver window according to state variable VR(UH) as follows:

-	a SN falls within the receiver window if (VR(UH) – UM_Window_Size) <= SN < VR(UH);

-	a SN falls outside of the receiver window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either discard the received UMD PDU or place it in the reception buffer (see sub clause 5.1.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Receiver as needed (see sub clause 5.1.2.2.3);

When t-Receiver expires, the receiving UM RLC entity shall:

-	update state variables, reassemble and deliver RLC SDUs to upper layer, discard segments which cannot be reassembled to complete SDU and start t-Receiver as needed (see sub clause 5.1.2.2.4).

##### 5.1.2.2.2	Actions when an UMD PDU is received from lower layer

When an UMD PDU with SN = x is received from lower layer, the receiving UM RLC entity shall:

-	if VR(UR) < x < VR(UH) and the UMD PDU with SN = x has been received before; or

-	if (VR(UH) – UM_Window_Size) <= x < VR(UR):

-	discard the received UMD PDU;

-	else:

-	place the received UMD PDU in the reception buffer.

##### 5.1.2.2.3	Actions when an UMD PDU is placed in the reception buffer

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if x falls outside of the receiver window:

-	update VR(UH) to x + 1;

-	reassemble RLC SDUs from any UMD PDUs with SN that falls outside of the reordering window, remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer;

-	if VR(UR) falls outside of the receiver window:

-	set VR(UR) to (VR(UH) – UM_Window_Size);

-	if the reception buffer contains an UMD PDU with SN = VR(UR):

-	update VR(UR) to the SN of the first UMD PDU with SN > current VR(UR) that has not been received;

-	reassemble RLC SDUs from any UMD PDUs with SN < updated VR(UR), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer;

-	if t-Receiver is running:

-	if VR(UX) <= VR(UR); or

-	if VR(UX) falls outside of the reordering window and VR(UX) is not equal to VR(UH):

-	stop and reset t-Receiver;

-	if t-Receiver is not running (includes the case when t-Receiver is stopped due to actions above):

-	if VR(UH) > VR(UR):

-	start t-Receiver;

-	set VR(UX) to VR(UH).

##### 5.1.2.2.4	Actions when t-Receiver expires

When t-Receiver expires, the receiving UM RLC entity shall:

-	update VR(UR) to the SN of the first UMD PDU with SN >= VR(UX) that has not been received;

-	reassemble RLC SDUs from any UMD PDUs with SN < updated VR(UR), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in ascending order of the RLC SN if not delivered before;

-	discard all RLC SDU segments with SN < VR(UR)

-	if VR(UH) > VR(UR):

-	start t-Receiver;

-	set VR(UX) to VR(UH).

===============

# 7	Variables, constants and timers

## 7.1	State variables

Each receiving UM RLC entity shall maintain the following state variables:

a) VR(UR) – UM receive state variable

This state variable holds the value of the SN of the earliest UMD PDU that is still considered for segment reassembly. It is initially set to 0.

b) VR(UX) – UM t-Receiver state variable

This state variable holds the value of the SN following the SN of the UMD PDU which triggered t-Receiver.

c) VR(UH) – UM highest received state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs, and it serves as the higher edge of the reordering window. It is initially set to 0.

## 7.3	Timers

The following timers are configured by RRC:

x) t-Receiver

This timer is used by the receiving side of an UM RLC entity to detect loss of RLC PDUs at lower layer (see sub clauses 5.1.2.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.