3GPP TSG-RAN WG2 #98	R2-1704371

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.3.3

Source:	Ericsson

Title:	Header compression in NR

Document for:	Discussion, Decision

# Discussion

## ROHC specifications

LTE specifications and the draft 38.323 TS currently point at the RFC 4996 specifications for ROHC. These specifications are obsolete as a new version, RFC6846, is available. This new version of the specification corrects errors from the previous version. Thus, it is important that RAN2 keeps an up-to-date version of the RFCs.

Rel-15 PDCP specifications should point to the ROHC RFC 6846 specifications

## ROHC feedback

ROHC feedback is transmitted in PDCP control PDUs mapped on the corresponding DRB. These PDCP PDUs do not contain a SN. It has been argued ROHC feedback shall be received in-sequence as it would break the compression algorithm. However, this is not correct.

- ROHC feedback is does not go through compression/decompression.

- ROHC feedback is only used to adapt the compressor confidence.

- ROHC feedback includes a MSN (Master Sequence Number) which identifies each compressed packet (created by the compressor). In other words, there is a one-to-one mapping between a compressed packet and a ROHC feedback packet.

As a result, for ROHCv2-profiles that should be used currently, the compressor can change it’s reordering tolerance and balance losses to 1/4, 1/2 or 3/4 of the window depending on the use of the MSN.

In conclusion, PDCP Control PDU format for interspersed ROHC feedback packet do not need a SN.

SN for PDCP Control PDU for interspersed ROHC feedback is not needed.

## ROHC configuration

In LTE, ROHC is configured for both DL and UL simultaneously. [1] has shown some use cases in which DL and UL could be configured independently, and RAN2 has agreed to make ROHC DL and UL configuration independent [2][3].

We think that the same situation as presented in [1,2,3] may appear in NR.

ROHC can be configured independently for the DL and UL

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Rel-15 PDCP specifications should point to the ROHC RFC 6846 specifications

Proposal 2	SN for PDCP Control PDU for interspersed ROHC feedback is not needed.

Proposal 3	ROHC can be configured independently for the DL and UL

# References

R2-1701761, ROHC Symmetric DL/UL Parameters Limitation in LTE. Apple, LG Electronics

R2-1703949, Enable Uplink-Only ROHC operations. Apple, MediaTek Inc

R2-1703951, Enable Uplink-Only ROHC operations. Apple, MediaTek Inc

# Text Proposal for TS 38.323

Note that the highlighted shows the differences with respect TS 36.323. Numbering of references, figures, and headings need to be updated by the rapporteur.

# 2	References

[a]	IETF RFC 4995: "The RObust Header Compression (ROHC) Framework".

[b]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[c]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[d]	IETF RFC 3843: "RObust Header Compression (ROHC): A Compression Profile for IP".

[e]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[f]	IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite”.

-----  NEXT UPDATE ------

## 5.7	Header compression and decompression

### 5.7.X	Supported header compression protocols and profiles

The header compression protocol is based on the Robust Header Compression (ROHC) framework [a]. There are multiple header compression algorithms, called profiles, defined for the ROHC framework. Each profile is specific to the particular network layer, transport layer or upper layer protocol combination e.g. TCP/IP and RTP/UDP/IP.

The detailed definition of the ROHC channel is specified as part of the ROHC framework in [a]. This includes how to multiplex different flows (header compressed or not) over the ROHC channel, as well as how to associate a specific IP flow with a specific context state during initialization of the compression algorithm for that flow.

The implementation of the functionality of the ROHC framework and of the functionality of the supported header compression profiles is not covered in this specification.

In this version of the specification the support of the following profiles is described:

Table 5.7.X.Y: Supported header compression protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 4995 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |