3GPP TSG-RAN WG2 #98	Tdoc R2-1704416

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	10.2.2

Source:	Ericsson

Title:	Inconsistencies regarding the agreements on the split bearer handling in case of RLF

Document for:	Discussion

# Discussion and Proposal

At the last RAN2 meeting the following agreements we made

Agreement

1:	If radio link failure is detected for MCG, UE initiates RRC connection re-establishment procedure with the PCell.

2:	If radio link failure is detected for SCG, UE suspends all SCG radio bearers (including SCG split bearers) and SCG transmissions for split radio bearers, and reports SCG failure information to MN.

And

Agreements:

1: In LTE-NR DC, following SgNB failure cases need to be supported:

-	SgNB RLF;

-	SgNB change failure;

-	exceeding the maximum uplink transmission timing difference (if EN-DC supports the synchronised operation case which is RAN1 decision);

-	SgNB configuration failure (only for message on SCG SRB);

-	SgNB RRC integrity check failure;

2: In LTE-NR DC, the UE shall report the SCGFailureInformation to the MeNB instead of triggering the reestablishment upon SgNB failure.

3: 	Upon SgNB failures, UE shall:

-	Suspend all SCG DRBs and suspend SCG transmission for MCG split DRBs, and SCG split DRBs;

-	Suspend direct SCG SRB and SCG transmission for MCG split SRB;

-	Reset SCG-MAC;

-	send the SCGFailureInformation message to the MeNB with corresponding cause values .

In our view these agreements are inconsistent since the first agreement states that all the SCG bearers (split/nosplit) are completely suspended, while in the second agreement only the SCG (nosplit) bearers and the SCG transmission of the split SCG bearers are suspended. In our view we think that second agreement is preferred for performance reasons i.e. that the MCG part of the SCG split bearer should be able to continue run at SeNB link failure. This would also make the handling of MCG split and SCG split bearer more consistent.

At SeNB failure only the SCG part of split bearers (MCG or SCG) should be suspended.