3GPP TSG-RAN WG2 Meeting #98	R2-1704491

Hangzhou, P. R. China, 15th – 19th May 2017

Source: 	AT&T

Title:	PDCP Status Reporting in NR

Agenda Item:	10.3.3

Document for:	Discussion

# Introduction

In this contribution we examine the conditions under which a UE is required to provide PDCP status report for LTE specifications, and provide justification for expansion of those conditions for NR PDCP specifications to better enable PDCP recovery at handover or for DC.

# Discussion

At the RAN2 #97bis meeting it was agreed to have some PDCP recovery mechanism based on PDCP Status Reports at least for the handover case and DC as stated below:

Agreements on PDCP recovery

=>	Some PDCP recovery mechanism based of PDCP status report is supported at least for the handover case and DC.  FFS if there are other cases in which this may be performed.

Per LTE PDCP specification 3GPP TS 36.423, for radio bearers that are mapped to RLC AM, the UE only sends a PDCP status report under two main conditions:

- When upper layers request a PDCP re-establishment

- For LWA operation, when:

  - PDCP Data PDU with polling bit P set to 1 is received, and UE is configured to send PDCP status report

  - t-StatusReportType1 expires, and UE is configured to send PDCP status report

Based on the RAN2 #97bis agreement stated above, for NR PDCP specification TS 38.423, the UE must be required to send a PDCP Status Report under a much broader set of conditions. In general, for radio bearers that are mapped to RLC AM, and when the UE is configured to send PDCP status report, the UE must respond with a PDCP status report transmission:

- In response to a poll – i.e. when PDCP Data PDU with polling bit P is set to 1 is received

- In response to a timer expiration – i.e. when status report time expires (equivalent of t-StatusReportType1)

Proposal 1: We propose that for radio bearers that are mapped to RLC AM, and when the UE is configured to send PDCP status reports, the UE must respond with a PDCP status report transmission:

- In response to a poll – i.e. when PDCP Data PDU with polling bit P is set to 1 is received

- In response to a timer expiration – i.e. when status report time expires (equivalent of t-StatusReportType1)

# Summary

In this contribution we examined the conditions under which a UE is required to provide PDCP status report for LTE specifications, and provided justification for expansion of those conditions for NR to better enable PDCP recovery at handover or for DC. We offer the following proposal for consideration:

Proposal 1: We propose that for radio bearers that are mapped to RLC AM, and when the UE is configured to send PDCP status reports, the UE must respond with a PDCP status report transmission:

- In response to a poll – i.e. when PDCP Data PDU with polling bit P is set to 1 is received

- In response to a timer expiration – i.e. when status report time expires (equivalent of t-StatusReportType1)