3GPP TSG-RAN WG2 Meeting #98                                                                   R2-1705383

Hangzhou, China, May 15 - 19, 2017

Title:	[DRAFT] Reply LS on NR Initial access

Response to:	R2-1703926 [R1-1706707]

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Samsung (to be replaced by RAN2)

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Mangesh Abhimanyu Ingale

#### E-mail Address:	m.ingale(at)samsung(dot)com

Attachments:	None

1. Overall Description:

RAN2 is thankful to RAN1 for LS R2-1703926 [R1-1706707] on the progress of initial access in NR.

Regarding the following aspect from the LS, RAN2 had discussions on the UE requirements to check if there is violation.

- Time index indication: PBCH conditioned that mobility and HO related requirements can be met - Note: RAN1 assumes that RAN2 will check against to RAN2 requirements

Referring to the LTE requirements upon handover specified in section 5.3.5.4 of TS 36.331 and NR requirements in section 10.2.2 of TR 38.804, RAN2 shares common understanding based on the following observations:

Observation#1: In RAN2 perspective, LTE UE is not required to decode PBCH of the target cell during handover.

Observation#2: In RAN2 perspective, NR UE is not required to decode PBCH of target cell(s) for RRM measurement reporting and during handover.

Based on Observation#1 and Observation#2, RAN2 believe the principle shall not be broken without strong justification and provides following guidance to RAN1:

RAN1 is kindly informed to design NR-SS block/PBCH for time indication which does not impose UE requirement to decode PBCH of target cell(s) for RRM measurement reporting and during handover.

2. Actions:

To RAN1

RAN2 respectfully asks RAN1 to take above guidance into account for further progress on initial access and NR-SS block/PBCH design.

4. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #NR-2	June 27-29, 2017		Qingdao, China

TSG RAN WG2 Meeting #99	August 21-25, 2017		Berlin, Germany