3GPP TSG-RAN2 Meeting #98	R2-1705462

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1113 | rev | - | Current version: | 14.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resources selection for PRACH triggered by a PDCCH order in eNB-IoT(Option2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRACH initiated by a PDCCH order has been introduced from R13 NB-IoT. The PRACH resource can be explicitly indicated by eNB through PDCCH order. However, without reserved PRACH resource and non-contention based random access procedure, the benefit of PRACH initiated by a PDCCH order cannot be fully achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Deterministic subcarrier selection at CEL change with consideration on reserved PRACH resource configuration is modified in section 5.1.2. - Deterministic carrier selection at CEL change with consideration on reserved PRACH resource configuration is introduced in section 5.1.1 and 5.1.4. - Remove the restrictions that NB-IoT UE doesn’t support non-contention based random access procedure in section 5.1.4. - It’s clarified that for the non-contention based random access procedure, the UL grant or DL assignment contained in the RAR is valid for the configured carrier if the UE is in RRC_CONNECTED and previously configured a carrier in section 5.1.4. Impact analysis Impacted functionality: The changes only impact the corrected functionality. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE will fail to determine PRACH resource for PRACH transmission in some scenarios. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT UE doesn’t support non-contention based random access procedure and the benefit of PRACH initiated by a PDCCH order cannot be fully achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.1.1	Random Access Procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC sublayer itself or by the RRC sublayer. Random Access procedure on an SCell shall only be initiated by a PDCCH order. If a MAC entity receives a PDCCH transmission consistent with a PDCCH order [5] masked with its C-RNTI, and for a specific Serving Cell, the MAC entity shall initiate a Random Access procedure on this Serving Cell. For Random Access on the SpCell a PDCCH order or RRC optionally indicate the ra-PreambleIndex and the ra-PRACH-MaskIndex, except for NB-IoT where the subcarrier index is indicated; and for Random Access on an SCell, the PDCCH order indicates the ra-PreambleIndex with a value different from 000000 and the ra-PRACH-MaskIndex. For the pTAG preamble transmission on PRACH and reception of a PDCCH order are only supported for SpCell. If the UE is an NB-IoT UE and is configured with a non-anchor carrier, the Random Access procedure is performed on the anchor carrier or one of the non-anchor carriers for which PRACH resource has been configured.

Before the procedure can be initiated, the following information for related Serving Cell is assumed to be available for UEs other than NB-IoT UEs, BL UEs or UEs in enhanced coverage [8], unless explicitly stated otherwise:

-	the available set of PRACH resources for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group (SpCell only):

The preambles that are contained in Random Access Preambles group A and Random Access Preambles group B are calculated from the parameters numberOfRA-Preambles and sizeOfRA-PreamblesGroupA:

If sizeOfRA-PreamblesGroupA is equal to numberOfRA-Preambles then there is no Random Access Preambles group B. The preambles in Random Access Preamble group A are the preambles 0 to sizeOfRA-PreamblesGroupA – 1 and, if it exists, the preambles in Random Access Preamble group B are the preambles sizeOfRA-PreamblesGroupA to numberOfRA-Preambles – 1 from the set of 64 preambles as defined in [7].

-	if Random Access Preambles group B exists, the thresholds, messagePowerOffsetGroupB and messageSizeGroupA, the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10], and the offset between the preamble and Msg3, deltaPreambleMsg3, that are required for selecting one of the two groups of Random Access Preambles (SpCell only).

-	the RA response window size ra-ResponseWindowSize.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6).

-	the maximum number of Msg3 HARQ transmissions maxHARQ-Msg3Tx (SpCell only).

-	the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only).

NOTE:	The above parameters may be updated from upper layers before each Random Access procedure is initiated.

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage [8]:

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the available set of PRACH resources associated with each enhanced coverage level supported in the Serving Cell for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group(SpCell only):

The preambles that are contained in Random Access Preamble groups for each enhanced coverage level, if it exists, are the preambles firstPreamble to lastPreamble.

If sizeOfRA-PreamblesGroupA is not equal to numberOfRA-Preambles, Random Access Preambles group B exists for all enhanced coverage levels and is calculated as above.

NOTE:	If Random Access Preamble group B exists, the eNB should ensure that at least one Random Access Preamble is contained in Random Access Preamble group A and Random Access Preamble group B for all enhanced coverage level.

-	if the UE is a NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, nprach-ConfigList-r14.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart and nprach-NumCBRA-StartSubcarriers as specified in TS 36.211 [7, 10.1.6.1]. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList.

-	each enhanced coverage level has one anchor carrier PRACH resource present in nprach-ParametersList and zero or one PRACH resource for each non-anchor carrier signalled in nprach-ConfigList-r14.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities:

-	the selection probability for the anchor carrier PRACH resource is given by nprach-ProbabilityAnchor

-	the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

-	if one or more PRACH resources are configured nprach-NumCBRA-StartSubcarriers with value less than nprach-NumSubcarriers for each enhanced coverage level:

-	these carriers should be used as reserved PRACH resources for this enhanced coverage level . These carriers should be indexed from 0 to the number of reserved PRACH resources -1for this enhanced coverage level. If anchor carrier is in the reserved PRACH resources, the index for the anchor carrier is ‘0’. The indexes for the non-anchor carriers are consistent with their order in the ul-CarrierConfigList.

-	else:

-	the carriers provide PRACH resources for the same enhanced coverage level are indexed from 0 to the number of PRACH resources-1 for this enhanced coverage level. The index for the anchor carrier is ‘0’. The indexes for the non-anchor carriers are consistent with their order in the ul-CarrierConfigList.

NOTE:	If nprach-NumCBRA-StartSubcarriers with value less than nprach-NumSubcarriers exists, the eNB should ensure that at least one PRACH resource is configured nprach-NumCBRA-StartSubcarriers with value less than nprach-NumSubcarriers for each enhanced coverage level.

-	the criteria to select PRACH resources based on RSRP measurement per enhanced coverage level supported in the Serving Cell rsrp-ThresholdsPrachInfoList.

-	the maximum number of preamble transmission attempts per enhanced coverage level supported in the Serving Cell maxNumPreambleAttemptCE.

-	the number of repetitions required for preamble transmission per attempt for each enhanced coverage level supported in the Serving Cell numRepetitionPerPreambleAttempt.

-	the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c [10].

-	the RA response window size ra-ResponseWindowSize and the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax-CE.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see subclause 7.6). For NB-IoT the DELTA_PREAMBLE is set to 0.

The Random Access procedure shall be performed as follows:

-	Flush the Msg3 buffer;

-	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	set the PREAMBLE_TRANSMISSION_COUNTER_CE to 1;

-	if the starting enhanced coverage level, or for NB-IoT the starting number of NPRACH repetitions, has been indicated in the PDCCH order which initiated the Random Access procedure, or if the starting enhanced coverage level has been provided by upper layers:

-	the MAC entity considers itself to be in that enhanced coverage level regardless of the measured RSRP;

-	else:

-	if the RSRP threshold of enhanced coverage level 3 is configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 3 and the UE is capable of enhanced coverage level 3 then:

-	the MAC entity considers to be in enhanced coverage level 3;

-	else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

-	set the backoff parameter value to 0 ms;

-	for the RN, suspend any RN subframe configuration;

-	proceed to the selection of the Random Access Resource (see subclause 5.1.2).

NOTE:	There is only one Random Access procedure ongoing at any point in time in a MAC entity. If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

NOTE:	A NB-IoT UE measures RSRP on the anchor carrier.

<End of change>

<Start of  change>

5.1.2	Random Access Resource selection

The Random Access Resource selection procedure shall be performed as follows:

-	If, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else, for NB-IoT, if ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-PreambleIndex is larger than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers- nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset,  nprach-NumSubcarriers and nprach-NumCBRA-StartSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	For NB-IoT, randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	If Msg3 has not yet been transmitted, the MAC entity shall, for BL UEs or UEs in enhanced coverage:

-	select the Random Access Preambles group and the PRACH resource corresponding to the selected enhanced coverage level;

-	except for NB-IoT, if Msg3 has not yet been transmitted, the MAC entity shall, except for BL UEs or UEs in enhanced coverage in case preamble group B does not exists:

-	if Random Access Preambles group B exists and any of the following events occur:

-	the potential message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is greater than messageSizeGroupA and the pathloss is less than PCMAX,c (of the Serving Cell performing the Random Access Procedure) – preambleInitialReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB;

-	the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC header is greater than messageSizeGroupA;

-	select the Random Access Preambles group B;

-	else:

-	select the Random Access Preambles group A.

-	else, if Msg3 is being retransmitted, the MAC entity shall:

-	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

-	randomly select a Random Access Preamble within the selected group. The random function shall be such that each of the allowed selections can be chosen with equal probability;

-	except for NB-IoT, set PRACH Mask Index to 0.

-	determine the next available subframe containing PRACH permitted by the restrictions given by the prach-ConfigIndex (except for NB-IoT), the PRACH Mask Index (except for NB-IoT, see subclause 7.3), physical layer timing requirements [2] and in case of NB-IoT, the subframes occupied by PRACH resources related to a higher enhanced coverage level (a MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH subframe);

-	if the transmission mode is TDD and the PRACH Mask Index is equal to zero:

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe.

-	else:

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe and the next two consecutive subframes.

-	else:

-	determine a PRACH within the determined subframe in accordance with the requirements of the PRACH Mask Index, if any.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, select the ra-ResponseWindowSize and mac-ContentionResolutionTimer corresponding to the selected enhanced coverage level and PRACH.

-	proceed to the transmission of the Random Access Preamble (see subclause 5.1.3).

<End of change>

<Start of change>

5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, the MAC entity shall monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI defined below, in the RA Response window which starts at the subframe that contains the end of the preamble transmission [7] plus three subframes and has length ra-ResponseWindowSize. If the UE is a BL UE or a UE in enhanced coverage, RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding coverage level. If the UE is an NB-IoT UE, in case the number of NPRACH repetitions is greater than or equal to 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 41 subframes and has length ra-ResponseWindowSize for the corresponding coverage level, and in case the number of NPRACH repetitions is less than 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 4 subframes and has length ra-ResponseWindowSize for the corresponding coverage level. The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

# RA-RNTI= 1 + t_id + 10*f_id

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in Section 5.7.1 of [7].

For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

# RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10))

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in Section 5.7.1 of [7].

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

# RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see subclause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see subclause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	if the SCell is configured with ul-Configuration-r14, ignore the received UL grant otherwise process the received UL grant value and indicate it to the lower layers;

-	if, except for NB-IoT, ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC) or for NB-IoT, ra-PreambleIndex was explicitly signalled and it was larger than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers:

-	consider the Random Access procedure successfully completed.

-	If NB-IoT UEs in RRC_CONNECTED and the UE is previously configured with a carrier:

-     the UL grant provided in the Random Access Response message is valid only for the configured carrier.

-	else, if, except for NB-IoT, the Random Access Preamble was selected by the MAC entity, or for NB-IoT, the Random Access Preamble was selected by the MAC entity or ra-PreambleIndex was explicitly signalled and it was equal to or less than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers:

-	set the Temporary C-RNTI to the value received in the Random Access Response message no later than at the time of the first transmission corresponding to the UL grant provided in the Random Access Response message;

-	if this is the first successfully received Random Access Response within this Random Access procedure:

-	if the transmission is not being made for the CCCH logical channel, indicate to the Multiplexing and assembly entity to include a C-RNTI MAC control element in the subsequent uplink transmission;

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

NOTE:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

If no Random Access Response is received within the RA Response window, or if none of all received Random Access Responses contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, the Random Access Response reception is considered not successful and the MAC entity shall:

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if the Random Access Preamble is transmitted on an SCell:

-	consider the Random Access procedure unsuccessfully completed.

-	if in this Random Access procedure, the Random Access Preamble was selected by MAC:

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	else if the SCell where the Random Access Preamble was transmitted is configured with ul-Configuration-r14:

-	delay the subsequent Random Access transmission until the Random Access Procedure is initiated by a PDCCH order with the same ra-PreambleIndex and ra-PRACH-MaskIndex;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	increment PREAMBLE_TRANSMISSION_COUNTER_CE by 1;

-	if PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding enhanced coverage level + 1:

-	reset PREAMBLE_TRANSMISSION_COUNTER_CE;

-	consider to be in the next enhanced coverage level, if it is supported by the Serving Cell and the UE, otherwise stay in the current enhanced coverage level;

-	for a BL UE or a UE in enhanced coverage, select the Random Access Preambles group and the PRACH resource corresponding to the selected enhanced coverage level;-	if the UE is an NB-IoT UE:

-	if the Random Access Procedure was initiated by a PDCCH order:

-	if  at least one reserved PRACH resources in the selected enhanced coverage:

-	select one of the PRACH resources which correspond to a UL carrier whose index among the multiple carriers for the selected enhanced coverage level is equal to [(signalled carrier indication) modulo (number of reserved PRACH resources corresponding to the selected enhanced coverage)];

-	else:

-	select one of the PRACH resources which correspond to a UL carrier whose index among the multiple carriers for the selected enhanced coverage level is equal to [(signalled carrier indication) modulo (number of PRACH resources corresponding to the selected enhanced coverage)];

-	consider the selected PRACH resource as explicitly signalled;

-	proceed to the selection of a Random Access Resource (see subclause 5.1.2).

<End of change>