3GPP TSG-RAN WG2 Meeting #98                                                              				         R2-1705784

Hangzhou, China, 15th-19th May 2017                                                    Revision of R2-1702925

Agenda item:	10.2.2

Source:	CMCC

Title:	Introducing Transmission Mode for PDCP Operation

Document for:	Discussion

Introduction

RAN2#97 has agreed that NR RLC supports three transmission modes, i.e., AM, UM and TM, and split bearer supports RLC AM as well as UM. In this contribution, we discuss the PDCP procedures for DRBs with RLC AM and UM, and present the necessity of introducing PDCP transmission modes in NR based on agreements of RLC transmission modes.

Discussion

RLC mode in LTE

According to [1], an RLC entity can be configured to perform data transfer in three modes: Transparent Mode (TM), Unacknowledged Mode (UM) and Acknowledged Mode (AM). Among them, TM is out of our discussion scope since RLC SDUs in TM are usually delivered to RRC directly without any operations at RLC. And main differences between UM and AM lie in ARQ, re-segmentation and reordering. More specifically, ARQ mechanism requires RLC to recognize the lost RLC PDU when reordering is performed at the receiving side, and retransmit the lost RLC PDU with re-segmentation if needed at the transmitting side. As the result, the main difference between RLC AM and UM is focused on the reordering operation.

In order to perform reordering which guarantees the in-sequence delivery, a transmitting window is adopted at the transmitting side for each AM RLC entity, and a receiving/reordering window is adopted at the receiving side of each RLC entity. In addition, several state variables are introduced for reordering operation. Table 1 summarizes the differences on state variables between UM and AM based on [1]. As can be seen in Table 1, UM adopts less state variables than AM because acknowledgement is not obligatory in UM. When the reordering timer is expired, UM RLC entity shall deliver the reassembled RLC SDUs to upper layer in ascending order of the RLC SN no matter the lost UM PDU is received or not.

Observation 1: In LTE, reordering operation is the main difference between RLC AM and UM.

Table 1 Differences on state variables for reordering between DRB with RLC AM and UM

|  | DRB with RLC AM mode | DRB with RLC UM mode |
| --- | --- | --- |
| Transmitting side | Acknowledgement state variable Maximum send state variable Send state variable Poll send state variable | N/A |
| Receiving side | Receive state variable Maximum acceptable receive state variable t-Reordering state variable Maximum STATUS transmit state variable Highest received state variable | UM receive state variable UM t-Reordering state variable UM highest received state variable |

PDCP operations in NR

In RAN2#95bis meeting, it was agreed that PDCP reordering is always enabled if in-sequence delivery to layer above PDCP is needed (i.e. even in non-DC case). In order to achieve in-sequence delivery in some service (such as TCP), state variables and transmitting/receiving windows should be adopted in NR PDCP when reordering is performed at PDCP. PDCP operation is related to the DRB mapped RLC mode. For RLC AM, PDCP have to guarantee the in-sequence delivery and data lossless in re-establishment and DC, which drives PDCP to have the ability to configure state variables for retransmission and data recovery. While for RLC UM, it is not necessary for PDCP to guarantee the in-sequence delivery during re-establishment. As a result, the PDCP procedures, functions and configurations for different RLC modes are obviously different.

Observation 2:  PDCP performs different procedures, functions and configurations for different RLC modes when reordering is operated at PDCP in NR.

The service requirements are also various and should be treated differently at PDCP. For instance, as mention in [2], duplication procedure is introduced in PDCP to achieve high reliability which is suitable for URLLC services; and for mMTC, overhead is the main concern since small packets generated by mMTC services are more sensitive to overhead. Therefore, PDCP should have the ability to support differentiated operations for different services.

Observation 3: In order to satisfy disparate requirements for different services, PDCP maintains different sets of PDCP procedures, functions and configurations.

PDCP Mode

To adapt to disparate requirements and clarify PDCP operations for different services, the PDCP transmission mode should be introduced in NR, which helps PDCP to process the traffic from different services accurately.

In this contribution, a possible division on PDCP mode is described. Note that the division given below is just an example which can be further discussed.

This example adopts four PDCP transmission modes, i.e. PDCP Mode1, PDCP Mode2, PDCP Mode3 and PDCP Mode4, to satisfy three typical scenarios in NR. The corresponding RLC mode which operates under each PDCP mode is given. When assigning RLC mode to PDCP mode, three factors are taken into consideration: reliability, delay-sensitivity and energy efficiency. In general, RLC AM is more suitable to support services with higher reliability, less delay sensitivity and less energy efficiency; while RLC UM is on the contrary.

PDCP Mode1 is configured to transfer delay-insensitive service with high reliability in eMBB, which is supported by RLC AM.

PDCP Mode2 is configured to transfer delay-sensitive service with low reliability in eMBB, which is supported by RLC UM.

PDCP Mode3 is configured to transfer delay-sensitive service with high reliability in URLLC, which is supported by RLC AM/UM.

PDCP Mode4 is configured to transfer small packets in mMTC, which is supported by RLC UM.

The detailed procedures, functions and configurations for each mode are given in Table 2.

| PDCP Mode | Corresponding RLC Mode | Traffic Type | Necessary Procedures, Functions and Configurations |
| --- | --- | --- | --- |
| PDCP Mode1 | RLC AM | eMBB with high reliability requirement | Procedures: PDCP status report, Data recovery Functions: In-sequence delivery Configurations: State variables for transmitting/receiving windows |
| PDCP Mode2 | RLC UM | eMBB with low reliability requirement | Functions: In-sequence delivery |
| PDCP Mode3 | RLC AM/UM | URLLC | Procedures: PDCP status report, Data duplication, Data recovery Functions: In-sequence delivery Configurations: State variables for transmitting/receiving windows, Processing delay reduction |
| PDCP Mode4 | RLC UM | mMTC | Functions: In-sequence delivery (optional), Header compression Configurations: Short PDCP SN |

Table 2 PDCP mode for different traffic types

Proposal 1: PDCP transmission mode should be introduced in NR.

Conclusions

In this contribution, we have discussed the necessity of introducing PDCP mode in NR and offered some suggestions. The observations and proposal is listed as follows:

Observation 1: In LTE, the main difference between RLC AM and UM is focused on the reordering operation.

Observation 2:  PDCP performs different procedures, functions and configurations for different RLC modes when reordering is operated at PDCP in NR.

Observation 3: In order to satisfy disparate requirements for different services, PDCP maintains different sets of PDCP procedures, functions and configurations.

Proposal 1: PDCP transmission mode should be introduced in NR.

References

- TS 36.322 Technical Specification Group Radio Access Network: Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Link Control (RLC) protocol specification

- TR 38.804 Study on New Radio Access Technology: Radio Interface Protocol Aspects