3GPP TSG-RAN WG2 #99	R2-1707848

Berlin, Germany, 21st – 25th August 2017	(Resubmission of R2-1707297)

Agenda Item:	10.4.1.3.1

Source:	Ericsson

Title:	Text proposal to RRC connection control

Document for:	Discussion, Decision

# Introduction

This contribution contains a text proposal to RRC connection control including introduction section and initial stubs for RRC Connection Setup, RRC Connection Resume, RRC Connection Release and RRC Connection Suspend procedure.

The text is based on LTE RRC specs. In order to highlight the changes compared to LTE change marks towards 36.331 are used.

In addition, we also propose that the TP is used as baseline to capture further agreements associated to the email discussions on RRC Connection Control triggered in RAN2#98 in Hangzhou.

# Scope of text proposal

The text proposal targets the connection setup and resume procedure and corresponding release and suspend procedure which are most critical and has also been the subject of the email discussion. Other procedure such as paging, initial security setup, RRC re-establishmend etc. can be captured later.

The following principles has been followed:

- Existing text from LTE RRC which also is likely to be applicable for NR has been capture.

- LTE specific topics which has sofar not been discussed for NR has not been captured.

Additionally with the introduction of the new RRC_INACTIVE state it is proposed ot separate the resume/suspend procedure from the connection setup and release procedures.

# Discussion topics

- E-UTRAN (and eNB) is replaced with “the network” to make as future proof solution as possible.

- In the text proposal, some issues are pending the conclusion on bearer harmonization.

- In LTE Resume Request is optional in the network, here it is assumed that the network at least understands the message, meaning UE can always send a resume request in NR.

- Support for UP integrity protection is added.

- Some changes to support RRC_INACTIVE.

- The following aspects are removed for time being compated to 36.331:

  - RN aspects

  - Discussion around critical extensions

# Text proposal 38.331

FIRST CHANGE

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG-C connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful security activation. However, the UE only accepts a handover message when security has been activated.

NOTE:	In case the serving frequency broadcasts multiple overlapping bands, the network can only configure measurements after having obtained the UE capabilities, as the measurement configuration needs to be set according to the band selected by the UE.

Upon receiving the UE context from the 5GC, the network activates security (both ciphering and integrity protection) using the initial security activation procedure. The RRC messages to activate security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered.

After having initiated the initial security activation procedure, the network initiates the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC connection reconfiguration messages used to establish SRB2 and DRBs. the network should release the RRC connection if the initial security activation and/ or the radio bearer establishment fails (i.e. security activation and DRB establishment are triggered by a joint NG-C-procedure, which does not support partial success).

For SRB2 and DRBs, security is always activated from the start, i.e. the the network does not establish these bearers prior to activating security.

After having initiated the initial security activation procedure, the network may configure a UE that supports CA, with one or more SCells in addition to the PCell that was initially configured during connection establishment. The PCell is used to provide the security inputs and upper layer system information (i.e. the NAS mobility information). SCells are used to provide additional downlink and optionally uplink radio resources. When not configured with DC all SCells the UE is configured with, if any, are part of the MCG. When configured with DC however, some of the SCells are part of a SCG. In this case, user data carried by a DRB may either be transferred via MCG (i.e. MCG-DRB), via SCG (SCG-DRB) or via both MCG and SCG in DL while the network configures the CG used in UL (split DRB). An RRC connection reconfiguration message may be used to change the DRB type from MCG-DRB to SCG-DRB or to split DRB, as well as from SCG-DRB or split DRB to MCG-DRB.

Note: The exact handling of DRBs during dual connectivity is FFS pending discussion on bearer harmonization, bearer switching etc.

SCG change is a synchronous SCG reconfiguration procedure (i.e. involving RA to the PSCell) including reset/ re-establishment of layer 2 and, if SCG DRBs are configured, refresh of security. The procedure is used in a number of different scenarios e.g. SCG establishment, PSCell change, Key refresh, change of DRB type. The UE performs the SCG change related actions upon receiving an RRCConnectionReconfiguration message including mobilityControlInfoSCG, see 5.3.10.10.

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an inter-RAT carrier frequency. Upon RRC connection release the UE will enter RRC_IDLE. Only in exceptional cases, as specified within this specification, TS 38.300 [9], TS 38.304 [4] or TS 24.301 [35], may the UE abort the RRC connection, i.e. move to RRC_IDLE without notifying the network.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE AS context and the resumeIdentity, and transitions to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered. Suspension can only be performed when at least 1 DRB is successfully established.

The resumption of a suspended RRC connection is initiated by upper layers when the UE is in RRC_INACTIVE, and the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state. When the RRC connection is resumed, RRC configures the UE according to the RRC connection resume procedure based on the stored UE AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates security and re-establishes SRB(s) and DRB(s). The request to resume the RRC connection includes the resumeIdentity. The request is not ciphered, but protected with a message authentication code.

In response to a request to resume the RRC connection, the network may resume the suspended RRC connection, reject the request to resume and instruct the UE to either keep or discard the stored context, or setup a new RRC connection.

#### 5.3.1.2	Security

AS security comprises of the integrity protection and ciphering of RRC signalling (SRBs) and user data (DRBs). The usage of integrity protection for user data is configured by the network.

RRC handles the configuration of the security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm and two parameters, namely the keyChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon handover, connection re-establishment and/ or connection resume.

The integrity protection algorithm is common for signalling radio bearers SRB1 and SRB2, and for DRBs when configured. The ciphering algorithm is common for all radio bearers (i.e. SRB1, SRB2 and DRBs). Neither integrity protection nor ciphering applies for SRB0.

RRC integrity and ciphering are always activated together, i.e. in one message/ procedure. RRC integrity and ciphering are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (eea0).

The 'NULL' integrity protection algorithm (eia0) is used only for the UE in limited service mode [32, TS33.401]. In case the 'NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity check has failed and indicate the integrity verification check failure to RRC.

NOTE 2:	The behaviour at integrity check failure for DRBs are FFS.

The AS applies four different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc), one for integrity protection of user data (KUPint) and one for the ciphering of user data (KUPenc). All four AS keys are derived from the KgNB key. The KgNB is based on the KASME key, which is handled by upper layers.

Note: If KASME is used or not is pending SA2/SA3 discussion.

Upon connection establishment new AS keys are derived. No AS-parameters are exchanged to serve as inputs for the derivation of the new AS keys at connection establishment.

The integrity and ciphering of the RRC message used to perform handover is based on the security configuration used prior to the handover and is performed by the source gNB.

The integrity and ciphering algorithms can only be changed upon handover. The five AS keys (KgNB, KRRCint, KRRCenc, KUPint and KUPenc) change upon every handover, connection re-establishment and connection resume. The keyChangeIndicator is used upon handover and indicates whether the UE should use the keys associated with the KASME key taken into use with the latest successful NAS SMC procedure. The nextHopChainingCount parameter is used upon handover, connection re-establishment and connection resume by the UE when deriving the new KgNB that is used to generate KRRCint, KRRCenc, KUPint and KUPenc (see TS 33.401 [32]). An intra cell handover procedure may be used to change the keys in RRC_CONNECTED.

For each radio bearer an independent counter (COUNT, as specified in TS 36.323 [8]) is maintained for each direction. For both DRBs and SRBs, the COUNT is used as input for ciphering and integrity protection when applicable. It is not allowed to use the same COUNT value more than once for a given security key. At connection resume the COUNT is reset. In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in TS 36.323 [8]). In addition, an overflow counter mechanism is used: the hyper frame number (TX_HFN and RX_HFN, as specified in TS 36.323 [8]). The HFN needs to be synchronized between the UE and the the network. The the network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same KgNB, e.g. due to the transfer of large volumes of data, release and establishment of new RBs. In order to avoid such re-use, the the network may e.g. use different RB identities for successive RB establishments, trigger an intra cell handover or an RRC_CONNECTED to RRC_IDLE to RRC_CONNECTED transition.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

In case of DC, a separate KgNB is used for SCG-DRBs (S-KgNB). This key is derived from the key used for the MCG (KgNB) and an SCG counter that is used to ensure freshness. To refresh the S-KgNB e.g. when the COUNT will wrap around, the network employs an SCG change, i.e. an RRCConnectionReconfiguration message including mobilityControlInfoSCG. When performing handover, while at least one SCG-DRB remains configured, both KgNB and S-KgNB are refreshed. In such case the network performs handover with SCG change i.e. an RRCConnectionReconfiguration message including both mobilityControlInfo and mobilityControlInfoSCG. The ciphering algorithm is common for all radio bearers within a CG but may be different between MCG and SCG. The ciphering algorithm for SCG DRBs can only be changed upon SCG change.

Note: The exact handling of DRBs during dual connectivity is FFS pending discussion on bearer harmonization, bearer switching etc.

NEXT CHANGE

### 5.3.A	RRC connection establishment

#### 5.3.A.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1  establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure as follows:

-	When establishing an RRC connection:

-	to establish SRB1

NEXT CHANGE

### 5.3.B	RRC connection resume

#### 5.3.B.1	General

Figure 5.3.3.1-3: RRC connection resume, successful

Figure 5.3.3.1-4: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.3.1-5: RRC connection resume, network reject or release

The purpose of this procedure is to resume an RRC connection including AS security. RRC connection resume involves SRB1, SRB2 and DRB re-establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure as follows:

-	When resuming an RRC connection:

-	to restore the AS configuration from a stored context including resuming SRB(s) and DRB(s).

NEXT CHANGE

### 5.3.C	RRC connection release

#### 5.3.C.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	 to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources;

NEXT CHANGE

### 5.3.D	RRC connection suspend

#### 5.3.D.1	General

Figure 5.3.8.1-1: RRC connection suspend, successful

The purpose of this procedure is:

-	to suspend the RRC connection, which includes the suspension of the established radio bearers.