3GPP TSG-RAN WG2 #99                                                                                     R2-1707894

Berlin, Germany, 21 - 25 Aug 2017

Source:	CATT

Title:	Discussion on QoS DC offloading

Agenda Item:	10.2.13

Document for:	Discussion and Decision

# Introduction

In the last RAN2#NR ADHOC6 meeting, the following agreements were made:

Agreements

1	At SN addition and at new PDU session establishment then MN makes the decision which QoS flows are moved SN

FFS Whether the SN can reject the movement of a QoS flow.

2	Irrespective of which node makes the decision of where a QoS flow is mapped (to MN or SN) then RAN2 will aim that the RRC signalling is the same.

Agreements

1: The MN makes the decision to move ongoing/existing QoS flows to the SN (this agreement does not imply whether the QoS flow is moved by moving a single flow or by moving a whole bearer)

FFS Whether MN or SN takes the decision for flows being moved from SN to MN

2: The SN can reject the addition of a QoS flow, and inform the MN.

3: The DRB level offloading (i.e. offloading all QoS flows of a DRB) is supported between the MN and SN.

FFS: The QoS flow level offloading between the MN and SN, and if supported then whether lossless handover can be supported.

4: The lossless handover user plane procedure could be reused for DRB level offloading, if mapping is maintained in the target node.

FFS: If the case where mapping is not maintained can support lossless handover

5: The SN is responsible for the DRB management  (e.g., setup, modify, release) of SCG/SCG-split bearers, and the QoS flow -> DRB mapping at the SN

This paper discusses the open issues listed in the last meeting.

# Discussion

## The QoS flow level offloading between the MN and SN, and if supported then whether lossless handover can be supported.

This open issue should be discussed in the following two scenarios:

Scenario 1: during the initial setup of a new PDU session, whether some of the QoS flows can be routed via MN, and other QoS flows are routed via SN

For the first scenario, which is actually in the phase of admission control, since we had agreement that at new PDU session establishment MN makes the decision which QoS flows are moved SN, so MN should decide which QoS flows should be loaded by MN and which QoS flows should be routed via SN. Thus we propose that when a new PDU session is being established, QoS flow is allowed to be offloaded to SN up to the decision of MN.

When a new QoS flow is arrival, it shall arrive to MN first, and MN decides whether this QoS flow should be loaded via MN or SN.

Proposal 1: whether a QoS flow of a new PDU session which is being established or a new QoS flow is joining an ongoing PDU session is allowed to be offloaded to SN is up to the decision of MN.

Scenario 2: after the PDU session is established, whether QoS flow level offloading between the MN and SN is allowed.

This scenario is if the MN is allowed to offload an ongoing QoS flow to MN.  I will give an example for this case. In Handover case, we have the common understanding that re-mapping is not supported, the source gNB forward the mapping to the target gNB, the target gNB accepts all the DRB with the original mapping between DRB and QoS flow. Inter- node offloading is very much like the handover case, the target node should accept all the DRB with the same mapping. Likewise, if a QoS flow is not supported from a node to another node in handover, why do we support a QoS flow is remapped from MN to SN or from SN to MN?

Another method as an gNB implementation approach is to change a SCG bearer to a SCG split bearer, since in SCG split bearer case, the UE has one PDCP entity, lossless remapping can be guaranteed.

Proposal 2: ongoing QoS flow level movement is not supported.

## Whether MN or SN takes the decision for flows being moved from SN to MN

Based on the discussion of 2.1, we will discuss whether MN or SN takes the decision for DRBs being moved from SN to MN.

For this case, we think SN can initial request to offload DRBs from SN to MN, according to implementation of SN, SN may figure out the radio resource is not sufficient for the time being. At that moment, SN is able to request an offload of an ongoing DRB from SN to MN. And as the master node, MN can decide whether to accept the request or reject.

Proposal 3: SN can request moving an ongoing DRB to MN, MN can decides to accept or reject.

## If the case where mapping is not maintained can support lossless handover

If the mapping rules are not maintained during the handover, in order to ensure lo lossless handover, source gNB and the target gNB should be enhanced to exchange the transmission status, and develop a data forwarding mechanism. Given the progress of this release, supporting changed mapping rules during handover doesn’t bring any gain, so we suggest only support unchanged mapping rules during handover in this release.

gNB is free to update the mapping rule when the transmission is over after the handover is completed, then lossless handover can be ensured by this gNB implementation. Other optimizations to ensure lossless handover of changed mapping rules should not be discussed in this release.

Proposal 4: Only unchanged mapping rules to ensure lossless handover during handover is supported as least in this release.

## UE side SDAP entity number in case of CN split

In the previous discussion of SDAP, we have the agreement that SDAP entity is configured per PDU session.

However, in case of QoS flow in DC, a PDU session may be split in two nodes, part of the QoS flows are loaded by MN, and the other of the QoS flows are loaded by MN. So here comes to the question: the UE side should configure 1 or 2 SDAP entities in case of QoS in CN split?

If there is only 1 SDAP entity in the UE side, however there are two SDAP entities in the network side, one in MN and the other one in SN. In this scenario of deployment, this SDAP entity in the UE should decide how to select MN entity or SN entity to transmit SDAP packets.

So if UE configures two SDAP entities, each UE SDAP entity is associated to MN SDAP entity, the other UE SDAP entity is associated to MN SDAP entity. If reflective QoS is configured, UE NAS layer delivers the IP packets to the reflected UL QoS flow, and from the QoS flow to the reflected DRB. If reflective QoS is not configured, UE put the UL IP packet in the QoS flow by default IP filter, and place the QoS flow to the default DRB.

Figure 1: two SDAP entities in UE side

If there are two SDAP entities, there should be only one default DRB. If there is one default DRB in MN, another default DRB in SN, how the UE select default DRB has an unpredicted risk.

Proposal 5: there are two SDAP entities configured in UE in case a PDU session is split in MN and SN.

Proposal 6: there is always only one default DRB even a PDU session is split in MN and SN.

# Conclusion

This paper discusses the QoS flow mapping issue for both uplink and downlink, and we have the following observations and proposals:

Proposal 1: whether a QoS flow of a new PDU session which is being established or a new QoS flow is joining an ongoing PDU session is allowed to be offloaded to SN is up to the decision of MN.

Proposal 2: ongoing QoS flow level movement is not supported.

Proposal 3: SN can request moving an ongoing DRB to MN, MN can decides to accept or reject.

Proposal 4: Only unchanged mapping rules to ensure lossless handover during handover is supported as least in this release.

Proposal 5: there are two SDAP entities configured in UE in case a PDU session is split in MN and SN.

Proposal 6: there is always only one default DRB even a PDU session is split in MN and SN.