3GPP TSG-RAN WG2 #99                                                                                     R2-1707896

Berlin, Germany, 21 - 25 Aug 2017		Resubmission of R2-1706398

Source:	CATT

Title:	RRC connection re-establishment and resume procedures in NR

Agenda Item:	10.4.1.3.2

Document for:	Discussion and Decision

Introduction

In RAN2#97bis meeting, some guidelines are agreed for the RRC messages and procedures [1].

Agreement 1 Aim to limit the number of RRC messages i.e. avoid introducing several messages with similar content/ similar procedural handling (details can be discusses when more progress has been made on the individual procedures)

This contribution discusses the details of individual procedures for RRC connection re-establishment and resume.

Discussion

RRC connection re-establishment

In LTE, The purpose of this procedure is to re-establish the RRC connection, which involves the resumption of SRB1 operation, the re-activation of security and the configuration of only the PCell.

Figure 1 RRC connection re-establishment, successful

Figure 2 RRC connection re-establishment, failure

The function of re-establishing the RRC connection should naturally be supported in NR for UE in connected mode, including the successful and failure procedures.

Proposal 1: Support the successful and failure procedures for RRC connection re-establishment in NR.

The content of the RRC messages for RRC connection re-establishment are listed in the table 1.

Table 1 content of the RRC messages for RRC connection re-establishment

| RRC message | Content in LTE | Proposed Content in NR |
| --- | --- | --- |
| RRCConnectionReestablishmentRequest | - ue-Identity - reestablishmentCause | - ue-Identity - reestablishmentCause |
| RRCConnectionReestablishment | - rrc-TransactionIdentifier - radioResourceConfigDedicated (for SRB1) - NextHopChainingCount | - rrc-TransactionIdentifier - radioResourceConfigDedicated (for SRB1) - NextHopChainingCount |
| RRCConnectionReestablishmentComplete | - rrc-TransactionIdentifier - rlf-InfoAvailable-r9 - logMeasAvailable-r10 - connEstFailInfoAvailable-r11 - logMeasAvailableMBSFN-r12 | - rrc-TransactionIdentifier |
| RRCConnectionReestablishmentReject | - No IE | - No IE |

For RRCConnectionReestablishmentRequest, RRCConnectionReestablishment and RRCConnectionReestablishmentReject messages, all parameters for LTE are also needed in NR.

For RRCConnectionReestablishmentComplete message, the IEs rlf-InfoAvailable-r9, logMeasAvailable-r10, connEstFailInfoAvailable-r11 and logMeasAvailableMBSFN-r12 are related to MDT/SON which are not supported in NR phase I, so only IE rrc-TransactionIdentifier is needed in NR.

The RRC connection re-establishment procedure is used for SRB1 resumption and re-activation of security and the configuration of only the PCell, so the following parameters are proposed to support.

Proposal 2: Support the following parameters for the RRC messages in RRC connection re-establishment procedure, the details of each parameter can be discussed further:

- RRCConnectionReestablishmentRequest: UE ID, Re-establishment Cause

- RRCConnectionReestablishment: Transaction ID, dedicated radio resource configuration for SRB1, NCC

- RRCConnectionReestablishmentComplete: Transaction ID

RRC connection resume

In LTE, the purpose of this procedure is to resume an RRC connection, which involves the resumption of all SRBs and DRBs, and the re-activation of security.

Figure 3 RRC connection resume, successful

Figure 4 RRC connection resume fallback to RRC connection establishment, successful

Figure 5 RRC connection resume, network reject or release

In RAN2#98, RAN2 agreed that as to the case that the UE wants to transition from INACTIVE to CONNECTED, “Initial UE RRC message from RRC_INACTIVE (e.g. MSG3) should be sent on SRB0” , and “If the UE received a resume message on MSG4 on SRB1 then the UE enters RRC Connected”[2]. Refer to light connection where Modelling A and Modelling B both include RRC resume procedure, we expect that the function of resuming the RRC connection should also be supported in NR for UE in inactive mode, including the successful resume, fallback to RRC connection establishment, network reject and release procedures. As to the successful resume, we should follow light connection and reuse the existing RRC resume mechanism/message with necessary add/delete. RAN2 agreed that ”FFS In case RAN is not successful in retrieving or verifying the UE context, MSG4 (can be at least be a message that requests the UE to trigger a new connection) will be sent on SRB0”, we could follow light connection and allow fallback to RRC connection establishment, which is decided by gNB. Moreover, RAN2 agree that “If the UE received a message suspending the UE on MSG4 on SRB1 then the UE remains in RRC Inactive”, it can be realized by network reject or release procedure. In addition, RRC release procedure could be used to move the UE to idle state.

Proposal 3: Support the successful resume, fallback to RRC connection establishment, network reject and release procedures for RRC connection resume in NR.

The content of the RRC messages for RRC connection resume are listed in the table 1.

Table 2 content of the RRC messages for RRC connection resume

| RRC message | Content in LTE | Proposed Content in NR |
| --- | --- | --- |
| RRCConnectionResumeRequest | - resumeIdentity-r13 - shortResumeMAC-I-r13 - resumeCause-r13 | - resumeIdentity-r13 - shortResumeMAC-I-r13 - resumeCause-r13 |
| RRCConnectionResume | - rrc-TransactionIdentifier - radioResourceConfigDedicated-r13(for all SRBs and DRBs) - nextHopChainingCount-r13 - measConfig-r13 - antennaInfoDedicatedPCell-r13 - drb-ContinueROHC-r13 | - rrc-TransactionIdentifier - radioResourceConfigDedicated-r13(for all SRBs and DRBs) - nextHopChainingCount-r13 - measConfig-r13 - antennaInfoDedicatedPCell-r13 - drb-ContinueROHC-r13 |
| RRCConnectionResumeComplete | - rrc-TransactionIdentifier - selectedPLMN-Identity-r13 - dedicatedInfoNAS-r13 - rlf-InfoAvailable-r13 - logMeasAvailable-r13 - connEstFailInfoAvailable-r13 - mobilityState-r13 - mobilityHistoryAvail-r13 - logMeasAvailableMBSFN-r13 | - rrc-TransactionIdentifier - selectedPLMN-Identity-r13 - dedicatedInfoNAS-r13 |
| RRCConnectionReject | - rrc-SuspendIndication-r13 | - rrc-SuspendIndication-r13 |

For RRCConnectionResumeRequest, RRCConnectionResume and RRCConnectionReject messages, all parameters for LTE are also needed in NR. As to RRCConnectionReject message, although in light connection a new rrc-LightConnectionIndication is introduced to indicate that the UE should remain lightly connected and not release its stored context if present [3], whether MSG 4 can be a reject to idle is for further study in case of inactive state. If it is agreed that MSG 4 can be a reject to idle afterwards, this kind of indication should be added in Table 2.

For RRCConnectionResumeComplete message, the IEs rlf-InfoAvailable-r13, logMeasAvailable-r13, connEstFailInfoAvailable-r13 and logMeasAvailableMBSFN-r13 are related to MDT/SON which are not supported in NR phase I. The IEs mobilityState-r13 and mobilityHistoryAvail-r13 are related to UE mobility history which is not supported in NR phase I. So only IEs rrc-TransactionIdentifier, selectedPLMN-Identity-r13 and dedicatedInfoNAS-r13 are needed in NR.The RRC connection resume procedure is used for all SRBs and DRBs resumption and re-activation of security and, so the following parameters are proposed to support.

Proposal 4: Support the following parameters for the RRC messages in RRC connection resume procedure, the details of each parameter can be discussed further:

- RRCConnectionResumeRequest: Resume ID, Resume Cause

- RRCConnectionResume: Transaction ID, dedicated radio resource configuration for all SRBs and DRBs, NCC, measurement Configuration, antenna info, Continue ROHC for DRB

- RRCConnectionResumeComplete: Transaction ID, selected PLMN ID, dedicated NAS info.

- RRCConnectionReject: Suspend Indication

Comparing of RRC connection re-establishment and resume

Comparison can be made based on the analysis of section 2.1 and 2.2. The details are shown below:

- RRC re-establishment is used for connected mode UE, and RRC resume is used for inactive mode UE. In these two cases, the network both has UE’s AS context.

- They are both used to re-active the security.

- RRC re-establishment can resume SRB1, and RRC resume can resume all SRBs and DRBs.

- From the procedure point of view, the successful and reject procedures are similar. The RRC resume procedure has a fallback mechanism to RRC connection establishment, which is not supported by RRC re-establishment.

- From the content point of view:

  - both RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest have UE ID and cause, although the details are not the same.

  - Both RRCConnectionReestablishment and RRCConnectionResume have Transaction ID, dedicated radio resource configuration and NCC, and the latter has more parameters for other purpose.

  - Both RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete have Transaction ID, and the latter has more parameters for other purpose.

  - For reject case, only reject message for resume procedure has a Suspend Indication.

So there are many commonalities between RRC connection re-establishment procedure and RRC connection resume procedures. Thus it should be aim to combine the RRC messages used for RRC connection re-establishment procedure and RRC connection resume procedure.

Proposal 5: Aim to combine the following RRC messages for NR:

- RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest

- RRCConnectionReestablishment and RRCConnectionResume

- RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete

- RRCConnectionReestablishmentReject and RRCConnectionReject

Conclusion

In this contribution, we discuss the individual procedures for RRC connection re-establishment and resume, and propose:

Proposal 1: Support the successful and failure procedures for RRC connection re-establishment in NR.

Proposal 2: Support the following parameters for the RRC messages in RRC connection re-establishment procedure, the details of each parameter can be discussed further:

- RRCConnectionReestablishmentRequest: UE ID, Re-establishment Cause

- RRCConnectionReestablishment: Transaction ID, dedicated radio resource configuration for SRB1, NCC

- RRCConnectionReestablishmentComplete: Transaction ID

Proposal 3: Support the successful resume, fallback to RRC connection establishment, network reject and release procedures for RRC connection resume in NR.

Proposal 4: Support the following parameters for the RRC messages in RRC connection resume procedure, the details of each parameter can be discussed further:

- RRCConnectionResumeRequest: Resume ID, Resume Cause

- RRCConnectionResume: Transaction ID, dedicated radio resource configuration for all SRBs and DRBs, NCC, measurement Configuration, antenna info, Continue ROHC for DRB

- RRCConnectionResumeComplete: Transaction ID, selected PLMN ID, dedicated NAS info.

- RRCConnectionReject: Suspend Indication

Proposal 5: Aim to combine the following RRC messages for NR:

- RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest

- RRCConnectionReestablishment and RRCConnectionResume

- RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete

- RRCConnectionReestablishmentReject and RRCConnectionReject

Reference

[1] RAN2#97bis, Chairman notes.

[2] RAN2#98, Chairman notes.

[3] R2-1701689, Light Connection Feature specification in 36.331 - TP agreements and outcome of email discussion [96#66], RAN2#97.