3GPP TSG-RAN WG2 #99	                                                             	R2-1707903

Berlin, Germany, 21 - 25 Aug 2017		Revision of R2-1706403

Source:	CATT

Title:	Inter-RAT measurement for NR

Agenda Item:	10.4.1.4.5

Document for:	Discussion and Decision

Introduction

In the NR WID [1], it is specified the dual connectivity is aimed to be supported as followings:

This work item is aimed at supporting the following connectivity options (*): For single connectivity option: - NR connected to 5G-CN (Option 2 in TR 38.801 section 7.1). For Dual Connectivity options: - E-UTRA-NR DC via EPC where the E-UTRA is the master (Option 3/3a/3x in TR 38.801 section 10.1.2); - E-UTRA-NR DC via 5G-CN where the E-UTRA is the master (Option 7/7a/7x in TR 38.801 section 10.1.4); - NR-E-UTRA DC via 5G-CN where the NR is the master (Option 4/4A in TR 38.801 section 10.1.3).

In RAN2 NR Ad Hoc#2 [2], the inter-RAT measurement for EN-DC was discussed, and the following agreements were made:

Agreements

1	LTE inter-RAT measurement trigger event B1 for NR to be supported

FFS Whether a measurement trigger event is needed in LTE which is triggered when NR PSCell becomes worse than an absolute threshold.

FFS A measurement trigger event is needed in LTE which is triggered when NR PSCell becomes worse than absolute threshold1 and NR neighbour cell on a different carrier becomes better than absolute threshold2.

This contribution discusses the inter-RAT measurement issues that impact the support of standalone NR and NR-E-UTRA DC via 5G-CN.

Discussion

Considering the SA scenario, for single connectivity (NR connected to 5G-CN), the handover from NR to LTE can be supported when EPC and 5G-CN has interface, or LTE connected to 5G-CN. So to support this scenario, the inter-RAT measurement event of B1 and B2 is needed.

Proposal 1: For NR specification, introduce the following NR inter-RAT measurement events for LTE:

- Event B1:  Neighbour becomes better than absolute threshold;

- Event B2:  PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

For NR-E-UTRA DC via 5G-CN where the NR is the master, the inter-RAT measurement events are needed to be considered.

For SeNB change, SeNB release, the measurement configuration can be done by SeNB itself or by MgNB depending on the scenario. If SeNB configures the measurement, it is intra-LTE measurement, the event A1~A6 can be used. If MgNB configure the inter-RAT measurement, the following NR inter-RAT measurement events can be considered for NE-DC:

Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2 (used for LTE SeNB change)

Event D2: All LTE SCG cells become worse than a threshold (used for LTE SeNB release)

So it is proposed:

Proposal 2: Introduce the following NR inter-RAT measurement event for LTE SeNB change initiated by NR MgNB:

- Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2.

Proposal 3: Introduce the following NR inter-RAT measurement event for LTE SeNB release initiated by NR MgNB:

- Event D2: All LTE SCG cells become worse than a threshold.

Considering the periodical inter-RAT measurement reporting in NR, it can be used for the ANR/SON feature, with LTE as the candidate measurement object. The purpose of periodical reporting can be reportStrongestCellsForSON and reportCGI.

Proposal 4: Introduce periodical inter-RAT measurement reporting in NR for ANR/SON feature.

Conclusion

In this contribution, we discuss the inter-RAT measurement and propose:

Proposal 1: Introduce the following NR inter-RAT measurement events:

- Event B1:  Neighbour becomes better than absolute threshold;

- Event B2:  PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

Proposal 2: Introduce the following NR inter-RAT measurement event for LTE SeNB change initiated by NR MgNB:

- Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2.

Proposal 3: Introduce the following NR inter-RAT measurement event for LTE SeNB release initiated by NR MgNB:

- Event D2: All LTE SCG cells become worse than a threshold.

Proposal 4: Introduce periodical inter-RAT measurement reporting in NR for ANR/SON feature.

Reference

[1] RP-170855, New WID on New Radio Access Technology

[2] RAN2 NR Ad Hoc#2, Chairman notes.