3GPP TSG-RAN WG2 #99	R2-1707981

Berlin, Germany, 21 - 25 August 2017

Agenda item:	10.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On use of LTE PDCP and NR PDCP in MR-DC

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution addresses the following open points identified by the previous RAN2 meeting.

Working assumption: For MCG bearer, either LTE or NR PDCP can be used,  configurable by the network.

FFS points:

1) which PDCP to use for MCG SRB at connection setup.

2) What mechanism is used (if needed) to indicate to network UE support of NR PDCP during connection setup?

3) whether to use LTE PDCP or NR PDCP for split SRBs

4) Whether to support a mechanism to reconfigure from LTE PDCP to NR PDCP without HO.  If so, what would the mechanism look like?

# 2	Discussion

Working assumption: For MCG bearer, either LTE or NR PDCP can be used,  configurable by the network.

If only LTE PDCP could be used for an MCG bearer, lossless bearer-type change from split bearer to MCG bearer would require to support a PDCP re-establishment procedure from NR PDCP into LTE PDCP. There seems to be no other compelling need for such a procedure.

If only NR PDCP could be used for an MCG bearer, once EN-DC is configured for a UE, all its MCG bearers running on LTE PDCP would need to be converted to NR PDCP. For Voip bearers running on RLC UM, if such conversion is done by PDCP re-establishment, unnecessary packet losses can occur. If packet losses are to be avoided, a new dedicated-purpose PDCP procedure other than PDCP re-establishment would be needed for such conversion. It would be attractive to avoid the need for such a procedure.

Proposal 1:	Confirm the working assumption that for an MCG bearer in EN-DC, either LTE or NR PDCP can be used, configurable by the network.

1) which PDCP to use for MCG SRB at connection setup.

2) What mechanism is used (if needed) to indicate to network UE support of NR PDCP during connection setup?

Except for LTE connection to NGC, it seems simplest to have the initial MCG SRBs be based on the PDCP native to the RAT in question. For LTE connection to NGC, it seems preferable to only rely on NR PDCP: the indication of UE support of NGC at connection set-up allows selecting NR PDCP for the MCG SRB.

Proposal 2:	At connection set-up, the MCG SRB uses LTE PDCP if and only if the connection is to EPC.

Proposal 3:	No explicit indication of UE support of NR PDCP is needed at connection set-up.

3) whether to use LTE PDCP or NR PDCP for split SRBs

Split SRBs require reordering from PDCP, which is a native feature of NR PDCP. On the other hand, use of LTE PDCP for the purpose would require introduction of reordering for SRBs in 36.323. It seems attractive to avoid such modification.

Proposal 4:	NR PDCP is used for split SRBs

4) Whether to support a mechanism to reconfigure from LTE PDCP to NR PDCP without HO.  If so, what would the mechanism look like?

Our understanding of this FFS is that whether or not handover is involved determines whether or not all security keys of the UE change. (From PDCP point of view, there is no handover, only the re-establishment procedure.)

Considering e.g. a bearer-type change from MCG to split bearer, there seems to be a need to support a reconfiguration from LTE PDCP to NR PDCP that does not involve changing all keys of the UE. If the PDCP-SN length changes in the process, in line with the recent RAN2 agreement, such a reconfiguration is most likely done via DRB release and re-setup; for other cases, a PDCP re-establishment procedure from LTE PDCP to NR PDCP seems needed. The remaining question is whether an even lighter procedure is needed for such a conversion, which would not involve change of any security key. As discussed above Proposal 1, we prefer to avoid introducing such a procedure.

Proposal 5:	Apart from DRB release and re-setup, LTE PDCP can be reconfigured to NR PDCP only via a PDCP re-establishment defined for such a conversion. Such a reconfiguration does not require PDCP re-establishment for any other bearer of the UE.

# 3	Conclusion

This contribution addresses open points on use of LTE PDCP and NR PDCP in MR-DC, and proposes the following.

Proposal 1:	Confirm the working assumption that for an MCG bearer in EN-DC, either LTE or NR PDCP can be used, configurable by the network.

Proposal 2:	At connection set-up, the MCG SRB uses LTE PDCP if and only if the connection is to EPC.

Proposal 3:	No explicit indication of UE support of NR PDCP is needed at connection set-up.

Proposal 4:	NR PDCP is used for split SRBs

Proposal 5:	Apart from DRB release and re-setup, LTE PDCP can be reconfigured to NR PDCP only via a PDCP re-establishment defined for such a conversion. Such a reconfiguration does not require PDCP re-establishment for any other bearer of the UE.