3GPP TSG-RAN WG2 #99	R2-1707986

Berlin, Germany, 21 - 25 August 2017

Agenda item:	10.2.13

Source:	Nokia, Nokia Shanghai Bell

Title:	Offloading granularity in NR-NR Dual Connectivity

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

RAN2 NR AH#2 meeting discussed DRB management in NR-NR dual connectivity and agreed that DRB level traffic offloading is supported between the MN (Master Node) and SN (Secondary Node)  [Draft_Report_RAN2_NR_AH#2_Qingdao_v1]. QoS flow level offloading was however left for further study. This contribution introduces a scenario that requires such offloading.

# 2	Discussion

As already introduced in [R2-1706546], traffic offloading can be performed at two levels:

1.	DRB level: complete DRB(s) are moved from MN to SN; some DRBs of a PDU session may remain in MN.

2.	QoS flow level: individual QoS flow(s) are offloaded to SN; some QoS flows may remain at MN, in existing DRBs. The DRB(s) in SN are essentially new one(s) i.e. DRBs are not relocated from MN, which makes the QoS flow level offloading equivalent to QoS flow relocation.

When an UL QoS flow, for which the UE does not have matching UL -filter, appears, the UE maps it to a default DRB [38.300]. A CN (Core Network) in turn forwards a new DL QoS flow to the MN.  If MN policy is to offload new QoS flows to the SN it basically has two options:

- Perform a DRB level offloading: which requires 1) establishment of a new DRB at MN for the new QoS flow; 2) relocation of the new QoS flow into the new DRB; and 3) offloading the new DRB of the new QoS flow to SN.

- Perform a QoS flow level offloading, which means that the new QoS flow is directly relocated to the SN from the default DRB.

Both approach includes a QoS flow remapping to a different DRB. In option a) the QoS flow is relocated to a new DRB in the same cell while in option b) the QoS flow is relocated to a DRB in the SN. Option a) necessitates also DRB establishment procedure in the MN for the new QoS flow, which is not needed in option b). Option b) is therefore preferred since the unnecessary DRB establishment (RRC reconfiguration transaction) can be avoided.

Proposal 1: QoS flow level traffic offloading should be supported in NR-NR dual connectivity.

# Conclusion

This contribution has discussed offloading granularity and has concluded the following:

Proposal 1: QoS flow level traffic offloading should be supported in NR-NR dual connectivity.