3GPP TSG-RAN WG2 #99	R2-1707992

Berlin, Germany, 21 - 25 August 2017

Agenda item:	10.3.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RQoS indication to UE

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

A principle of two level data mapping to DRBs is adopted in a 5G system. User data is first mapped to QoS flows on a NAS layer and in a second phase QoS flows are mapped to DRBs on an AS layer. Reflective QoS (RQoS) can be utilized on both layers for configuring UL mapping rules.  This contribution discusses how the use of the RQoS can be indicated to an UE.

# 2	NAS layer RQoS

A chapter 5.7.5.4 of [TS 23.501] specifies the control of NAS level RQoS as follows:

“Reflective QoS is controlled on per-packet basis by using the RQI in the encapsulation header on N3 reference point together with the QFI, together with a Reflective QoS Timer (RQ Timer) value that is either signaled to the UE upon PDU Session establishment or set to a default value.

When the 5GC determines to use Reflective QoS for a specific SDF, the SMF shall

in the case of NG-RAN, include a Reflective QoS Attribute (RQA) in the QoS flow parameters which are sent to the RAN via N2; and

-	include an indication in the corresponding SDF information provided to the UPF via N4 interface.

For DL packets corresponding to this SDF, the UPF sets the RQI bit in the encapsulation header on the N3 reference point.

When an RQI is received by (R)AN in a DL packet on N3 reference point, the (R)AN shall indicate to the UE the QFI and that this DL packet is subject to Reflective QoS.”

The TS 23.501 specifies how the use of RQoS is indicated to RAN with the RQI bit in the N3 encapsulation header, but it leaves open how it is indicated to the UE over a radio interface. Since an idea of the RQoS is to avoid C-plane signalling in UL mapping rule configuration and NAS layer RQoS is performed on packet basis, only reasonable solution to indicate NAS layer RQoS to the UE is an indicator in user plane radio packet headers.

Proposal 1: Use of NAS layer RQoS is indicated to the UE by a one bit flag in a SDAP header.

# 3	AS layer RQoS

A RAN2 made following agreements in a NR AH#2 meeting about control of AS layer RQoS [Draft_Report_RAN2_NR_AH#2_Qingdao_v1].

- There is a need to tell the UE that it has to update the mapping rule.   For the AS RQoS it is up to the RAN to decide when to update the mapping rules.  FFS on the details of the header format.

- It is up to the RAN to decide when and which mechanism, explicit RRC re-configuration and/or AS RQoS, should be used to provide mapping information to the UE.

The agreements imply that RQoS on AS layer is controlled by the RAN and decoupled from the use of RQoS on NAS layer, therefore separate indication to the UE for AS layer RQoS is needed. As NAS layer RQoS, AS layer RQoS should be configurable on packet basis, which means that the indication must be included in U-plane packets, the SDAP frame (as a QoS layer) being a natural place.

Four different combinations with two layers RQoS can be identified.

1.	Both NAS and AS -layers do RQoS;

2.	Neither NAS, nor AS -layers do RQoS;

3.	AS layer does RQoS, but NAS layer does not;

4.	NAS layer does RQoS, but AS layer does not.

All combinations can be indicated to UE with two indicator bits, each layer specific bit indicating whether to perform RQoS on a layer.

Observation 1: All NAS/AS layer RQoS combinations can be indicated with two indicator bits in a SDAP header.

However, since the QFI is needed only in packets that are subject to RQoS (either at NAS or AS), the presence of the QFI itself could be used as an indicator to save one bit. With the RQI and the QoS flow ID following RQoS combinations can be indicated to the UE.

2.	QFI omitted → neither AS nor NAS do RQoS;

1.	QFI included, RQI = 1 → Both NAS and AS layers do RQoS;

3.	QFI included, RQI = 0 → AS layer does RQoS, but NAS layer does not.

NOTE 1:	This is not about transparent SDAP mode but omitting the QFI from the SDAP frame.

NOTE 2: 	The feasibility of the QFI omission from the SDAP header depends on length of the QFI, more precisely if QFI fits into first SDAP octet or does it necessitate additional octet. SA2 has not yet decided the length of the QFI.

Only combination 4 (NAS layer does RQoS, but AS layer does not) cannot be indicated to the UE, and thus whenever NAS performs RQoS, so does AS. Whether it is a problem depends on the need to support asymmetric QoS flow to DRB mapping between UL and DL. If asymmetric mapping needs to be supported on AS layer, UE must never perform RQoS, otherwise the mapping becomes symmetric. If symmetric mapping is always assumed, doing RQoS at AS and NAS always does not harm. The only downside is that the AS layer filter determination must be performed by the UE always when NAS layer reflective QoS is activated even if the AS layer mapping is not changing.

NOTE 3:	Symmetric mapping on AS layer does not prevent providing asymmetric QoS for SDFs, since asymmetric QoS can be enforced in NAS layer mapping.

Observation 2: By using the QoS flow ID (QFI) as a RQoS indicator in addition to one indicator flag, 3 out of 4 RQoS combinations can be indicated to the UE.

Proposal 2: It’s proposed to discuss if the asymmetric QoS flow to DRB mapping between UL and DL is required on AS layer and if the AS layer RQoS requires dedicated indicator bit in the SDAP header.

# Conclusion

Proposal 1: Use of NAS layer RQoS is indicated to the UE by a one bit flag in a SDAP header.

Observation 1: All NAS/AS layer RQoS combinations can be indicated with two indicator bits in a SDAP header.

Observation 2: By using the QoS flow ID (QFI) as a RQoS indicator in addition to one indicator flag, 3 out of 4 RQoS combinations can be indicated to the UE.

Proposal 2: it is proposed to discuss if the asymmetric QoS flow to DRB mapping between UL and DL is required on AS layer and if the AS layer RQoS requires dedicated indicator bit in the SDAP header.