3GPP TSG-RAN WG2 Meeting #99	Tdoc R2-1708032

Berlin, Germany, 21st – 25th August 2017	(update of R2-1706577)

Agenda Item:	10.2.6.3 – NR – Stage-2 – UE capabilities - Coordination

Source:	Ericsson

Title:	UE capability coordination for LTE-NR interworking

Document for:	Discussion, Decision

# Introduction

RAN2 discussed the capability coordination for LTE/NR interworking and made a couple of agreements and assumptions which are summarized in section 5 of this document. It is however still “FFS whether LTE/NR DC specific capabilities requiring coordination between eNB and gNB are included in NR-Capability or LTE Capability or a new LTE/NR-Capability container”.

There have been proposals to keep LTE capabilities (e.g. band combinations) in the UE-EUTRA-Capability IE, to specify the corresponding NR capabilities (band combinations) in the UE-NR-Capability IE and to let the UE indicate by means of a two-dimensional matrix or by pointers which combination of NR- and LTE band combinations is supported. Such a cross-referencing approach appears to have several advantages but as we explain in this document, we also see a number of drawbacks when peeking into the details.

# Discussion

UE capability reporting for LTE DC was built on top of the same structure used for carrier aggregation. MeNB and SeNB used that same UE capability IE and exchanged and comprehended each other’s configuration for the UE.

For LTE-NR tight interworking, reporting and coordination both become more complex, since it involves two RATs, so it will not be possible to reuse an existing structure, as was done for DC. And because of the fairly complex UE capability structures of LTE, with feature support signalled per supported band combination, it would be preferable not to introduce the same complexity in NR. So far RAN2 has agreed that UE capabilities requiring coordination include band combinations across LTE and NR and L2 buffer size. An LS has been sent to RAN1 and RAN4 to get input on further capabilities requiring coordination [2]. The received feedback is of little help but this should not stop RAN2 from designing the NR capability signalling and the EN-DC capability coordination.

## Band Combinations

Due to limitations in UE RF design, the UE needs to be able to indicate to the network which LTE band combinations can be configured together with which NR band combinations. The selected band combination of MCG and SCG will be reflected in the MCG configuration and the SCG configuration respectively.

It has been proposed in [1] to introduce abstract coordination for the coordination of band combinations, where a matrix is proposed to indicate compatible band combinations across the two RATs. Instead of a matrix the UE-EUTRA-Capability IE specified in 36.331 would point (e.g. with an index) from E-UTRA band combinations to corresponding NR band combination(s) that can be simultaneously configured for LTE/NR DC. Vice versa NR band combinations would refer to compatible EUTRA band combination.

Generally, the abstract coordination of UE capabilities appears attractive since the UE anyway lists its band combinations for single RAT operation. Hence, the “only” overhead is the matrix or the additional pointers/indexes.

However, if LTE and NR share RF or baseband components in the UE, the band combination entries applicable for single-RAT operation will likely not be applicable for dual-connectivity mode. That means, the “lowest” fallback band combination for a certain (set of) band(s) included by the LTE-only UE will likely not be supported when this chipset runs also NR (i.e., EN-DC).

This applies in particular for the interFreqNeedForGaps which is likely affected if the UE is additionally configured with an NR carrier. For examples, a UE with two RFs that support a carrier on a Band X and a carrier on Band Y respectively, will usually support measurement on Band Y without gaps when configured only with a serving cell on Band X. Hence, the UE would indicate in its LTE single-band band combination for Band X that it can perform inter-frequency measurements on Band Y without gaps. However, if the UE would be configured with an additional NR serving cell (PSCell) on Band Y, it could most likely not perform inter-frequency measurements on another carrier in Band Y. Therefore, the UE cannot link its above-mentioned legacy LTE Band Combination entry for Band X with the NR Band Combination entry of Band Y. It would rather have to include a second LTE Band Combination entry for Band X that differs (at least) in the “interFreqNeedForGaps” bitmap. It should be noted that, as already in LTE, the configuration of a serving cell on a carrier does not only impact the interFreqNeedForGaps for that band but also for other bands (which the UE serves by the same RF).

A UE sharing RF and/or baseband components between NR and LTE, will likely include additional “LTE fallback band combination” for the purpose of linking them to NR band combinations and vice versa.

Furthermore, connecting NR band combinations with LTE band combinations by means of a matrix bears the risk that RAN2 inherits also the current capability signalling problems to NR. In particular, it would make it likely that a UE lists all “NR fallback band combinations” explicitly in order to combine them with different LTE band combinations. Hence, even though RAN2 aims to define NR band combination signalling which avoids the problematic inclusion of fallback band combinations, the linking to LTE band combinations would re-enforce that problem.

A matrix of LTE- and NR Band Combinations bears the risk that UEs advertise all NR (and LTE) fallback band combinations explicitly.

We therefore suggested earlier that RAN2 should put attention to improve and simplify the UE-NR-Capability structure before or at least at the same time as discussing the capability coordination for EN-DC.

RAN2 should simplify the UE capability structures for NR. The capability signalling for EN-DC shall not inherit the problems identified in the LTE capability signalling structure.

RAN2 agreed to avoid duplication of band combinations by extracting baseband capabilities from the supportedBandCombination structure. Changing the entire LTE capability signalling in a similar way is difficult due to the need to support also legacy networks. That means, an LTE UE supporting a lean but non-backwards-compatible LTE capability signalling would also need to provide the legacy capability structure in order to function in legacy LTE networks. However, EN-DC will only be supported by new/upgraded eNBs anyway and therefore legacy eNBs do not need to comprehend the EN-DC band combination capabilities. Hence, the NR+LTE band combination capabilities could also adopt a better and leaner structure that RAN2 hopefully introduces for NR (e.g. listing only top-level band combinations; separating baseband from RF capabilities; cost function or table to express dependencies of processing-heavy features; …).

EN-DC band combination entries may use a leaner structure than the established LTE band combinations since they do not need to be comprehended by legacy LTE eNBs.

In [3] we suggest using a cost-function to express dependencies among NR capabilities (e.g. among MIMO-, NAICS and CSI-RS processing). If RAN2 agrees to such or similar proposals, we believe that such principle should also be considered for the EN-DC capabilities. Following the principles suggested in [3], a UE would indicate its total baseband “processing budget” as well as the “processing price” for each feature. For EN-DC, the MeNB and the SgNB could share that budget, i.e., the MeNB indicates to the SgNB the “processing cost” of the features that it configured. Hence, the SgNB may use the remainder. If necessary, the UE could indicate different cost functions for the NR and the LTE feature (e.g. if 4 layer MIMO in LTE “cost” more than 4 layer MIMO in NR).

Cost-function based capability signalling could also be applied for EN-DC capabilities.

If RAN2 agrees to simplify the NR capability signalling by means of a cost function, the same principle should also be applied for EN-DC capabilities.

## Comprehending the capability structure of the other RAT

A claimed benefit of cross-referencing between EUTRA and NR capabilities was that each network side would only need to comprehend its own capability structure, i.e., the LTE eNB would only need to comprehend the UE-EUTRA-Capability IE, select a band combination that is suitable from LTE MCG perspective and inform the NR SgNB about the indexes of the compatible NR band combinations.

However, it is obvious that the MeNB must be able to comprehend in which NR bands the UE supports LTE/NR DC and determine whether the UE supports any EN-DC band combinations that it supports with its peer SgNB. This was also captured as an agreement at the RAN2#98 meeting:

1 For each LTE BC in the UE capabilities at least the possible NR frequency bands that can operate with this LTE BC should be visible to the LTE MN

In RAN2#NR-AH2, this was complemented with the following agreement for NE-DC:

1: 	In case of NE-DC, for each NR BC in the UE capabilities at least the possible LTE frequency bands that can operate with this NR BC should be visible to the NR MN.

One of the issues to solve for the indexing solution is how the LTE/NR MN can learn the possible NR/LTE frequency bands that can operate with LTE/NR BC. Minutes from RAN2#NR-AH2 mention that e.g. for each index there could be a frequency list of the NR/LTE frequency bands that can be understood by the MN, so the frequency part could be visible to both MN and SN. Such frequency information would however increase the signalling overhead of the cross-referencing solution, and this increased overhead must also be considered in the comparison with explicit signalling of the combinations.

Adding the supported frequency information to each index will increase the signalling overhead of the cross-referencing solution.

Preferably, in addition to the NR frequency bands, the MeNB should also be able to assess the UE’s capabilities in those bands (MIMO, CA, CoMP, …) to determine whether EN-DC may result in the desired performance, given e.g. the measurement report from the UE on detected and measure NR carriers. Therefore, the MeNB must anyway comprehend to some extent the linked UE-NR-Capabilities.

Even with cross-referencing between UE-EUTRA-Capability and UE-NR-Capability, at least the MeNB must comprehend the other RAT’s capabilities to determine whether and how the UE supports EN-DC in its network.

## Conclusion: Cross-Referencing vs. Explicit

As discussed in the preceding sub-sections, the idea to link elements in the EUTRA capabilities with other elements in the NR capabilities introduces a number of problems and does not achieve the desired “re-use” of existing signalling. It rather bears the risk to create an unnecessarily large NR capability structure due to the legacy capability signalling structure inherited from LTE.

Therefore, we propose the following:

Do not proceed with the attempt to cross-reference NR- and EUTRA capabilities for LTE/NR Dual Connectivity.

We consider it more promising to include the LTE/NR capabilities in a common capability structure which could either be the one of the RAT that is the master node or a separate “EN-DC capability”.

Include capabilities for LTE/NR Dual Connectivity in a common capability structure which both the EUTRA and the NR node comprehends.

Discuss whether to include the EN-DC UE capabilities in the UE-EUTRA-Capability IE (when LTE is the Master) or in a new separate capability container.

Regardless of the signalling method for the EN-DC capabilities, it would be important not to enforce listing all fallback band combinations per RAT, in order to list supported LTE-NR combinations. That would counter act the efforts spent to simplify the NR capability structure [3], e.g. by extracting the baseband capabilities from the band combinations, which is partly done to avoid signalling of fall back band combinations.

The introduction of EN-DC capabilities must not re-enforce listing all fallback band combinations only for the purpose of EN-DC

## Example capability structure

In the following sub-section we provide examples how the capability signalling for EN-DC could be added to the UE-EUTRA-Capability IE. If RAN2 decides to create a separate EN-DC capability structure, the ASN.1 would look similar.

### Example for per-UE capabilities

We start in this sub-section with examples for some of the per-UE capabilities. This covers capability bits for NR features that the UE supports when configured with EN-DC. But it also covers capability bits that define LTE features that are or are not supported when the UE is configured with EN-DC.

The latter set of capability bits may duplicate some of the capability bits that exist already in the UE-EUTRA-Capability IE.

More interesting are the NR capability bits. To avoid specifying the capability fields and IEs in two specifications, we suggest nevertheless to specify NR related capabilities in the 38-series and to use them as transparent container in the LTE capabilities or in a separate EN-DC capability container.

Table 1: UE-EUTRA-Capabilities for Rel-15 containing an IE collecting per-UE capabilities 
applicable when EUTRA-NR DC is configured. This IE may contain LTE specific 
parameters and (primarily) an OCTET STRING with an IE from the NR specification.

-- ASN1START

UE-EUTRA-Capability-v15xy-IEs ::= SEQUENCE {

lteNrDualConnectivity				ENDC-Capabilities-v15xy			OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

...

ENDC-DC-Capabilities-v15xy ::= SEQUENCE {

eutra-specific-Capabilities 	SEQUENCE {

-- EUTRA capabilities applicable when configured with LTE-NR DC (if any)

},

nr-specific-Capablities			OCTET STRING (CONTAINING UE-ENDC-NR-Capability-Container-15xy)									OPTIONAL

}

Table 2: UE-NR-Capabilities for Rel-15 containing IEs collecting NR capabilities 
applicable when EUTRA-NR DC is configured (specified in 38.331).

UE-ENDC-NR-Capability-Container-15xy ::=	SEQUENCE {

pdcp-Parameters						PDCP-Parameters,

phyLayerParameters					PhyLayerParameters,

rf-Parameters						RF-Parameters,

...

}

### Example for Band-Combination capabilities

As explained in the previous section, we propose to define the LTE+NR band combination in a common capability structure. Table 3 shows how the LTE BandCombinationParameters could define a combination of an LTE Band (defined by an entry of “BandParameters-r13” and an entry of “NR-BandParameters”. The latter would be specified in the NR RRC specification but is supposed to be comprehended by an LTE eNB that intends to support EN-DC. In other words, an LTE eNB not supporting EN-DC and generally all legacy LTE eNBs would ignore such LTE+NR band combination entries. As mentioned above, this allows RAN2 to consider further optimizations of these band combinations and to apply a similar structure as we propose for NR-only operation [3].

Table 3: EUTRA Band Combination Parameters including NR bands for EN-DC.

BandCombinationParameters-r15 ::=	SEQUENCE {

differentFallbackSupported-r15	ENUMERATED {true}				OPTIONAL,

bandParameterList-r15			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-r15,

supportedBandwidthCombinationSet-r15	SupportedBandwidthCombinationSet-r10	OPTIONAL,

multipleTimingAdvance-r15		ENUMERATED {supported}				OPTIONAL,

simultaneousRx-Tx-r15			ENUMERATED {supported}				OPTIONAL,

bandInfoEUTRA-r15				BandInfoEUTRA,

dc-Support-r15					SEQUENCE {

asynchronous-r15			ENUMERATED {supported}				OPTIONAL,

supportedCellGrouping-r15		CHOICE {

threeEntries-r15				BIT STRING (SIZE(3)),

fourEntries-r15					BIT STRING (SIZE(7)),

fiveEntries-r15					BIT STRING (SIZE(15))

}																OPTIONAL

}																	OPTIONAL,

supportedNAICS-2CRS-AP-r15		BIT STRING (SIZE (1..maxNAICS-Entries-r12))	OPTIONAL,

commSupportedBandsPerBC-r15		BIT STRING (SIZE (1.. maxBands))		OPTIONAL

}

BandParameters-r15 ::= SEQUENCE {

CHOICE {

bandEUTRA		BandParameters-r13,

bandNR			OCTET STRING (CONTAINING NR-BandParameters)

}

}

BandParameters-r13 ::= SEQUENCE {

bandEUTRA-r13					FreqBandIndicator-r11,

bandParametersUL-r13			BandParametersUL-r13				OPTIONAL,

bandParametersDL-r13			BandParametersDL-r13				OPTIONAL,

supportedCSI-Proc-r13			ENUMERATED {n1, n3, n4}				OPTIONAL

}

BandParametersUL-r13 ::= CA-MIMO-ParametersUL-r10

CA-MIMO-ParametersUL-r10 ::= SEQUENCE {

ca-BandwidthClassUL-r10				CA-BandwidthClass-r10,

supportedMIMO-CapabilityUL-r10		MIMO-CapabilityUL-r10				OPTIONAL

}

BandParametersDL-r13 ::= CA-MIMO-ParametersDL-r13

CA-MIMO-ParametersDL-r13 ::= SEQUENCE {

ca-BandwidthClassDL-r13					CA-BandwidthClass-r10,

supportedMIMO-CapabilityDL-r13			MIMO-CapabilityDL-r10				OPTIONAL,

fourLayerTM3-TM4-r13						ENUMERATED {supported}				OPTIONAL,

intraBandContiguousCC-InfoList-r13		SEQUENCE (SIZE (1..maxServCell-r13)) OF

IntraBandContiguousCC-Info-r12

}

While the detailed design of the capability signalling for EN-DC cannot be decided yet, we think that RAN2 could aim to avoid duplicating capability IEs in LTE and NR specifications and rather inherit e.g. an NR capability IE into the LTE specification.

### Distinguishing Standalone from Non-Standalone

At least initial UEs will not support NR standalone operation and this information must be conveyed in the inter-RAT IEs inside the LTE capabilities so that the LTE network knows whether or not it may handover the UE to NR (if the UE supports NR standalone) or use it only in DC mode.

UE-EUTRA-Capability information element

-- ASN1START

UE-EUTRA-Capability ::=			SEQUENCE {

accessStratumRelease				AccessStratumRelease,

ue-Category							INTEGER (1..5),

pdcp-Parameters						PDCP-Parameters,

phyLayerParameters					PhyLayerParameters,

rf-Parameters						RF-Parameters,

measParameters						MeasParameters,

featureGroupIndicators				BIT STRING (SIZE (32))				OPTIONAL,

interRAT-Parameters				SEQUENCE {

utraFDD								IRAT-ParametersUTRA-FDD				OPTIONAL,

utraTDD128							IRAT-ParametersUTRA-TDD128				OPTIONAL,

utraTDD384							IRAT-ParametersUTRA-TDD384				OPTIONAL,

utraTDD768							IRAT-ParametersUTRA-TDD768				OPTIONAL,

geran								IRAT-ParametersGERAN					OPTIONAL,

cdma2000-HRPD						IRAT-ParametersCDMA2000-HRPD			OPTIONAL,

cdma2000-1xRTT						IRAT-ParametersCDMA2000-1XRTT			OPTIONAL

},

nonCriticalExtension				UE-EUTRA-Capability-v920-IEs		OPTIONAL

}

One could consider that UEs not supporting NR in standalone mode do not at all list “NR” in the interRAT-Parameters. However, the list is also referred to from within the measParameters (InterRAT-BandList InterRAT-BandInfointerRAT-NeedForGaps) to indicate whether the UE requires gaps to measure those NR bands (including the case when LTE/NR DC is not yet configured). Hence, even UEs not supporting standalone NR (on all their NR carriers) should list the NR bands in the interRAT-Parameters.

# Conclusion

Based on the discussion we propose the following:

Proposal 1	RAN2 should simplify the UE capability structures for NR. The capability signalling for EN-DC shall not inherit the problems identified in the LTE capability signalling structure.

Proposal 2	If RAN2 agrees to simplify the NR capability signalling by means of a cost function, the same principle should also be applied for EN-DC capabilities.

Proposal 3	Do not proceed with the attempt to cross-reference NR- and EUTRA capabilities for LTE/NR Dual Connectivity.

Proposal 4	Include capabilities for LTE/NR Dual Connectivity in a common capability structure which both the EUTRA and the NR node comprehends.

Proposal 5	Discuss whether to include the EN-DC UE capabilities in the UE-EUTRA-Capability IE (when LTE is the Master) or in a new separate capability container.

Proposal 6	The introduction of EN-DC capabilities must not re-enforce listing all fallback band combinations only for the purpose of EN-DC

# References

R2-168609,	Capability Coordination across RATs,	Qualcomm Incorporated

R2-168961,	LS on UE capability aspects for LTE/NR tight interworking (to: RAN4, RAN1; contact: Nokia)

R2-1708031, UE capability Signalling Structure for NR, Ericsson, RAN2-99

# Annex – Agreements reached so far

RAN2#95:

1 From a RAN2 perspective, we aim to have an independent capability information for NR and LTE (meaning that node of one RAT does not need to look at the capabilities of the other RAT). Does not preclude that capabilities of one RAT might contain some information related to the other RAT (e.g. at least measurement capabilities) 2 RAN2 should study further how to coordinate capabilities between the UE, LTE eNB and NR gNB

RAN2#95bis

1: 	RAN2 shall consider the LTE/NR tight interworking (with LTE eNB, NR gNB or eLTE eNB as a master node) for the coordination of capabilities.

2:	We should aim to minimum the differences between the NR capability reporting across the LTE/NR tight interworking cases (NR gNB as a master node) and the standalone NR gNB case.

3	At least some band combinations across RATs should be coordinated across the master and the secondary nodes.

4	Layer 2 buffer capabilities should be coordinated across the RATs should be coordinated across the master and the secondary nodes.

5	RAN2 aim for a solution where the master node and secondary node are not required to comprehend each others UE configuration.

Agreements:

1: 	Agree the following principle: the master node and the secondary node only need to use own RAT UE capabilities (which will include some other RAT capabilities relating to the interworking). At least for the initial configuration of interworking case these are provided on the master node RAT or from core network

2:	Allow gNB to format NR RRC PDUs for the UE configuration.

RAN2 NR-AH1

Agreements

1: 	Only two nodes (i.e. one LTE eNB and one NR gNB) need to be considered in the LTE/NR capability coordination. The forward compatibility with multiple nodes can also be considered.

2: 	For capabilities for which coordination is needed, then it is up to master node to make the decision on how to resolve the dependency..

3: 	For capabilities for which coordination is needed, the secondary node is allowed to initiate the re-negotiation of capability, and with the re-negotiation request from secondary node, it is up to master node to make the final decision.

and

Agreements

LTE capabilities changes to support EN-DC

1: LTE capabilites shall include information related to NR measurements

2: LTE capabilites shall include support of EN-DC

3: Further changes to LTE capabilities are FFS

NR capability reporting

4: NR shall support independent capabilities reporting (this does not preclude the NR and LTE capabilities indicating dependencies in the capabilities reported)

LTE/NR capabilites dependencies to support EN-DC

5: For Type I capabilities (where the use of the capability is isolated to the RAT), no coordination is needed and the NR specific capabilities are just forwarded by the MeNB to the SgNB using LTE DC as a baseline

RAN2#97 Athens:

Agreements

1	Type definitions are guidance for the purpose of discussion in the SI and early part of the WI phase. They will not limit further discussion and will not be captured in the specifications.

2	Type II, the use of the capability in one RAT has impacts to the other RAT, however the use of capability in one RAT is not understood by the NW side of the other RAT.

3	Type III, the use of the capability in one RAT has impact to the other RAT, and the use of capability in one RAT is understood by the NW side of the other RAT.

4:	Some capabilities (e.g. RF capability) are coordinated using Xx and involve a reconfiguration of the UE. The configuration of the UE does not exceeds its capabilities.

5:	Some capabilities (e.g. buffer size) are coordinated using Xx and will not involve a reconfiguration of the UE. The ongoing operation of the network does not exceed the UEs capabilities

RAN2#97bis Spokane:

Agreements 1: NR Capability is defined as the UE capability container for NR to include all NR specific capabilities required for the standalone operation. 1a: The capabilities for CA/DC within NR are reported, if supported, in the NR Capability. FFS Whether LTE/NR DC specific capabilities requiring coordination between eNB and gNB are included in NR-Capability or LTE Capability or a new LTE/NR-Capability container. 3: The eNB/gNB should be able to retrieve NR Capability, LTE/NR-Capability (if agreed) and LTE-Capability depending on the NSA/SA operations. 4: In case of LTE-NR DC/MC, the master node should be able to forward LTE/NR-Capability (if agreed) and the secondary RAT specific capability (NR-Capability or EUTRA-Capability) to the secondary node within “SCG-ConfigInfo” (The IE name for LTE-NR DC/MC is TBD).

RAN2#NR-AH2 Qingdao:

Agreements:

1:	In case of NE-DC, for each NR BC in the UE capabilities at least the possible LTE frequency bands that can operate with this NR BC should be visible to the NR MN.

2:	For MR-DC, capability signalling and coordination will support shared baseband capabilities between LTE and NR. The exact capabilities for coordination should FFS and dependent on RAN1/4 discussion.

3:	RAN2 continues to work on capability coordination not requiring MN and SN comprehend each other’s UE configuration (e.g. the index based coordination).

4:	For the index-based coordination, the following open issues need to be resolved:

-	How can the LTE/NR MN learn the possible NR/LTE frequency bands that can operate with LTE/NR BC? (e.g. For each index there is a frequency list of the NR/LTE frequency bands that can be understood by the MN, frequency part is visible to both MN and SN)

-	How to address baseband capability dependency between LTE and NR (Proposal 4)? (Depends on outcome of RAN1/4 discussion)

-	How can the MN/SN decide MCG/SCG configuration if LTE-NR band combination is defined within the same frequency band?

-	If the above issues cannot be resolved then the fallback will be to use the LTE baseline.