3GPP TSG-RAN WG2 #99 Meeting	R2-1708072

Berlin, Germany, 21 – 25 August, 2017 	Revision of R2-1706768

Source: 	Huawei, HiSilicon

Title: 	On demand SI acquisition and failure handling

Agenda Item:	10.4.1.5.5

Document for:	Discussion and decision

# Introduction

In RAN 2 NR Ad-hoc#1 meeting, there is highlighted FFS on whether there is a need for an additional indication that an on-demand SI is being broadcast or not.

Agreements related to SI provided by broadcast

3: The scheduling information for other SI includes SIB type, validity information, periodicity, and SI-window information in minimum SI irrespective of whether other SI is periodically broadcasted or provided on demand.

FFS Whether there is an additional indication that an on demand SI is actually being broadcast at this instant in time.

The last RAN2 Ad hoc meeting, the following agreements are achieved for MSG1 and MSG3 based on-demand SI request:

Agreements for Msg1 based SI request method:

1:	RAPID is included in Msg2.

2: 	Fields Timing Alignment Information, UL grant and Temporary C-RNTI are not included in Msg2.

3:	RACH procedure for SI requests is considered successful when Msg2 containing a RAPID corresponding to the transmitted preamble is received.

4:	Msg2 reception uses RA-RNTI that corresponds to the Msg1 transmitted by the UE (details of RA-RNTI selection left to UP discussion)

5:	UE retransmits RACH preamble according to NR RACH power ramping

6: 	Msg1 for SI request re-transmission is continued until reaching max preamble transmissions. Thereafter, a Random Access problem to upper layers is indicated. (depending on the NR RACH procedure design)

FFS: Upper layer actions when MAC reports Random Access problem. To be discussed in CP session.

7:	Back off is applicable for Msg1 based SI requests but no special Back off subheader/ procedure is required.

Agreements for Msg3 based SI request method:

1: 	UE determines successful Msg3 based on reception of Msg4

FFS Details of the Msg4 content used to confirm successful Msg3. To be discussed initially CP.

2:	Preamble(s) for SI request using Msg3 based Method are not reserved.

3:	RRC signalling is used for SI request in Msg3.

FFS: RRC signalling how to indicate the requested SI/SIB details left to ASN.1 work.

5:	Temporary C-RNTI received in Msg2 is used for Msg4 reception

However, some issues still need to be addressed for MSG1 and MSG3 based SI request. This paper further discusses issues related to other SI request.

# Indication for other SI

Before UE sends SI request, the UE needs to check the indication in minimum SI. This section further discusses the solutions of indication of other SI in minimum SI.

Two design options for the indication were discussed in previous papers [1] [2]:

Option a): a single bit that is dynamically changed to indicate a SIB is periodically broadcasted or provided on demand;

Option b): two bits are configured per other SI in scheduling information: the first indicator indicates whether the other SI is periodically broadcasted or provided on demand while the second one indicates whether the on-demand other SI is actually being temporarily broadcasted.

According to [3], the scheduling information in minimum SI includes an indicator whether the concerned SI-block is periodically broadcasted or provided on demand. If minimum SI indicates that a SIB is not broadcasted, then UE does not assume that this SIB is a periodically broadcasted in its SI-window at every SI periodicity. For the UE needs to acquire the SIB, it can send an SI request to receive this SIB if it is not acquired.

For option a, when the network broadcasts the requested other SI, it can change the related indicator to indicate that the SIB is broadcasted. For a later UE (e.g. entering the cell after the SI has started to be broadcast), if it wants the SIB and checks the indicator, it finds that the wanted SIB is being broadcasted. As a result, unnecessary SI requests will be avoided.

For option b, the network will keep the first indicator and change the second one if the requested on demand other SI is being broadcasted. For a later UE, it checks the second indicator and will not trigger SI request.

Based on the above discussion, it is worth noting that a single indicator can prevent the unnecessary SI requests from other UEs for a SIB while the SIB is being broadcasted due to one UE’s request. There is no need for an additional indication for this purpose. In addition, option b) introduces extra overhead.

There are two solutions to signal the explicit indicator for option a):

Solution 1): the indicator is included in each scheduling information IE of corresponding SIB as one bit indication;

Solution 2): the indicator is formed as a bitmap, in a separate IE to the scheduling information sub-item of the other SIB.

For solution 2), the UE doesn’t need to check each of the specific contents of scheduling information of the other SIB for detecting whether it is broadcast or not. It can determine the information just by checking the bitmap IE. And as it is a relatively independent IE, the change operation can be decoupled with the specific content of scheduling information of the other SIB rather than waiting for a modification period boundary it can be updated immediately. Furthermore, if the bitmap is signaled in a fixed SIB sequence (i.e. each bit in the bitmap always refers to the same SIB), regardless of whether the gNB broadcasts that SIB or not, the UE may directly be aware of whether to send a SI request or not without further having to refer e.g. to the order of SIBs in the scheduling information. Therefore, it is proposed:

- Signal a single bit per SIB that is dynamically changed for indication of whether one on-demand SI is broadcast or not, and the indication is broadcast in the form of bitmap in a separate IE.

Additionally, the change frequency of the indication depends on that of the UE’s SI request. And the requested UE needs not to know the indication change triggered by itself. Therefore, the UE is not required to update the bitmap all the time. Conversely, it is beneficial for the UE only to update the bitmap before initiating the related Other SI request.

- The UE is only required to check the latest bitmap before initiating the Other SI request.

For the UE, after sending the OSI request, it will not wait to check whether the bitmap changes. To help the UE acquire the requested OSI as soon as possible, it is desirable that the network can deliver the requested OSI in the nearest SI window no matter whether the bitmap changes. From a network perspective, the SI can be delivered as soon as possible without waiting for some specific transmission window, e.g. modification period.

- If SI is requested, it can be delivered at any time, i.e. SI delivery is not restricted to some modification period.

- After sending SI request, the UE can check immediately whether the requested SI is broadcast or not, even if the bitmap is not changed in minimum SI.

Fig 1 below shows general illustration of the above solutions.

Fig 1: the procedure of bitmap changes and SI acquisition for UE

# SI request

According to the agreements in the last meeting, after successfully sending the SI request via MSG1 or MSG3, for receiving the requested SIB, the UE monitors the SI window of the requested SIB in one or more periods according to the SI scheduling information indicated in MSI (Minimum SI). In the following sections, we will give detailed considerations on how to design MSG1 and MSG3 based request, acknowledgement transmission after SI request and the error handling mechanism.

## MSG1 based other SI request

MSG1 based SI request will transmit a preconfigured preamble corresponding to the requested SIB(s) on the preconfigured PRACH resource. The number of preambles is limited. If too many preambles are allocated for SI request, it will reduce the available preambles for normal RACH and consequently lead to a higher collision probability of RACH.

In order to avoid the impacts on the normal RACH, we think preambles for SI request should not be reserved on all of PRACH resources, i.e. only part of PRACH resources can be used to reserve preambles for on demand SI request. The possible solutions can be:

- Solution1: To reserve preambles for SI request on the same PRACH resource. The remaining of preambles can be used for normal RACH on this resource.

- Solution 2: To reserve preambles for SI request on different PRACH resources. For example, if 4 preambles are reserved for SI request, these 4 preambles can be reserved on different PRACH resources. The preambles reserved on different PRACH resource can be the same but for different SI request.

The mapping of reserved preambles, PRACH resources and corresponding SI should be indicated to UE in Remaining Minimum SI.

These solutions can be shown in Fig.2 as below.

Fig 2: PRACH resource and preamble allocation for MSG1 based other SI request

For solution 1 as shown in Fig 2(a), all SI requests are initiated on the same resource and it could lead to the simultaneously SI transmission. However from configuration point of view, this solution could be simpler than the second one.

For solution 2 as shown in Fig 2 (b), the SI requests are distributed on multiple slots and the simultaneous SI request can be reduced. Although this solution may need more signalling, it is more flexible than solution 1.

We should avoid reservation on all PRACH resources for all SIs request considering normal RACH should not be impacted because of SI request.

- For MSG1 based method, SI can be requested on different PRACH Time/Frequency resource and the same preamble on different T/F resource can be used to request different SI.

## MSG3 based other SI request

For MSG3 based SI request, there are still some issues which need to be addressed, and we will discussed one by one in the following section.

        - What’s the granularity of requested SI for MSG3 SI request?

In RAN2#98 meeting, it was agreed that for MSG1 based SI request, the minimum granularity of requested SI is one SI message. However there is no confirmation on the minimum granularity of MSG3 based SI request. In our understanding, the minimum granularity of SI request in MSG3 is also one SI message.

- For MSG3 based method, the minimum granularity of requested SI is one SI message.

        - What RRC message should be used to request SI for MSG3 based solution?

It is agreed in the last meeting that RRC signalling is used for SI request in Msg3. But which RRC message is used for such request is not clear.

In LTE, only RRCConnectionRequest and RRCConnectionReestablishmentRequest can initiate service in RRC layer. In our understanding, both of them are not suitable for SI request. So, a new type of service request RRCSystemInfoRequest can be defined in order to support SI request.

- A new type of RRC message RRCSystemInfoRequest should be defined to support SI request.

For the UE in RRC_IDLE, there is no dedicated resource or security activation. The message can only be transmitted via CCCH with the signalling radio bearer of SRB0. For inactive state UE, it depends on the detailed design.

Accordingly, for the UE in RRC_IDLE, the message can only be transmitted via CCCH. The RLC Mode can only be TM mode. For inactive state UE, it depends on the detailed design.

- For the UE in RRC_IDLE state, the SI request is sent via CCCH with the signalling radio bearer of SRB0, and the RLC Mode for SI request can only be TM mode. Inactive state UE needs further discussion.

In the last meeting, bitmap based RRC message was discussed, but how to indicate the requested SI/SIB details of the RRC was left to ASN.1 work. Our understanding is that the basic principle shall be defined and we propose the following table of RRC content for MSG3 based SI request message for the two states:

Table 1 a profile of the SI request message for the two states

|  |  | RRC_Idle | RRC_Inactive |
| --- | --- | --- | --- |
| Content | requested SI information | mandatory | mandatory |
|  | requested UE information | mandatory | mandatory |
| The Signalling radio bearer |  | CCCH | FFS |
| The RLC Mode |  | TM mode | FFS |

Based on the above table, we provide ASN.1 of the SI request message:

#### –	SIRequest

The SIRequest message is used to request the other SI.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to NR

RRCSystemInfoRequest message

-- ASN1START

# RRCSystemInfoRequest-r15 ::= SEQUENCE {

criticalExtensions					CHOICE {

RRCSystemInfoRequest-r15			RRCSystemInfoRequest-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

# RRCSystemInfoRequest-r15-IEs ::= SEQUENCE {

request-SIType-List				BIT STRING (SIZE (40)),

spare								BIT STRING (SIZE (8))

}

-- ASN1STOP

- Use the ASN.1 above as baseline for MSG3 based RRC message. Each bit of the bitmap corresponds to one SI in the same order as appeared in the scheduling information.

        - What’s the content of MSG4?

In the last meeting, it was agreed that UE determines successful MSG3 based on reception of MSG4. But as for the content of MSG4 used to confirm successful MSG3, it was left for FFS.

As discussed above, it’s possible that multiple UEs may use the same preamble to initiate RACH at the same time. In this case, contention will happen and there are at least two cases should be considered:

Case 1: the contention is between a normal RACH UE and the UE for SI request or two UEs request different SIs.

Case 2: the contention is between UEs request for the same SIs;

For the first case, network can only send one acknowledgement with contention resolution ID to one of them if legacy RACH procedure is followed. The legacy contention resolution method makes sure the following allocated radio resources can be used for one certain UE. But from the UE(s) sending SI request perspective, it has no data which needs to be transmitted and it only cares whether SI request for specific SI is received by the network In this case, one possible solution is that network indicates SI requests received for SI based request. This indication can be one bit, a bitmap corresponding to SIs, or something else. This indication is not used for contention resolution, and it is just regarded as the acknowledgement of MSG3 SI request. If UE receives this indication, it ignores the contention resolution ID and to receive SI for some duration, if the SI is not what it requested, it will request again.

For the second case, if the MSG4 from network has no corresponding contention resolution ID for the UE, the contention resolution will fail for this UE even though they are requesting for the same SIs if legacy contention resolution is used. The same method as described for case 1 also can be used.  Another possible solution is to make the UEs request for the same SI have same UE ID for resolution.

For both cases, after sending SI request in MSG3, UE can check whether the requested SI is broadcast or not, as well as the acknowledgement for MSG3 during the widnow of contention resolution. If the requested SI is broadcast, the UE can read the SI and ignore the acknowledgement.

- An indication in MSG4 is used to indicate SI request is received by the network. If this indication indicates SI request has been received, the UE skips checking contention resolution ID and proceeds to receive SI.

If considering the UE only sends SI request, as per proposal 7, a new RRC message type is used for SI request. Then, the indication with a bitmap can be used to replace contention resolution ID using UE ID or a random value in LTE because a bitmap indicates which SI will be broadcast due to SI request is enough for the UE to receive SI.

For example UE1 requests SI1, SI2, and UE2 requests SI1, SI2, SI3. If the network indicates SI1, SI2, SI3 will be broadcast, both UEs should consider SI request is successful. If the network indicates SI1 and SI2 will be broadcast, UE2 considers its SI request is failed. In this case, the UE should check whether the  SIs that the network will broadcast include the SIs it requests.

- Instead of contention resolution MSG4 includes a bitmap to allow multiple UEs requesting SI messages using same PRACH resource (time/frequency/preamble resource) to check the requested SI is included in the bitmap or not.

        - RRC response is needed or not?

When the UE sends SI request, it will receive the response from network. If the RACH procedure using for SI request failed, UE will send indication to RRC. Otherwise, UE will receive SI based on the request from RRC. So, there is no need for the network to send RRC response to UE after MSG4.

- RRC response message is not needed for MSG3 based SI request.

## Connected mode SI request

Connected mode SI request has not been discussed yet. Compared with idle/inactive mode SI request, it would be easier for connected mode to request SI.

For connected state UE with valid TA, UE can request only the SIBs that are necessary in order to save signalling overhead. On the other hand, one UE can request multiple SIBs in one request. A new RRC message with a bitmap for each SIB shall be used for connected mode UE to request SI.

While for connected state UE without valid TA, according to the agreement in RAN2#96 meeting that for UEs in connected, dedicated RRC signalling can be used for the request and delivery of other SI, UE shall initiate the RACH procedure to synchronize to the network and then acquire the SI using RRC message. However, the content of the RRC message for connected UEs was not discussed yet. In our understanding, the content of the RRC message for connected UE is based on a bitmap for each SIB because the granularity for connected mode UE can be different from idle/inactive mode UEs.

- RRC message with bitmap for each SIB shall be used for connected mode SI request.

- For connected state with valid TA, the minimum granularity of requested system information is SIB in order to save the signalling overhead and one request procedure can request one or multiple SIBs.

- For connected state without valid TA, the UE initiates RACH procedure to synchronize to the network and uses dedicate RRC SI request to acquire on-demand SI.

## Failure handing in case SI is not received

It was previously agreed to use retransmissions in the case of MSG1 transmission failure. But for MSG3 based SI request, if the corresponding SI request response (i.e. MSG4) is not received during configured monitoring time, how to handle it was not discussed yet.

In this case, UE shall initiate another round of SI request based on back off indication(if included in the received RAR) until the maximum request number is reached. The power ramping can be applied according to NR RACH power ramping as MSG1 based mechanism.

- For MSG3 based SI request, if SI request response (i.e. MSG4) is not received, and the maximum SI request number is not reached, another RACH procedure shall be started in order to retry Msg3 transmission.

For each SI request, a maximum SI request number should be defined. For MSG1 based request, the maximum SI request number can follow the normal RACH procedure. For MSG3 based SI request the maximum SI request number for MSG3 based SI request should be configured in minimum SI. If SI request reaches the maximum number, the on-demand SI request shall be considered failed, the UE behaviour of failure of on-demand SI request may be different depending on whether the requested SI impacts the service or not. If the requested on-demand SI doesn’t prevent the UE from using regular services, UE can just label the SI as not available in the cell and mark the function as unavailable. Otherwise, cell reselection can be triggered when the request of SI reaches the maximum number.

- A maximum SI request number and monitoring time duration should be included in minimum SI for MSG3 based SI request.

- If SI request reaches the maximum number, the UE can label the SI as not available in the cell or trigger cell reselection.

# Conclusion

In this paper we discuss the design for MSG 1 based request and failure handling, and have the following proposals:

- Signal a single bit per SIB that is dynamically changed for indication of whether one on-demand SI is broadcast or not, and the indication is broadcast in the form of bitmap in a separate IE.

- The UE is only required to check the latest bitmap before initiating the Other SI request.

- If SI is requested, it can be delivered at any time, i.e. SI delivery is not restricted to some modification period.

- After sending SI request, the UE can check immediately whether the requested SI is broadcast or not, even if the bitmap changing in minimum SI is not acquired.

- For MSG1 based method, SI can be requested on different PRACH Time/Frequency resource and the same preamble on different T/F resource can be used to request different SI.

- For MSG3 based method, the minimum granularity of requested SI is one SI message.

- A new type of RRC message RRCSystemInfoRequest should be defined to support SI request.

- For the UE in RRC_IDLE state, the SI request is sent via CCCH with the signalling radio bearer of SRB0, and the RLC Mode for SI request can only be TM mode. Inactive state UE needs further discussion.

- Use the ASN.1 above as baseline for MSG3 based RRC message. Each bit of the bitmap corresponds to one SI in the same order as appeared in the scheduling information.

- An indication in MSG4 is used to indicate SI request is received by the network. If this indication indicates SI request has been received, the UE skips checking contention resolution ID and proceeds to receive SI.

- Instead of contention resolution MSG4 includes a bitmap to allow multiple UEs requesting SI messages using same PRACH resource (time/frequency/preamble resource) to check the requested SI is included in the bitmap or not.

- RRC response message is not needed for MSG3 based SI request.

- RRC message with bitmap for each SIB shall be used for connected mode SI request.

- For connected state with valid TA, the minimum granularity of requested system information is SIB in order to save the signalling overhead and one request procedure can request one or multiple SIBs.

- For connected state without valid TA, the UE initiates RACH procedure to synchronize to the network and uses dedicate RRC SI request to acquire on-demand SI.

- For MSG3 based SI request, if SI request response (i.e. MSG4) is not received, and the maximum SI request number is not reached, another RACH procedure shall be started in order to retry Msg3 transmission.

- A maximum SI request number and monitoring time duration should be included in minimum SI for MSG3 based SI request.

- If SI request reaches the maximum number, the UE can label the SI as not available in the cell or trigger cell reselection.

# Reference

R2-1706822	Indication for On-Demand SI Broadcast	Nokia, Alcatel-Lucent Shanghai Bell

R2-1706768	On demand SI acquisition and failure handling	Huawei, HiSilicon

TS 38.304 e.0.0

3GPP RAN2#97bis, Chairman’s note, April, 2017.

3GPP RAN2#AH, Chairman’s note, January, 2017.

Samsung, “On Demand SI Request TX”, R2-1702970, Spokane, USA, April 3 - 7 2017.

LG Electronics Inc, “SI request procedure using MSG3”, R2-1703602, Spokane, USA, April 3 - 7 2017.

Samsung, “On Demand SI Delivery: Signaling Aspects”, R2-1700011, Spokane, USA, January 17- 19 2017.

3GPP RAN2#98, Chairman’s note, May, 2017.