3GPP TSG-RAN WG2#99	Tdoc R2- 1708157

Berlin, Germany, 21st – 25th August 2017	(revision of R2-1707241)

Agenda Item:	10.2.15

Source:	Ericsson, NTT DOCOMO INC., SoftBank

Title:	Public Warning System in NR

Document for:	Discussion, Decision

# Introduction

In the WID for New Access Technology that was approved at the RAN#75 meeting (in RP-170847), one of the objectives is to support Public Warning System (PWS).

This document discusses how PWS can be supported in NR. A text proposal to include a description on Public Warning System is also included in Annex A.

# Discussion

In LTE, Public Warning System (PWS) consist of Earthquake and Tsunami Warning System (ETWS) and Commercial Mobile Alert Service (CMAS). When a warning needs to be transmitted to the UEs it is done through broadcast of system information messages. ETWS notifications are then transmitted with SIB10 (ETWS primary notification) and SIB11 (ETWS secondary notification) whereas CMAS notifications are transmitted with SIB12. UEs in RRC_IDLE and RRC_CONNECTED state receive an indication in the paging message when there is an ongoing warning transfer. When the UE receives the indication it reads the system information scheduling information from the SIB1 message to find out where to receive the warning notification, i.e. SIB10, SIB11 or SIB12. In the LTE Rel-14 work with further enhancements for eMBMS, it has been agreed to add a common field in DCI format 1-C for ETWS and CMAS indications. This option is then used on such an FeMBMS carrier. A reason is that such a carrier does not support paging.

The PWS requirements can be found in [1]. For an ETWS primary notification, it includes a requirement to be successfully delivered to the UEs within 4 seconds from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network. This is due to that the notification may be due to an earthquake and thus needs to reach the recipients quickly so that persons can take needed precautions.

For ETWS, it is also required that the UE in RRC_IDLE and RRC CONNECTED needs to be able to receive the Primary and Secondary Notifications.

In LTE, PWS is supported through broadcast of system information messages SIB10, SIB11 and SIB12.

Notifications for ongoing warning transmission are in LTE included in the paging message or, on an FeMBMS carrier, in the DCI format 1-C on the PDCCH.

In LTE, warning notifications can be received by both UE in RRC_IDLE and RRC_CONNECTED.

As described above, in LTE the warning notifications are sent as system information messages, which thus are addressed to all UEs in the cell. Since the system information mechanism in NR will be similar to the one in LTE, it is thus reasonable to also reuse the PWS mechanism from LTE in order to fulfil the existing PWS requirements. The warning notifications should then also in NR be transmitted as separate SIBs in system information messages.

The PWS solution in LTE should be reused for NR. Different types of warning notifications should be transmitted as separate SIBs in system information messages.

In order to successfully deliver the warning notification to all UEs, even at the higher frequency bands that are to be supported by NR, the broadcasted transmissions should be performed in a manner that enables a coverage at least similar to the one for the minimum SI. This could then be achieved through mechanisms such as e.g. beam-sweeping, timely repetitions or power boosting.

With the warning notification being transmitted in system information messages, there are different options for indicating to the UEs that there is an ongoing warning transfer. The indication(s) may, just as in LTE, be included in the paging messages, and thus be detected by the UE when checking its paging occasions, or as an alternative be sent on the PDCCH at configured occasions.

The indication about an ongoing warning transfer should be included in the paging message as in LTE. RAN2 should discuss if it can be also provided on the PDCCH.

Similarly as in LTE, warning notifications should be able to be received by the UE in all RRC states (RRC_IDLE, RRC_CONNECTED, RRC_INACTIVE). It is assumed that also UEs in RRC_CONNECTED will be reached through the broadcasted transmissions, just as in LTE, as long as the network schedules the transmissions accordingly.

RAN2 to agree that the warning notifications should be able to be received by the UE in RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE.

As proposed above, the PWS solution in NR should be based on the one in LTE, with an open question on how the indication is provided to the UE. A text proposal to Stage 2 for Public Warning System, with the applicable parts from TS 36.300, is therefore provided in Annex A. It is here proposed to include it into TS 38.300.

Include the Text Proposal on Public Warning System in Annex A into TS 38.300

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE, PWS is supported through broadcast of system information messages SIB10, SIB11 and SIB12.

Observation 2	Notifications for ongoing warning transmission are in LTE included in the paging message or, on an FeMBMS carrier, in the DCI format 1-C on the PDCCH.

Observation 3	In LTE, warning notifications can be received by both UE in RRC_IDLE and RRC_CONNECTED.

Based on the discussion in section 2 we propose the following:

Proposal 1	The PWS solution in LTE should be reused for NR. Different types of warning notifications should be transmitted as separate SIBs in system information messages.

Proposal 2	The indication about an ongoing warning transfer should be included in the paging message as in LTE. RAN2 should discuss if it can be also provided on the PDCCH.

Proposal 3	RAN2 to agree that the warning notifications should be able to be received by the UE in RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE.

Proposal 4	Include the Text Proposal on Public Warning System in Annex A into TS 38.300

# References

3GPP TS 22.268 v14.0.0, Public Warning System (PWS) requirements

# Annex A: Text Proposal to 38.300

## 17.3	PWS

The NG-RAN provides support for warning systems through means of system information broadcast capability. The NG-RAN performs scheduling and broadcasting of the "warning message content" received from the CBC, which is forwarded to the NG-RAN by the AMF. The schedule information for the broadcast is received along with the "warning message content" from the CBC. The NG-RAN is also responsible for providing indication to the UE that the warning notification is being broadcast. The "warning message content" received by the NG-RAN contains an instance of the warning notification. Depending on the size, NG-RAN may segment the warning message before sending it over the radio interface.