3GPP TSG-RAN WG2#99	Tdoc R2-1708164

Berlin, Germany, 21st – 25th August 2017	(revision of R2-1706580)

Agenda Item:	10.4.1.5.1

Source:	Ericsson

Title:	System information handling in EN-DC

Document for:	Discussion, Decision

# Introduction

NR system information structure and content for standalone operation is presented in [1]. In this paper, we discuss which of this information is needed during non-standalone operation (EN-DC), and how this information is delivered to the UE. It is important to identify which IEs that are needed for EN-DC operation, since some of these need to be ready for the December drop.

# Discussion

A baseline for NR-SIB1 contents is proposed in [1], and repeated in table 1, including analysis for each field whether it is needed for EN-DC operation, or not.

Table 1: Baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

| SIB1 | Comment |  | NR location | Needed in EN-DC? |
| --- | --- | --- | --- | --- |
| plmn-IdentityList | Including a per PLMN indication of a cell reserved for operator use. |  | NR-SIB1 | No. UE gets this from the LTE SI. |
| trackingAreaCode |  |  | NR-SIB1 | No. UE gets this from the LTE SI. |
| cellIdentity |  |  | NR-SIB1 (already agreed to be part of minimum SI) | Not needed for the December drop, since it was agreed in RAN2#99 to not support ANR in the December drop. |
| cellBarred |  |  | NR-SIB1 (already agreed to be part of minimum SI) | No. UE will not camp on NR cell in EN-DC. |
| intraFreqReselection | Indicating whether intra-frequency reselection is allowed in barred cells. |  | NR-SIB1 (already agreed to be part of minimum SI) | No. UE will not camp on NR cell in EN-DC. |
| CSG related information csg-Indication csg-Identity |  |  | CSG is not supported in NR in Rel-15 | No. |
| Cell Selection Information q-RxLevMin q-RxLevMinOffset |  |  | NR-SIB1 (already agreed to be part of minimum SI) | No. UE will not camp on NR cell in EN-DC. |
| p-Max |  |  | NR-SIB1 (to be verified by RAN1) | Yes, this is probably needed. Need to be verified with RAN1 |
| freqBandIndicator |  |  | NR-SIB1 | Yes. This information is needed, for UE to know the NR band information. |
| schedulingInfoList | List of SI periodicity and mapping of SIBs to SI messages |  | NR-SIB1 | No. UE does not have to read further SI from NR cell. |
| tdd-Config |  |  | To be verified by RAN1 (might be not needed due to dynamic TDD?). | Yes, if RAN1 confirms this IE is needed. |
| WindowLength |  |  | NR-SIB1 | No. UE does not have to read further SI from NR cell. |
| systemInfoValueTag |  |  | NR-SIB1 | No. UE does not have to read further SI from NR cell. |
| SIB2 |  | Comment | NR location |  |
| Access barring information |  |  | NR-SIB1 | No, access barring will be based on LTE. |
| Radio Resource Config Common |  |  | NR-SIB1 (at least for parameters needed for initial access). | Yes, UE needs RACH configuration to access the NR cell. |
| UE Timers and Constants |  |  | NR-SIB2+ | No. These will be received by UE dedicated values in RRC connection |
| Frequency information |  | UL carrier frequency, ARFCN, bandwidth and additional spectrum mask | NR-SIB1 | Yes. This information is needed, for UE to know the NR band information. |
| MBSFN configuration |  |  | NR-SIB2+ (if needed) | ? |
| timeAlignmentTimerCommon |  |  | NR-SIB2+ | No. This will be received by UE dedicated values in RRC connection |

To ensure timely completion of NR SI parameters needed for EN-DC operation in the December drop, it is proposed to focus completion on the parameters identified in table 1 as needed for EN-DC operation, namely p-Max, freqBandIndicator, tdd-Config (if needed), Radio Resource Config Common and Frequency Band Information. All these parameters can be signalled to the UE in RRCConnectionReconfiguration, when EN-DC is configured.

Work should focus on completing NR SIB1 IEs needed during EN-DC operation, namely p-Max, freqBandIndicator, tdd-Config (if needed), Radio Resource Config Common and Frequency Band Information.

The above information will be provided to the UE as embedded NR RRC in LTE RRCConnectionReconfiguration, during SCG addition, i.e. NR SIB1 broadcast is not needed in the NR cell.

The content of other NR SIBs has not yet been agreed, but looking at the LTE equivalents, they include information used for cell re-selection (inter- or intra-frequency, intra- or interRAT), ETWS, MBMS, etc. This kind of information will not be needed from the NR cell in EN-DC operation. Instead, UEs will use the corresponding LTE information.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Work should focus on completing NR SIB1 IEs needed during EN-DC operation, namely p-Max, freqBandIndicator, tdd-Config (if needed), Radio Resource Config Common and Frequency Band Information.

Proposal 2	The above information will be provided to the UE as embedded NR RRC in LTE RRCConnectionReconfiguration, during SCG addition, i.e. NR SIB1 broadcast is not needed in the NR cell.

# References

R2-1708162, System information structure and contents, Ericsson