3GPP TSG-RAN WG2 #99	Tdoc R2-1708182

Berlin, Germany, 21st – 25th August 2017	Resubmission of R2-1707116

Agenda Item:	10.2.2.4

Source:	Ericsson

Title:	Activation and Deactivation time of Secondary Cells

Document for:	Discussion, Decision

# Introduction

For UEs in LTE with CA configured, an activation/deactivation mechanism of SCells is supported to enable energy savings for the UE. The activation/deactivation is done either by MAC CE or by the use of a timer.

At RAN2#98 the following were agreed:

Agreements

- Activation/deactivation is supported for NR CA

- As in LTE, Activation/deactivation is controlled per CC by explicit indication and implicit mechanism for NR CA

- Configuration of implicit deactivation mechanism is per CC for NR CA

This paper aims to discuss the activation/deactivation mechanism in NR and tries to identify possible improvements.

# Discussion

In LTE if the UE is configured with one or more SCells, the eNB may activate and deactivate the configured SCells upon demand. After the SCell is Configured, the SCell is still in deactivated state. In order for the UE to receive data (PDSCH/PDCCH) on the SCell, it must be activated. It is proposed to reuse this default state for NR.

The SCell will be in deactivated state after configuration until it is activated by the NW.

## Activation/Deactivation using MAC CE

If an LTE UE receives an Activation/Deactivation MAC CE in subframe #n deactivating the SCell, certain delay requirements in [1] states that the UE shall deactivate the SCell no later than in subframe #n+8 regardless of the number of SCells the deactivating command applies to. In NR due to improved processing capabilities it is our belief these delay requirements can be reduced to enable faster deactivation of SCells in NR. Further, in [3], CA enhancements have been discussed where the activation delay is one issue discussed

RAN2 should agree on having this delay requirement reduced but leave it up to RAN4 to agree on the actual numbers.

Acknowledge that from a RAN2 point of view it is beneficial to reduce the delay to enable faster deactivation of SCells in NR.

Similar, if an LTE UE receives an Activation/Deactivation MAC CE in subframe #n activating the SCell, the delay requirements in [1] depends on if certain conditions are met in each SCell, if PUCCH is configured and also the number of SCells configured. In NR due to improved processing capabilities it is our belief these delay requirements can be reduced to enable faster activation of SCells in NR. RAN2 should agree on having these delay requirements reduced but leave it up to RAN4 to agree on the actual numbers.

Acknowledge that from a RAN2 point of view it is beneficial to reduce the delay to enable faster activation of SCells in NR.

RAN2 should send an LS to RAN4 asking them to study the delay requirements for SCell activation and deactivation.

# Conclusion

We have the following proposals:

Proposal 1	The SCell will be in deactivated state after configuration until it is activated by the NW.

Proposal 2	Acknowledge that from a RAN2 point of view it is beneficial to reduce the delay to enable faster deactivation of SCells in NR.

Proposal 3	Acknowledge that from a RAN2 point of view it is beneficial to reduce the delay to enable faster activation of SCells in NR.

Proposal 4	RAN2 should send an LS to RAN4 asking them to study the delay requirements for SCell activation and deactivation.

- References

3GPP TS 36.133, Requirements for support of radio resource management

3GPP TS 36.121, Medium Access Control (MAC) protocol specification

RP-170805, New WID on Enhancing CA Utilization, Nokia et.al., Dubrovnik, Croatia, March 2017