3GPP TSG-RAN WG2 Meeting #99	R2-1708234

Berlin, Germany, 21 – 25 Aug 2017

Agenda item:	9.13.1

Source: 	Qualcomm Incorporated

Title: 	DL early data transmission

Document for:     Discussion/Decision

# Introduction

The WI description for further NB-IoT enhanced is given in [1]. One of the objectives of the WI is:

Further latency and power consumption reduction - Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed. [RAN2, RAN1, RAN3]

In this contribution, we discuss some aspects of supporting early DL data transmission for UEs in IDLE mode. Note, RAN1 has indicated in their LS in 0 that it is beneficial to support early data transmission for NB-IoT/BL/CE UEs with any CE level or coverage.

# Background

Many NB-IoT applications involve relatively infrequent exchange of small amounts of data.  For example, metering, alarm etc. are expected to produce small UL data whereas queries, notifications of update, command to actuators, etc. generates small DL data transmissions.

Full RRC Connection establishment procedure for Idle UEs involving RA procedure has a large overhead. For example, for MT data, the sequence of messages involves Paging, Msg1 (NPRACH preamble), Msg2 (RAR), Msg3 (RRC Connection Request or RRC Connection Re-establishment Request or RRC Connection Resume Request etc. depending on the reason for RA procedure), Msg4 (early contention resolution, RRC Connection Setup etc.), Msg5 (RRC Connection Setup Complete), and finally Msg6 which can be used for the DL data. This involves 6 message exchanges before actual payload transmission for a small DL data. This is a large overhead for applications that transmit downlink data that fits into one TBS.

Significant overhead to resume or setup RRC connection for transferring small DL data (e.g., one MAC block).

# Random Access Procedure

A brief overview of contention based random access procedure is given in [2].

Figure 10.1.5.1-1 in [2]: Contention based Random Access Procedure

For NB-IoT UEs, this procedure is performed for (among other reasons), initial access from RRC_IDLE and DL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised".

Normal DL/UL transmission can take place after the random access procedure. Therefore, there are a large number of message exchanges before the actual payload transmission, even for very small and infrequent payload.

The number of steps involved in establishing RRC connection for downlink is the same regardless of the volume of data to be transferred.

It should be noted that for each message exchange, there is not only communication overhead, but also inherent latency involved. Therefore, it is reasonable to assume that fewer message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Fewer number of message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Note that the following discussion assumes the UE is in IDLE mode at the time of the initiation of data transmission.

# Downlink Data transmission

For a MT call for IDLE UE, the first downlink message is paging from the network. Once the UE receives paging message (i.e., a paging record corresponding to UE’s NAS identity is present), it initiates RA procedure. Therefore, the early data transmission in DL can be achieved by:

- Including small DL data as NAS message in paging message

- Small DL data after paging

- Small DL data after paging and NPRACH preamble

For reference, existing MT data transport using NAS in CP CIoT EPS Optimization is shown in the Figure 1 below (from TS 23.401). In the call flow, step 5 can potentially include all Msg1-Msg5 from legacy RACH procedure. Our objective here is to optimize steps 5 + 14 as well as move step 4 and part of 5 to later, and skip steps 6 to 11 in current procedure. The proposed (simplified) call flows for different options are discussed below.

Figure 1 MT Data transport in NAS PDUs (Figure 5.3.4B.3-1 in TS 23.401).

Note that in Figure 1, the steps with dashed-lines starting at step 16 represent the procedure assuming UL data exists at the UE. In case of no UL data, all steps with dashed-lines after step 15 would be skipped. Note also that, at the end of this procedure, UE would be in RRC_CONNECTED state, and further steps are necessary for RRC connection release which includes multiple signaling steps not shown here before the UE can move to IDLE.

## DL data in paging

It should be possible to include additional payload in paging record for the UE. The payload would be included as a NAS message. The advantage is that there is no need to even have a full RA procedure. Only Msg1 transmission may be needed which can be used as ACK for DL data. Furthermore, no significant RAN1 impact is foreseen with such a solution. However, the challenge is that the paging load may significantly increase since each paging record could be large. The proposed call flow with small DL data in paging is shown below in Figure 2.

Figure 2 DL data in paging

The steps are briefly described below:

1) S-GW receives DL data. It needs to know that the UE for which it is directed supports small data over paging.

2) The S-GW sends a “small data transfer request” to the MME with the data to transmit. The MME buffers this data.

Note, if the “data” is not included in step 2, it may be sent after 3a separately (which corresponds to step 11 in existing procedure).

3a) In case of MME overloading, it can reject the request. Otherwise it confirms it.

A) The MME encapsulates the data in a NAS PDU and prepares a paging message.

Note, this corresponds to step 12 in current procedure.

3b) MME sends the paging message (with data) to the eNBs in the tracking area.

Note, this can be viewed as steps 3 and 13 combined in the current procedure. Also note, paging record includes ue-Identity which can be s-TMSI or IMSI. Additionally, since the paging record may be sent prior to AS security activation, the NAS PDU needs to be protected by NAS security.

B)  Upon reception of the paging message, the eNB determines a NPRACH resource for contention-free random access and include it in the paging message.

4)  The UE monitors for paging. If the UE decodes paging containing DL data, the UE delivers the PDU to NAS.

Note, this corresponds to existing steps 4 and 14 combined.

C) The eNB monitors for NPRACH transmission from the UE.

5) If the DL data reception was successful in step 4, the UE transmits the dedicated NPRACH preamble which indicates ACK of data.

Note that steps B, (part of) 4, C and 5 together correspond to step 14 in current procedure. In addition, step 5 in proposed procedure also corresponds to part of step 5 in current procedure.

6) eNB ACKs the MME about successful transfer of DL data, which corresponds to step 15 in existing flow.

Note that in the proposed method, the UE starts from IDLE state, and at the end of the procedure continues to be in RRC_IDLE, thereby avoiding the costly procedure of RRC connection release.

## DL data after paging

Including the actual payload in paging message may increase the size of paging message and hence the paging load significantly. This means the UEs for which the paging is not intended may also need to decode the large paging messages. Therefore, it may be reasonable to include a RNTI information in the paging record instead of the actual payload. After receiving such information, the UE monitors NPDCCH scrambled with a given RNTI. The NPDSCH contains a NAS PDU. This is illustrated in the following figure.

Figure 3 DL data after paging

## DL data after NPRACH preamble

The main issue with the above solutions is that all the eNBs where the UE will be paged will need to allocate large NPDSCH (either to send paging message with payload, or send payload in a separate PDSCH assignment). This can lead to significant resource wastage at the eNB as well as high backhaul load between MME and eNBs (since the payload needs to be sent to all eNBs in the paging area). To overcome this, MME can indicate to the eNBs in the paging area that small data tx is required for a certain UE. Paging record for the UE can indicate dedicated NPRACH resource (preamble) and RNTI. The UE after receiving the page uses the NPRACH resource to transmit the dedicated preamble. Only the eNB that receives the NPRACH preamble needs to retrieve the NAS PDU from MME and respond with downlink transmission using the RNTI. This way while NPRACH preamble and RNTI would be reserved in all the eNBs that page the UE, only one eNB will use downlink NPDSCH resources for the actual payload. This is illustrated in the following figure.

Figure 4 DL data after NPRACH preamble

## Summary

In our understanding, among the options above, the third option (DL data after PRACH preamble) is the most efficient one. In addition, above methods can be achieved with enhancements mostly handled by RAN2 keeping the impact minimal to RAN1/RAN3 and potentially an LS-exchange with SA3. This is also in-line with the objective where it is noted that the sub item is owned by the WGs in this order: “RAN2, RAN1, RAN3”. Therefore, we propose:

Early DL data transmission with paging enhancements is supported (based on option 3).

Send LS to other WGs with following actions:

- To SA2 requesting for their feedback on architectural implications of early DL data transmission and whether use of early data transfer needs to be negotiated at NAS just like the use of other CIoT optimisations.

- To RAN3 requesting them to consider changes required to S1 interface.

- To CT1 requesting them to consider changes required to their specifications to support early DL data transmission.

# Conclusion

In this contribution, we discussed some aspects of support for DL data transmission with paging enhancements. Based on the above discussion, we made the following observations:

Observation 1.	Significant overhead to resume or setup RRC connection for transferring small DL data (e.g., one MAC block).

Observation 2.	The number of steps involved in establishing RRC connection for downlink is the same regardless of the volume of data to be transferred.

Observation 3.	Fewer number of message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Based on the discussion and observations, we propose:

Proposal 1.	Early DL data transmission with paging enhancements is supported (based on option 3).

Proposal 2.	Send LS to other WGs with following actions:

 To SA2 requesting for their feedback on architectural implications of early DL data transmission and whether use of early data transfer needs to be negotiated at NAS just like the use of other CIoT optimisations.

 To RAN3 requesting them to consider changes required to S1 interface.

 To CT1 requesting them to consider changes required to their specifications to support early DL data transmission.

# References

[1] RP-170852, New WID on Further NB-IoT enhancements, Huawei, HiSilicon, Neul

[2] 3GPP TS 36.300, Overall description, Stage 2 specifications

[3] 3GPP TS 36.321, MAC specifications

[4] 3GPP TR 36.912, Feasibility study for LTE-Advanced

[5] 3GPP TS 23.401, E-UTRAN Stage 2

[6] R2-1707615/R1-1709835, LS on early data transmission in MTC and NB-IoT in Rel-15, RAN2#99, August 2017