3GPP TSG-RAN WG2 Meeting #99	R2-1708239

Berlin, Germany, 21 – 25 Aug 2017

Agenda item:	9.14.2

Source: 	Qualcomm Incorporated

Title: 	UL early data transmission

Document for:     Discussion/Decision

# Introduction

The WI description for even further enhanced MTC for LTE (efeMTC) is given in [1]. One of the objectives of the WI is:

Improved latency: - Support early data transmission [RAN2 lead, RAN1, RAN3] - Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure (after PRACH transmission and before the RRC connection setup is completed) at least in the RRC Suspend/Resume case.

In this contribution, we discuss some aspects of supporting UL data transmission during the RA procedure for UEs in IDLE mode. Note, RAN1 has indicated in their LS in [6] that it is beneficial to support early data transmission for BL/CE UEs with any CE level or coverage.

# Background

Many MTC applications involve relatively infrequent exchange of small amounts of data (e.g., one uplink packet).  For example, metering, alarm etc. are expected to produce small UL data whereas queries, notifications of update, command to actuators, etc. generates small DL data transmissions.

Full RRC Connection establishment procedure for Idle UEs involving RA procedure has a large overhead. For example, the sequence of messages involves Msg1 (PRACH preamble), Msg2 (RAR), Msg3 (RRC Connection Request or RRC Connection Re-establishment Request or RRC Connection Resume Request etc. depending on the reason for RA procedure), Msg4 (early contention resolution, RRC Connection Setup etc.), and finally Msg5 which can be used for the UL data (unless SR/BSR is required before actual payload transmission). This involves 5 or more messages for UL data before actual payload transmission. This is a large overhead for applications that transmit uplink data that fits into one TBS.

For the case of RRC connection setup (i.e. CP CIoT EPS Opt), Figure 1 shows the current procedure (reconstructed from TS 23.401 [5]) while for the case of RRC suspend/resume (i.e., UP CIoT EPS Opt) Figure 2 shows the current procedure (reconstructed from TS 36.300 [2]).

Figure 1 RRC Connection Establishment (Adapted from TS 23.401 Fig. 5.3.4B.2-1)

Note that in Figure 1, the steps with dashed-lines represent the procedure assuming DL data exists at the SGW for the concerned UE. In case of no DL data, step 7 would be a different message (Connection Establishment Indication) and other steps with dashed-lines would be skipped.

Figure 2 RRC Connection Resume procedure (TS 36.300)

Note also that, for both Figure 1 and Figure 2, RRC connection release procedure (step 12) includes multiple signaling steps which are not shown here.

As can be seen from Figure 1 and Figure 2 there is substantial overhead for applications that require very small amount of data to transfer (e.g. 1 MAC block) for a UE in IDLE state.

Significant overhead to resume or setup RRC connection for transferring just one MAC block.

# Random Access Procedure

A brief overview of contention based random access procedure is given in [2].

Figure 10.1.5.1-1 in [2]: Contention based Random Access Procedure

For eMTC/CE UEs, this procedure is performed for:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	Handover (if contention-free RACH fails)

-	DL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised".

-	UL data arrival during RRC_CONNECTED requiring random access procedure, e.g. when UL synchronisation status is "non-synchronised" or there are no PUCCH resources for SR available.

- 	establishing time alignment for the corresponding sTAG etc.

Normal DL/UL transmission can take place after the random access procedure. Therefore, there are a large number of message exchanges before the actual payload transmission, even for very small and infrequent payload.

The number of steps involved in establishing RRC connection for uplink is the same regardless of the volume of data to be transferred.

It should be noted that for each message exchange, there is not only communication overhead, but also inherent latency involved. For example, Table 1 shows typical values of delays for each of these steps in legacy LTE (extracted from [4]). The values assume error-free first transmissions without repetitions or HARQ delays. Since eMTC involves repetitions of certain messages, the delays are expected to be much larger compared to the values in the table below.

Table 1: Uplink initiated dormant to active transition for unsynchronized UE (error free) (Table B.1.2.1.2-1 in [4])

| Component | Description | Minimum [ms] | Average [ms] |
| --- | --- | --- | --- |
| 1 | Average delay due to RACH scheduling period | 0.5 | 2.5 |
| 2 | RACH Preamble | 1 | 1 |
| 3 | Preamble detection and transmission of RA response (Time between the end of RACH transmission and UE’s reception of scheduling grant and timing adj.) | 3 | 5 |
| 4 | UE Processing Delay (decoding of scheduling grant and timing alignment + L1 encoding of UL data) | 5 | 5 |
| 5 | Transmission of UL data | 1 | 1 |
|  | Total delay | 10.5 | 14.5 |

Therefore, it is reasonable to assume that fewer message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Fewer number of message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Note that the following discussion assumes the UE is in IDLE mode at the time of the initiation of data transmission.

# Uplink data transmission

In the Figure 1 and Figure 2 shown in section 3, the first UL signal from UE is Msg1, whereas the first scheduled UL transmission is Msg3. Therefore, it is natural that “early” data transmission in UL can target to transmit payload in Msg1 or Msg3 instead of Msg5 or later.

## UL data in Msg1

Uplink data transmission without having dedicated grant has been previously discussed in RAN1/2 for CONNECTED mode UEs where it was generally referred to as “contention-based” UL. In such case, the basic idea would be to allow grant sharing among multiple UEs, assuming that the UL is synchronized.

However, it should be noted that there are several challenges, especially for UEs in IDLE mode. For example:

- UL synchronization: The UE may not have uplink timing sync, so a new waveform or new procedures are needed.

  - Alt 1: Define a new waveform to be used as “training sequence” to acquire UL sync. This approach may not be preferable for LTE evolution as this involves significant PHY changes.

  - Alt 2: Use LTE PRACH preamble as training sequence. The idea would be to include payload along with the PRACH preamble (e.g. PRACH followed by PUSCH).

  - Alt 3: The UE remembers the TA from the last connection, such that it remains orthogonal to other users. This assumes that the UE has not “drifted” since the last UL transmission, therefore it may not be practical for infrequent small data transmission, as the UE is likely to have been idle for a long time.

- Channel adaptation: The initial UL MCS also needs to be predetermined. In addition, the initial CE levels (e.g. repetition levels) need to be determined.

  - It may be possible for the UE to choose between different initial MCS’s/CE levels based on “history”, however in this case, signaling on the UL to indicate this MCS to the eNB is required.

    - This may be a separate UL control signal

    - This may be implicitly indicated by partitioning the PRACH sequences.

- Power control: Initial tx power may be blindly selected (open loop), or selected based on history.

- Contention resolution: the UL tx needs to include some form of UE ID also in Msg1 for contention resolution purposes.

- Payload Data: The payload would be a NAS message as DRB/PDCP/RLC are not yet established for this data since this is transmission without RRC connection. The payload would have to be secured by NAS (Integrity and ciphering).

Figure 3 below shows the components of an UL transmission using PRACH preamble along with payload data in Msg1.

| PRACH preamble | UE ID | Uplink Control Information | Payload Data (NAS PDU) |
| --- | --- | --- | --- |

Figure 3 UL data transmission in Msg1

Since the UL synchronization is not guaranteed, the fact that Msg1 is much bigger compared to simply the preamble (due to addition of UE-ID, other control information and the payload data) increases the probability of collision and therefore reduction in resource efficiency significantly. Additionally, the introduction of this feature requires non-trivial changes at the physical layer due to the need of decoding long packets on a shared channel without uplink synchronization.

## UL data in Msg3

The transmission of Msg3 is performed on the initial UL grant provided by RAR, therefore the size of the transport block depends on the grant. When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Respons [3].

Msg3 uses HARQ. According to current specification, Msg3 conveys at least NAS UE identifier for initial access, but no NAS message [2]. Msg3 transmission is done using a separate Msg3 buffer (which has higher priority than the UL buffer). Not only Msg3 uses HARQ but UE (MAC) can retransmit the message incase it does not receive MAC level response from eNB (e.g. UE does not receive Msg4 and this leads to contention resolution failure and UE (MAC) can re-attempt access from idle).

The RA procedure can be enhanced with minimal specification impacts to support UL data transmission in Msg3. The payload may be included as CCCH SDU.

### CP CIoT EPS Optimisation

A call flow allowing data transmission in Msg3 as NAS PDU is shown in Figure 4.

Note: this procedure is not applicable for UE that needs to perform NAS procedures that require more than one packet to transmit.

Figure 4 Proposed CP CIoT EPS Opt data transmission in Msg3

The steps are briefly described below:

A. Resource determination: eNB decides to allow transmission of small data packet sizes (e.g. 10 bytes to 50 bytes) without establishing full RRC connection. It allocates PRACH resources for this purpose (resources may also be different for different CE levels).

0. eNB announces the resources via system information broadcast (SIB)

B. The UE selects a PRACH resource based on the announced resources and the amount of data to transmit. The selection may be based on random selection from the corresponding PRACH pool. If enough resources are available as the traffic frequency may be very low, a dedicated allocation may be possible (described later).

Note: If no PRACH resource is broadcasted for data in Msg3 procedure, or UE intends to perform NAS signaling procedure or UE intends to send/receive more data than possible in one MAC block then UE uses legacy RRC connection establishment/resumption procedure.

1. UE transmits the PRACH preamble (Msg1).

2. The UE receives RAR. In current LTE, RAR may contain uplink grant, and TA (in addition to Temporary C-RNTI etc.). To enable Msg3+data, RAR may be updated to include power control information. Otherwise, the UE may use open-loop power control (i.e., UE decides on transmit power itself).

3. UE transmits Msg3 with UEID (S-TMSI) + NAS PDU using the UL grant indicated in RAR. UE takes into account the power control information from RAR, if included. UE starts a contention resolution timer after this step.

4. eNB selects the MME based on the S-TMSI and forwards the NAS PDU to the MME. eNB may signal to MME that there is only one uplink NAS PDU. If DL data is available for the UE, the SGW provides the downlink data to the MME, which forwards it as NAS PDU to the eNB to be delivered to UE. If the eNB has indicated that there is only one UL NAS PDU, in response the MME should close the S1-AP connection after forwarding any downlink NAS PDU. This indication may also be used by MME to prioritize processing of the UL data and expedite step 6.

7. Network responds with Msg4 which confirms reception of Msg3, completes contention resolution and includes NAS PDU (if necessary). If NAS PDU is included then NAS will be able to confirm if it is communicating with a valid network.

Note: Current NAS specification does not mandate MME to respond with anything to confirm reception of NAS PDU hence the reason why NAS PDU is optional.

### UP CIoT EPS Optimisation

If the UE has been previously suspended, the UEID can be the Resume ID and message similar to RRCConnectionResumeRequest can be used but including application data. New resume cause value can be defined for Data in Msg3. If this new cause value is signaled, the procedure excludes actual RRC resume but instead eNB only forwards the data to MME. Alternatively, a new message could be defined to carry a combination of uncyphered and ciphered payload as depicted in Figure 5.

Figure 5 Proposed UP CIoT EPS Opt data transmission in Msg3

Key difference with resume compared to CP RRC connection is that:

- UE needs to apply security to data PDU carried by Msg3. Integrity could be applied to entire Msg3 but only data IE can be ciphered.

- UE needs to have the security keys to use for integrity and (optionally) ciphering. This requires UE to be provided with NextHopChainingCount as well as resumeID during suspension. This needs to be confirmed with SA3.

- Preferably HARQ transmitted once AS security passes for the received AS message.

Procedures for step 0, 1 and 2 are same as for CP proposal.

At step 3, UE uses security parameters based on the NextHopChainingCount provided during last suspension. UE ciphers (if required) data PDU and computes integrity key (over entire RRC Connectionless Resume Request message). In this case the KeNB, KRRCint and KRRCenc security parameters are used for MAC calculation and optional ciphering.

Note: resumeID, resumeCause and shortResumeMAC-I are not ciphered.

3. UE transmits connectionless resume message. The eNB decodes the RRC message, fetches UE context and verifies integrity. If successful, then it deciphers data (if required).

4-6. eNB triggers MME to resume connection. MME configures/resumes bearers and confirms to eNB.

7. The eNB forwards data PDU to the SGW.

8. The eNB sends the contention resolution message with UE ID (Msg4), NextHopChaingingCount and resumeID to UE. This message is integrity protected.

Note: steps 7 and 8 may happen in any order.

### Size of UL grant for Msg3

Currently the minimum size of Msg3 UL grant is defined (56bits for legacy, 88bits for NB-IoT). This is currently captured in 36.321 as follows.

NOTE:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

There is no restriction on eNB to provide larger grants to the UE. However, this involves tradeoff based on likelihood of collision of Msg3, and the tradeoff becomes more important where large number of repetitions are involved because the resource (a large block of it) is blocked for multiple of TTIs in NB-IoT/eMTC/CE.

The eNB may be able to use some statistical models to guess/estimate the collision of preambles with the knowledge of total number of preambles, and expected number of “active” UEs (arrival) based on traffic pattern.  The eNB may use the estimated collision probability to decide between providing larger grants for “Msg3+data” vs smaller grant for legacy Msg3.

The UE can differentiate between grants for Msg3+data vs grant for legacy Msg3 based on size of the grant implicitly (if grant > threshold, then it is for Msg3+data), or based on explicit flag in the grant. This may be up to the UE to decide how to use this grant (for RRC connection request with or without data in Msg3).

### Putting data payload in Msg3 grant

Currently, there is no restriction from MAC specification point of view that restricts using grants obtained in RAR to send actual payload (it can already carry higher layer i.e. RRC payload). However, there is a separate Msg3 buffer (which has higher priority than the UL buffer).

- The payload may be included as CCCH SDU as NAS PDU.

- In the current NAS procedure, NAS COUNT (24 bit) and NAS SN (8 LSBs of NAS COUNT) are used to keep track of NAS PDUs (TS 24301, 4.4.3.1). Only NAS SN is exchanged over-the-air, the overflow counter (16 MSBs of COUNT) is not exchanged, but maintained. This is similar to PDCP COUNT/SN. After each new or retransmitted outbound security protected NAS message carrying application data, the sender increments the NAS COUNT (as per legacy DoNAS procedure).

- Msg3 buffer is flushed when RACH fails. So, with existing specs, the payload data would be lost if RACH fails due to Msg3 collision, for example.

- Currently, TTI bundling is not applicable for Msg3, but UL repetition is used.

### Implications for RRC, PDCP, and MAC

For early data transmission with CP enhancement (Figure 4), DRB/PDCP/RLC are not established for the data transmission since the transmission happens without RRC connection and using control plane RRC messaging. UE remains in RRC_IDLE state.

For early data transmission with UP enhancement (Figure 5), DRB/PDCP contexts are in suspended state. There is possibility to use PDCP ciphering for the data. However, for the eNB to decipher the PDU, it needs to wait until UE context is resumed/available. From UE perspective, UE remains in RRC_IDLE state as the data transmission happens without RRC connection resumption. From CN perspective, UE’s context is temporarily resumed and suspended again.

### Performance evaluation

A simple evaluation of Rx/Tx bits for the CP CIoT EPS optimisation is shown below. Note that the following tables are based on the message/channel names and sizes for NB-IoT; however, given that eMTC may involve some additional overhead (larger MPDCCH with hopping, PDCP overhead in RRC message), we expect that the savings from proposed method can be at least same as or better than NB-IoT. Table 2 shows the message sizes as per the current specification and Table 3 shows the sizes for the design proposed in this paper.

Table 2 Total bits exchanged between eNB and UE as per current design

| Message | Channel | Size (bits) |
| --- | --- | --- |
| RRC Connection Request | NPUSCH | 88 |
| DCI | NPDCCH | 23 |
| RRC Connection Setup | NPDSCH | 64 |
| HARQ | NPUCCH | 1 |
| DCI | NPDCCH | 23 |
| RRC Connection Setup Complete (Note 1) | NPUSCH | 530 |
| RRC Connection Release | See [7] | 155 |
| Total (bits) |  | 852 |

Note 1: Assumes NAS PDU size is 500 bits.

Table 3 Total bits exchanged between eNB and UE as per proposed design

| Message | Channel | Size (bits) |
| --- | --- | --- |
| RRC Connectionless Request | NPUSCH | 536 |
| DCI | NPDCCH | 23 |
| RRC Connectionless Confirm (Note 2) | NPDSCH | 40 |
| HARQ | NPUCCH | 1 |
| Total (bits) |  | 600 |

Note 2: Assume same size as RRC Connection Release Message

In this computation, the first two steps (PRACH and RAR) are not considered.

This simple evaluation shows a saving of around 30% in the number of bits exchanged between eNB and UE. The total time UE is in non-idle state is considerably reduced with the new design compared to legacy design hence the overall power saving is likely to be considerable for applications that only send 1 packet (TBS) on the uplink every time it needs to report something infrequently.

The savings in terms of both the power consumption and overhead reduction with the proposed early UL data transmission solutions is significant.

### Contention-free Msg3

Given that the expected data arrival may be infrequent, it may be possible for the eNB to allocate contention-free PRACH to each UE. A predefined timing, frequency, periodicity or pattern may be defined for each UE to have access to a dedicated PRACH preamble resource. The predefined resource may be present during a predefined time window to increase the probability of successful acquisition and usage of such resource.

If such method is considered, it can further reduce delay because the contention resolution (step 5) may not be needed. In such scenarios, a separate ACK indication (1-bit) using a separate control channel instead of a contention resolution MAC PDU in shared data channel may be introduced.

This method requires that the network has knowledge of UEs’ traffic patterns. The UE may provide this information to the network at the time of initial attach or it may indicate how long it will be until its next expected transmission. The network may indicate the future dedicated PRACH resource (or its index) in one of the steps of current session.

If there is a concern that the network may not have such level of resources to provide contention free RA, a hybrid approach can be considered where the eNB may allocate a predefined “time window” within which the UE performs a contention based RA. This can be beneficial when a large number of UEs are present in the network and the network wants to “smooth” out the possible peaks of access requests by distributing the UEs across different time windows.

### Summary

Based on the above discussion, it is clear that UL data transmission in Msg1 requires a lot of RAN1/RAN2 time whereas UL data transmission in Msg3 can be achieved with enhancements mostly handled by RAN2 keeping the impact minimal to RAN1/RAN3 and potentially an LS-exchange with other WGs. This is also in-line with the objective where it is noted that the sub item is owned by the WGs: “RAN2 lead, RAN1, RAN3”. Therefore, we propose:

Early UL data transmission in Msg3 for IDLE mode UE will be supported for control plane CIoT EPS optimisation.

Early UL data transmission in Msg3 for IDLE mode UE will be supported for user plane CIoT EPS optimisation.

Send an LS to other relevant WGs with following actions:

- to SA2 to request for their feedback on architectural implications of data transmission in Msg3 and whether use of early data transfer needs to be negotiated at NAS just like the use of other CIoT optimisations.

- to RAN3 requesting them to consider changes required to S1 interface.

- to CT1 requesting them to consider changes required in their specifications to support data transmission in Msg3.

- to SA3 requesting feedback on UE and eNB storing nextHopChainingCount during RRC idle mode and using this for security protection of Msg3.

# Conclusion

In this contribution, we discussed some aspects of support for UL data transmission during the RA procedure. Based on the above discussion, we made the following observations:

Observation 1.	Significant overhead to resume or setup RRC connection for transferring just one MAC block.

Observation 2.	The number of steps involved in establishing RRC connection for uplink is the same regardless of the volume of data to be transferred.

Observation 3.	Fewer number of message exchanges leads to lower latency, lower power consumption and lower radio resource usage.

Observation 4.	The savings in terms of both the power consumption and overhead reduction with the proposed early UL data transmission solutions is significant.

Based on the discussion and observations, we propose:

Proposal 1.	Early UL data transmission in Msg3 for IDLE mode UE will be supported for control plane CIoT EPS optimisation.

Proposal 2.	Early UL data transmission in Msg3 for IDLE mode UE will be supported for user plane CIoT EPS optimisation.

Proposal 3.	Send an LS to other relevant WGs with following actions:

	to SA2 to request for their feedback on architectural implications of data transmission in Msg3 and whether use of early data transfer needs to be negotiated at NAS just like the use of other CIoT optimisations.

 to RAN3 requesting them to consider changes required to S1 interface.

 to CT1 requesting them to consider changes required in their specifications to support data transmission in Msg3.

 to SA3 requesting feedback on UE and eNB storing nextHopChainingCount during RRC idle mode and using this for security protection of Msg3.

# References

- RP-170732, New WID on Even further enhanced MTC for LTE, Ericsson, Qualcomm

- 3GPP TS 36.300, Overall description, Stage 2 specifications

- 3GPP TS 36.321, MAC specifications

- 3GPP TR 36.912, Feasibility study for LTE-Advanced

- 3GPP TS 23.401, E-UTRAN Stage 2

- R2-1707615/R1-1709835, LS on early data transmission in MTC and NB-IoT in Rel-15, RAN2#99, August 2017

- R2-1708620, NB-IoT RRC connection release enhancements, Qualcomm, RAN2#99, August 2017.