3GPP TSG-RAN WG2 Meeting #99	       R2-1708268

Berlin, Germany, 21st - 25th August, 2017

Agenda Item:	10.3.2.1

Souce:	MediaTek Inc. (Rapporteur)

Title:	Text Proposal for RLC AM Polling Mechanism

Document for:		Discussion and decision

# 1 Introduction

In RAN2#98, we decided to adopt LTE’s RLC AM polling mechanism for NR.

-    RLC polling mechanisms are the same as in the LTE.   No additional enhancements are needed.

However the current draft TS 38.322 is missing text related to when a POLL bit is included. In this document, we provide a text proposal to include such text.

Proposal 1: RAN2 is requested to endorse the TP provided in the Annex for inclusion in TS 38.322.

# Annex: Text Proposal

5.2.2. Polling

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.2.2.1	Transmission of a AMD PDU

Upon assembly of a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte;

-	include a poll in the AMD PDU as described below.

Upon assembly of an AMD PDU, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted AMD PDU awaiting for acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted AMD PDU awaiting for acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0;

-	restart t-PollRetransmit;