3GPP TSG-RAN WG2 #99	Tdoc R2-1708318

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	Default DRB Establishment

Document for:	Discussion, Decision

# Introduction

In RAN2 June Ad Hoc agreed:

Agreements:

- There is a need to tell the UE that it has to update the mapping rule.   For the AS reflective QoS it is up to the RAN to decide when to update the mapping rules.  FFS on the details of the header format.

- It is up to the RAN to decide when and which mechanism, explicit RRC re-configuration and/or AS reflective QoS, should be used to provide mapping information to the UE.

- A UE follows the latest QoS flow to DRB mapping information regardless of the fact whether it was explicit RRC or AS reflective QoS.

- Whether a SDAP header is present or not is configured by RRC per DRB

- The gNB indicates to UE using RRC signaling the default DRB for a PDU session.

- RAN decides and configures the default DRB for a PDU session.

This contribution discusses about the need for default DRB.

# Discussion

Figure 1: gNB and UE reference point representation.

### NAS configured QoS rule via N1 & N2

The 5CN uses NAS configured QoS rules, i.e. it uses N1 (between UE and AMF) signalling to convey QoS information, such as TFT, a QFI and possibly other QoS parameters to the UE. As the 5CN informs UE via N1, it appears straightforward to inform RAN via N2 (between gNB and AMF) about new flow being established. Based on the N2 signalling, the gNB may map the QFI to new DRB and inform UE via RRC about new QFI to DRB mapping. Alternatively, the gNB may inform UE via RRC to map the QFI to existing DRB for which it may or may not adjust its internal QoS handling parameters. This mechanism guarantees that UEs SDAP layer does not observe any UL packets without QFI-to-DRB mapping.

When QFI-to-DRB mapping is derived from N1 & N2 signaling, UE does not observe UL packets without QFI to DRB mapping.

### Using NAS level reflective Mapping

When 5CN uses NAS level reflective QoS, this implies that the first packets occurring on a new QFI are downlink packets. NAS level reflective QoS with previously unknown QFI may be only used with standardized QFI/5QI. In order to use non-standardized QFIs the 5CN must provide N2 signaling before first packets are marked with non-standard QFI. In both cases, gNB is able to decide to which DRB to map new QFI and inform UE either via reflective AS QoS or via RRC. This results that the SDAP layer of the UE does not observe any UL packets without QFI to DRB mapping.

When QFI-to-DRB mapping is derived from N3 header, UE does not observe UL packets without QFI to DRB mapping.

Based on observation 2 and 3, the following may be observed.

There are no obvious benefits of defining default DRB

The default DRB was defined previously in SA2 based on the assumption that the standardized QFIs may be used by 5CN or UE without N1 or N2 signaling. The situation is changed as the SA2 has agreed to have dedicated signaling only for non-3GPP access. This results that instead of having default DRB, the gNB is always able to determine QFI to DRB mapping based on N2 signaling from the CN. The mapping is then propagated to the UE with RRC signaling or AS-reflective QoS mechanism.

gNB provides always QFI to DRB mapping explicitly to UE either with dedicated RRC signaling or with AS reflective QoS mechanism.

Default DRB is not defined for PDU session.

# Conclusion

In section 2 we made the following observations:

Observation 1	When QFI-to-DRB mapping is derived from N1 & N2 signaling, UE does not observe UL packets without QFI to DRB mapping.

Observation 2	When QFI-to-DRB mapping is derived from N3 header, UE does not observe UL packets without QFI to DRB mapping.

Observation 3	There are no obvious benefits of defining default DRB

Based on the discussion in section 2 we propose the following:

Proposal 1	gNB provides always QFI to DRB mapping explicitly to UE either with dedicated RRC signaling or with AS reflective QoS mechanism.

# References

R2-1708325, SDAP configurations with NAS-Reflective QoS. Ericsson.  Berlin, Germany, 21st – 25th August 2017