3GPP TSG-RAN WG2 #99	Tdoc R2-1708319

Berlin, Germany, 21st – 25th August 2017

Title:	[DRAFT] Reply LS to SA2 on 5QIs for URLLC

Response to:	R2-1707655: LS on EPC QoS aspects for TSG RAN’s lower latency features

Release:	Rel-15

Work Item:	FS_NR_newRAT

Source:	Ericsson (to be RAN2)

To:	SA WG2

Contact Person:

#### Name:	Henrik Enbuske

E-mail Address:	henrik.enbuske@ericsson.com

1. Overall Description:

RAN WG2 received an LS R2-1707655 from SA2. This contribution proposes draft reply for the questions.

Q1 from SA2 LS: Input on the combination of packet one-way latencies over the eNodeB and the radio interface and the associated packet error loss rates that the RAN groups intend to meet. SA WG2 are interested in whether the values for NR and E-UTRA are different.

The minimum latency and the lowest reliability value supported is not yet defined by RAN1. For RAN2, it is however clear that NR will support one-way latency down to 0.5ms with reliability of 1-10-5 when packet size is 32 bytes. This is specified in TR 38.913 and with the latency and reliability values, NR is able to meet the corresponding IMT-2020 latency requirements. For LTE, there are latency and reliability work items in RAN1, which are not finished. However, it is expected by RAN2 that the LTE latency and reliability requirements match with the NR scenarios.

It is important to notice that the reliability and latency that can be achieved are depending on radio channel conditions, required packet size and system capacity.

The table below provides examples on useful combinations of Packet Delay Budget (PDB) and Packet Error Rates (PER). The values in row 1 correspond to improved latency in LTE due to sTTI. The following cases 2 - 4 correspond to low latency and high reliability in LTE and NR, and the values in 2 – 4 are assumed to be sufficient to cover the most relevant cases currently foreseen.

Table 1: Example services with corresponding PDB and PER parameters

|  | Packet Delay Budget | Packet Error Rate | Example Services |
| --- | --- | --- | --- |
| 1. | 20 ms | 10-2 | Similar to “Process automation in TS 22.261, Table 7.2.2-1 |
| 2. | 1 ms | 10-5 | Factory automation. Similar to “Discrete automation – motion control” in TS 22.261, Table 7.2.2-1 |
| 3 | 5 ms | 10-5 | Power grid sub-station. Similar to “Electricity distribution – high voltage” in TS 22.261, Table 7.2.2-1 PDB: ~2 ms RAN latency, ~1ms CN |
| 4. | 10 ms | 10-5 | Haptic remote control. Similar to “Remote control” in TS 22.261, Table 7.2.2-1 |

Q2 from SA2 LS: guidance on other parameter values, e.g. whether a non-GBR very low latency QCI would still need a maximum bit rate parameter and/or whether very low latency services impose limits on the maximum packet size. SA WG2 are interested in whether the values for NR and E-UTRA are different.

RAN2 believes that the RAN would need to know about required latency, reliability, and priority. Additionally, RAN would need to be aware of Packet Size as well as packet periodicity and/or the bitrate. Packet periodicity indicates how often will packet arrive, this enables RAN to make estimation of resources that will be allocated to the user.

RAN2 is not aware of other parameters associated to limitations on bitrate or packet size as packets can be segmented in the RLC layer. On the other hand, the bitrate is potentially limited due to repetitions and low coding rate to ensure reliability.

Table 2 provides example values for packet size, periodicity, latency, and reliability. The table assumes GBR DRB.

Table 2: Example values for GBR use cases

| Payload size | Periodicity | Latency | Reliability | Example Services |
| --- | --- | --- | --- | --- |
| 40 bytes | 2 ms | 1 ms | 10-5 | Factory automation |
| 80 bytes | 10 ms | 2 ms | 10-4 | Power grid sub-station |

2. Actions:

To SA2.

ACTION: 	RAN2 kindly asks SA2 to take the RAN2 responses on 5QIs for URLLC provided above into account.

3. Date of Next RAN2 Meetings:

RAN2 #99bis	9 - 13 Oct 2017	Prague, Czech Republic

RAN2 #100	27 Nov – 1 Dec 2017	Reno, Nevada, USA