3GPP TSG-RAN WG2 #99	R2-1708338

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	Further details on RLC UM operation

Document for:	Discussion, Decision

# Introduction

The following agreements considering RLC UM operation have been made in RAN2:

Agreements

1.	RLC UM without SN for the complete SDU is selected.   FFS which options is selected

2.	SI field is included in RLC UM header to differentiate complete RLC SDU, the first SDU segment, the middle SDU segment, and the last SDU segment.

-	The header of unsegmented SDU contains only SI field.

-	The header of first segment contains only SI field and SN.

-	The header of middle and last segment contains SI field, SN, and SO.

In this contribution, we discuss further details of the RLC UM operation.

# Discussion

## RLC UM window operation

RAN2 agreed that only UMD PDU segments carry a Sequence Number while UMD PDUs carrying a complete RLC SDU do not carry a SN. As a SN is not included in all PDUs, a SN-based receiver window operation is not possible in RLC UM mode.

Thus, RLC UM is almost the same as RLC TM except that it supports segmentation. Whenever a PDU without SN arrives, the RLC receiver removes any headers and passes the SDU to upper layer (i.e. PDCP). In our view, this will increase the probability of desync in higher layers and other issues RAN2 has not discussed what the expected performance and impacts may be. In the case the new design causes unforeseen problems we should revert to the previous operation (similar to LTE).

Current RAN2 agreements have deprecated the UM receiver window operation

It is clear that given these RAN2 agreements so far, there is no basis to introduce any new mechanism to control an RLC UM window. This is not technically feasible. Thus, the discussion in RAN2 should focus only on how to control the RLC SDU segments and how to discard these segments if needed.

## Control of RLC SDU segments

During the email discussion, three RLC UM operation mechanisms were considered. These mechanisms can only be used to control the segments. More specifically, these mechanisms can only be used to decide when to discard segments when a RLC SDU is considered lost.

- Option 2-1 with window mechanism
Similar to option 1-1, this option relies on receive window to trigger segment discard

- Option 2-2 with one T-reassembly timer
One T-reassembly timer may be used to detect the reassembly failure of multiple gaps based on state variables similar to LTE

- Option 2-3 with multiple T-reassembly timers
Multiple t-reassembly timers

As there is no SN included in complete PDUs, a window-based segment discard in option 2-1 would not work properly. We can certainly introduce a window to control the segments but, effectively, this control means very little as the receiver side cannot know when the next segmented RLC UM SDU, with SN, will come. A very long time could pass between the reception of a PDU with a segment and the next. This means that the receiver only needs a function that discards the segments after waiting for a period of time.

As SN is included only in SDU segments, a window-based receiver and reassembly is not sufficient to guarantee proper reassembly. A timer is needed to discard SDU segments.

Option 2-2 and option 2-3 are thus the only options which can achieve robust segment discard based on time. Option 2-3 has a higher complexity as it needs to follow a set of timers and there will be a need to keep a set of variables. It is unclear what would be the benefit of having such a detailed track of the segments. Thus, the simplest approach is Option 2-2. This option ensures a simple mechanism to discard segments after a period waiting for pending segments. This option has a very similar operation as the T-reordering timer.

One T-reassembly timer is introduced. The timer operation is as follows:

  - The timer is started when the receiver receives a first segment (UMD PDU) of an RLC SDU.

  - The timer is stopped when all segments of the RLC SDU have been received.

  - Upon expiration of the timer, all received segments of the RLC SDU are discarded. The timer is re-started if there are any segments pending to be received of any other RLC SDU.

## SN setting

Another issue to discuss is how the SN is set. There are, at least, two possible options:

- The transmitter associates a SN to every RLC SDU. Only UMD PDUs which are segments carry the associated SN in the header.

- The transmitter assigns a SN only to those RLC SDUs which are segmented. With this option, the SN can only be assigned when the transmitter knows if a RLC SDU is going to be segmented.

There are no great advantages or disadvantages of each of the two alternatives. Either approach is feasible.

The transmitter only associates and assigns a SN to UMD PDUs which are segmented.

# Text Proposal

### 5.1.2	UM data transfer

#### 5.1.2.1	Transmit operations

##### 5.1.2.1.1	General

When delivering an UMD PDU that contains a segment of a RLC SDU, to lower layer, the transmitting side of an UM RLC entity shall:

-	if the RLC SDU did not have associated a SN:

-	associate a SN with the RLC SDU equal to TX_UM_Next

-	increment TX_UM_Next by one.

-	set the SN of the UMD PDU to the SN of the corresponding RLC SDU.

#### 5.1.2.2	Receive operations

##### 5.1.2.2.1	General

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	place it in the reception buffer (see sub clause 5.1.2.2.3);

-	if the UMD PDU is a complete RLC SDU, or

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from UMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if t-Reordering is running:

-	if all bytes of the RLC SDU with SN = x have been received, and

-	if no other RLC SDUs is waiting for byte segments

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case t-Reordering is stopped due to actions above):

-	if all bytes of the RLC SDU with SN = x have not been received

-	set Pending_SN_Reord to x

-	start t-Reordering;

##### 5.1.2.2.4	Actions when t-Reordering expires

When t-Reordering expires, the receiving UM RLC entity shall:

-	discard all bytes of the RLC SDU which triggered the T-reordering i.e. Pending_SN_Reord

-	if any RLC SDU is waiting for any byte segment

-	set Pending_SN_Reord to the next RLC SN for which all byte segments have not been received

-	start t-Reordering;

# Conclusion

In section 2 we made the following observations:

Observation 1	Current RAN2 agreements have deprecated the UM receiver window operation

Observation 2	As SN is included only in SDU segments, a window-based receiver and reassembly is not sufficient to guarantee proper reassembly. A timer is needed to discard SDU segments.

Based on the discussion in section 2 we propose the following:

Proposal 1	One T-reassembly timer is introduced. The timer operation is as follows:

a.	The timer is started when the receiver receives a first segment (UMD PDU) of an RLC SDU.

b.	The timer is stopped when all segments of the RLC SDU have been received.

c.	Upon expiration of the timer, all received segments of the RLC SDU are discarded. The timer is re-started if there are any segments pending to be received of any other RLC SDU.

Proposal 2	The transmitter only associates and assigns a SN to UMD PDUs which are segmented.

# References

R2-1706794, “E-mail discussion summary of RLC UM”, Qualcomm, RAN2 NR AdHoc, Qingdao, China, June 27-29, 2017

R2-1708343, “RLC UM header formats”, Ericsson, RAN2#99, Berlin, Germany, August 21-25, 2017