3GPP TSG-RAN WG2 #99	Tdoc R2-1708345

Berlin, Germany, 21st – 25th August 2017	Resubmission of R2-1707167

Agenda Item:	10.3.2.5

Source:	Ericsson

Title:	UP timers in RLC

# Introduction

At the RAN1#86bis meeting it was agreed that the duration of a subframe in NR is 1 ms and the reference numerology is 15 kHz:

Agreements

•	Sub-frame duration is fixed to 1ms

•	Reference numerology for defining subframe duration is 15 kHz

In this contribution we discuss some impacts on different user plane related timers in RLC given this agreement and if/how these timers should be configured for different numerologies and TTI durations.

# Discussion

The RLC timers as used in LTE are investigated in this paper. Whether or not all of them will be adopted into NR is not decided but they have been used as a baseline in this investigation on which timers will be impacted on having a fixed subframe duration of 1ms and if the configurable value range of each timer is also adequate for NR. In the following text, we have excluded timers which impact parts of the protocols not included in the first phase of NR, e.g. timers related to WLAN and sidelink operation. A similar paper for timers related to PDCP can be found in [2]

## RLC Timers

No timers in the Radio Link Control Protocol in LTE are configured by using the time unit of a subframe or a PDCCH-subframe, see related discussion in [1]. To avoid affecting these timers if different TTI lengths are configured dynamically in MAC by the use of different numerologies or sTTI and/or mini-slots, they should be kept using an absolute value in milliseconds.

The RLC timers in LTE are configured with an absolute value in milliseconds, keeping this for NR will avoid having these timers affected if the TTI length is dynamically changed in the MAC layer.

Under assumption that the RLC timers are usually configured to multiples of the HARQ RTT value and since the value of the HARQ RTT is assumed to be lower than in LTE, timers adopted to NR should be evaluated and perhaps adapted in their configurable value range for NR, to ensure optimal operation.

The Radio Link Control Protocol in LTE supports the following timers:

-	t-PollRetransmit (configured in milliseconds)

-	Used by the transmitting side in order to retransmit a poll. A poll can e.g. be done every x PDU or every x byte. The configurable value range are from 5ms to 4000ms. Should be sufficient for numerologies in NR with shorter TTIs.

-	t-Reordering/t-Reassembly (configured in milliseconds)

-	Updated in NR with new name, t-Reassembly. Used by the receiving side in order to detect loss of RLC PDUs at lower layer. It is typically chosen to not expire for a number of HARQ retransmissions, which implies that also for numerologies with short TTIs, it will have a value of a few ms and the current values are configurable from 0ms to 200ms and should be sufficient for numerologies in NR with shorter TTIs.

-	t-StatusProhibit (configured in milliseconds)

-	Used by the receiving side in order to prohibit transmission of a STATUS PDU. The current values are configurable from 0ms to 800ms and should be sufficient for numerologies in NR with shorter TTIs.

Reuse the time unit of milliseconds and the current timing durations for the RLC timers t-PollRetransmit, t-Reordering/t-Reassembly and t-StatusProhibit in NR.

Additional values for these timers are not needed for the first release.

# Conclusion

We have the following proposals and observations:

Observation 1	The RLC timers in LTE are configured with an absolute value in milliseconds, keeping this for NR will avoid having these timers affected if the TTI length is dynamically changed in the MAC layer.

Proposal 1	Reuse the time unit of milliseconds and the current timing durations for the RLC timers t-PollRetransmit, t-Reordering/t-Reassembly and t-StatusProhibit in NR.

Proposal 2	Additional values for these timers are not needed for the first release.

- References

R2-1708187, PDCCH, CORESET and event-driven MAC, Ericsson, Berlin, Germany, August 2017

R2-1708330, UP Timers in PDCP, Ericsson, Berlin, Germany, August 2017